3GPP TSG-SA5 Meeting #128 	S5-197568

# Zhuhai (CN), 18 - 22 November 2019  	             	                       			 Revision of S5-197325

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28.552 | CR |  |  |  | 0160 |  |  |  | rev |  |  | - |  |  | Current version: |  |  |  |  |  |  | 16.3.0 |  |  |  |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proposed change affects: |  |  |  |  |  |  |  | UICC apps |  |  |  |  |  | ME |  |  | Radio Access Network |  |  |  |  | X |  | Core Network |  |  |
| Title: |  |  | PDCP Data Volume measurements update |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: |  |  | Ericsson, Verizon |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  | S5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: |  |  | 5G_SLICE_ePA |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  | 2019-11-04 |  |  |  |  |  |
| Category: |  |  | C | B |  |  |  |  |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-16 |  |  |  |  |  |
|  |  |  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |  |  |  |  |  |
| Reason for change: |  |  |  |  | The PDCP SDU and PDU Data Volume measurements per S-NSSAI is of importance to the operator to monitor the average Data Volume for different S-NSSAIs. The existing definition of PDCP SDU and PDU Data Volume measurements are missing measurements per S-NSSAI. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  |  |  |  | The following PDCP SDU and PDU Data Volume measurements are updated in line with the other measurements in 28.552 (that supports S-NSSAI) to optionally have subcounters per S-NSSAI: - DL PDCP PDU Data Volume - UL PDCP PDU Data Volume - DL PDCP SDU Data Volume - UL PDCP SDU Data Volume |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  | Those PDCP SDU and PDU Data Volume measurements in 28.552 will not be defined for network slicing. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  | 5.1.3.6.1.1, 5.1.3.6.1.2, 5.1.3.6.2.1, 5.1.3.6.2.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  |  |  |  | X |  | Other core specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| affected: |  |  |  |  |  |  | X |  | Test specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| (show related CRs) |  |  |  |  |  |  | X |  | O&M Specifications |  |  |  |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

1st modified section

5.1.3.6.1.1	DL PDCP PDU Data Volume

- This measurement provides the Data Volume (amount of PDCP PDU bits) in the downlink delivered from GNB-CU to GNB-DU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

b)	CC.

c)	This measurement is obtained by counting the number of DL PDCP PDU bits sent to GNB-DU. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

The measurements of DL Cell PDCP PDU Data Volume in Dual-Connectivity scenarios is not included.

d)	Each measurement is an integer value representing the number of bits measured in Mbits (1MBits=1000*1000 bits). The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of supported S-NSSAIs.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpPduVolumeDL_Filter.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

Next modified section

5.1.3.6.1.2	UL PDCP PDU Data Volume

a)  This measurement provides the Data Volume (amount of PDCP PDU bits) in the uplink delievered from GNB-DU to GNB-CU. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. The unit is Mbit (1MBits=1000*1000 bits).

b)	CC

c)	This measurement is obtained by counting the number of bits entering the GNB-CU. The measurement is performed at the PDCP PDU level.  The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

The measurements of UL Cell PDCP PDU Data Volume in Dual-Connectivity scenarios is not included.

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of supported S-NSSAIs.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpPduVolumeUl_Filter.

f)	GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

Next modified section

5.1.3.6.2.1	DL PDCP SDU Data Volume

This measurement provides the Data Volume (amount of PDCP SDU bits) in the downlink delivered to PDCP layer. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

The unit is Mbit.

b)	CC

c)	This measurement is obtained by counting the number of bits entering the NG-RAN PDCP layer. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of supported S-NSSAIs.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpSduVolumeDl_Filter.

f)	 GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

Next modified section

5.1.3.6.2.2	UL PDCP SDU Data Volume

a)	This measurement provides the Data Volume (amount of PDCP SDU bits) in the uplink delivered from PDCP layer to SDAP layer or UPF. The measurement is calculated per PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI. 
The unit is Mbit.

b)	CC.

c)	This measurement is obtained by counting the number of bits leaving the NG-RAN PDCP layer. The measurement is performed at the PDCP SDU level. The measurement is performed per configured PLMN ID and per QoS level (mapped 5QI or QCI in NR option 3) and per supported S-NSSAI.

d)	Each measurement is an integer value representing the number of bits measured in Mbits. The number of measurements is equal to the number of PLMNs multiplied by the number of QoS levels or multiplied by the number of supported S-NSSAIs.
[Total no. of measurement instances] x [no. of filter values for all measurements] (DL and UL) ≤ 100.

e)	The measurement name has the form QosFlow.PdcpSduVolumeUL_Filter.

f)   GNBCUUPFunction.

NRCellCU.

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is for performance assurance within integrity area (user plane connection quality). NRCellCU measurement applies only for 2-split deployment.

End of modified section