| 3GPP TR 28.810 V0.2.0 (2019-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on concept, requirements and solutions for levels of autonomous network; (Release 16 ) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	6

3.3	Abbreviations	7

4	Background and concepts	7

4.1	Background	7

4.1.1	Overview	7

4.1.2	Relevant study in other SDOs or industry parties	7

4.1.2.1	Relevant study in automotive industry	7

4.1.2.2	Relevant study in GSMA	7

4.1.2.3	Relevant study in TMF	7

4.1.2.4	Relevant study in ITU	8

4.1.3 	Benefits of having autonomous network level	8

4.2	Concept of network autonomy	8

4.3	Concept of network autonomy level	8

4.4	Potential dimensions for classification of network autonomy	9

4.4.1 	Introduction	9

4.4.2	Workflow	9

4.4.3	Management scope	9

5	Scenarios	10

5.1	Fault RCA and recovery scenario example for network autonomy level	10

5.2	Coverage optimization scenario example for classification of network autonomy	10

5.3	RAN NE provisioning scenario example for classification of network autonomy	11

5.4	Multi-domain/layer/technology management service coordination automation scenario example	11

6	Potential solutions	12

6.1	Potential solutions for network autonomy level	12

6.1.1 	Introduction	12

6.1.2 	Framework approach for classification of network autonomy level	12

7	Conclusion and recommendation	13

Annex <A> (normative): <Normative annex for a Technical Specification>	13

Annex <B> (informative): <Informative annex for a Technical Specification>	14

B.1	Heading levels in an annex	14

Annex <B>: <Informative annex title for a Technical Report>	15

Annex <C> (informative): Bibliography	16

Annex <D> (informative): Index	17

Annex <X> (informative): Change history	18

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

1	Scope

The present document describes the background, concept, definition and classification of network autonomy levels, typical scenarios for managing network and service which need autonomous support, the potential requirement and solutions and recommendation for the way forward in normative work.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	SAE J3016_201401:”Taxonomy and Definitions for Terms Related to On-Road Motor Vehicle Automated Driving Systems”.

[3]	GSMA https://www.gsma.com/futurenetworks/wiki/ai-automation-an-overview/ : “AI & Automation: An Overview”.

[4]	TMF https://www.tmforum.org/wp-content/uploads/2019/05/22553-Autonomous-Networks-whitepaper.pdf.: “A whitepaper of autonomous networks: empowering digital transformation for the telecoms industry”.

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Background and concepts

Editor's note: this clause will contain background of use and benefits of classification of network autonomy level, concepts and classification of network autonomy level, and factors which may influence the availability of network with various levels of autonomy.

4.1	Background

4.1.1	Overview

Autonomous network is one of the important topics in 5G network. Complexity of 5G network increases with large number of devices and diversity of services. Different autonomous mechanisms are introduced by the industry to reduce the complexity of mobile network and service management. Different autonomous mechanisms in the networks may lead to different capabilities of autonomy and different performance on network and service management, which may lead to different autonomous networks levels.

4.1.2	Relevant study in other SDOs or industry parties

4.1.2.1	Relevant study in automotive industry

In 2014, the Society of Automotive Engineers (SAE) proposed the automatic driving levels in [2], clarifying the 6-level automatic driving rating standard (L0～L5). The SAE J3016 scheme determines the automatic driving level by evaluating the participation degree of the driver and the automatic driving system from four evaluation dimensions: motion control (acceleration/deceleration/steering), driving environment monitoring, fallback performance of dynamic driving task, and system capability (driving modes). This standard has been widely accepted by the automotive industry and applied to industry analysis, product planning, and been used for guiding the automotive industry to carry out five stages of work to achieving full automatic driving.

SAE J3016 provides a helpful reference to telecommunications industry for the study of autonomous networks levels. However, there are some fundamental differences between the automotive industry and telecommunication networks, thus the classification of network autonomy levels and automatic driving levels is different as well.

4.1.2.2	Relevant study in GSMA

The transformation from “no intelligence” to “full intelligence” is considered to be a long term with a step-by-step process and introducing automation with AI abilities into network can bring value to operators. GSMA calls for a harmonized classification system and supporting definitions regarding intelligent network, see [3]. A harmonized classification system of intelligent network is considered to be helpful to define the concept, levels of intelligent network, describe categorical distinction and educate a broader community while a network automation system is engaged.

GSMA is interested on this topic and encourage relevant studies.

4.1.2.3	Relevant study in TMF

Similar with [3], a harmonized classification system and supporting definitions for autonomous networks is delivered in [4]. The following factors of autonomous networks are introduced for evaluation of autonomous networks levels: execution, awareness, analysis, decision, intent/experience, applicability.

The following levels of autonomous networks are described as well: level 0 - manual management, level 1 - assisted management, level 2 - partial autonomous network, level 3 - conditional autonomous network, level 4 - high autonomous network, level 5 - full autonomous network.

4.1.2.4	Relevant study in ITU

A framework for evaluating intelligence of future networks including IMT-2020 is specified in Draft new Recommendation ITU-T Y.3173. A method for evaluating intelligence level and an architecture view for evaluating network intelligence level are introduced. The following dimensions for evaluating network intelligence level are identified: demand mapping, data collection, analysis, decision, action implementation. The following network intelligence levels are specified: L0: Manual network operation, L1: Assisted network operation, L2: Preliminary intelligence, L3: Intermediate intelligence, L4: Advanced intelligence, L5: Full intelligence.

4.1.3 	Benefits of having autonomous network level

As described in clause 4.1.2.1, the document of SAE J3016 has been widely applied to automobile industry analysis, consulting of motor vehicle products planning, and also been used for guiding the automotive industry to carry out five stages of work to achieving full automatic driving.

Similar to what SAE J3016 means to the automotive industry, a common understanding of the levels of autonomous network will be helpful to the telecommunications industry. The operator’s operational and management efforts and dedicated relevant resources will vary according to different autonomous networks levels. It would be beneficial for operators to have clear view on the expectation of the level of their network (s) to indicate the maturity of the network autonomy, so that they could first focus on the important features which should be prioritized to achieve certain level of autonomy in their networks. This approach will help operators to smoothly migrate to a higher autonomy level and a better operational efficiency.

Having autonomous network level has the following benefits:

-	Providing evaluation basis for measuring the level of an autonomous network along with its components and workflows.

-	Providing reference for gaps and priorities analysis for standardization works on network autonomy.

-	Providing guidance to operators, vendors and other participants of telecommunications industry for roadmap planning.

4.2	Concept of network autonomy

Network autonomy describes the telecom system (including management system and network) capability which is able to be governed by itself with minimal to no human intervention. Some features discussed in 3GPP are related to network autonomy. Following are some examples:

- Self-Organizing Network (SON)

- Management data analytics

- Intent driven management

- Close loop SLS assurance

Editor’s Note: whether using other term to replace the term “Network autonomy” in this document is FFS.

4.3	Concept of network autonomy level

Network autonomy level describes the level of application of autonomy capabilities in the network management workflow. The participation of the human and telecom system in the network management workflow are important factors to evaluate the network autonomy level. For each network autonomy level, which tasks can be performed by telecom system, which tasks can be performed by human, and which tasks can be performed by cooperation of human and telecom system needs to be clarified. For example, in the highest autonomy level, all tasks are performed by telecom system.

4.4	Potential dimensions for classification of network autonomy

4.4.1 	Introduction

This clause describes the potential dimensions for classification of network autonomy.

4.4.2	Workflow

Workflow is used to describe the necessary steps to fulfil certain management purposes. A workflow is composed of one or more management tasks. The autonomy capabilities of the tasks in the workflow may impact the network autonomy level.

Following are the potential categorization of the tasks in a workflow.

-	Awareness: The group of tasks which monitor network by collecting network information.

-	Analysis: The group of tasks which analyse the collected information (e.g. information about network status, network issues and so on) or based on the historical data to further predict the future change trend of the above network status, and make recommendation for decision.

-	Decision: The group of tasks which decide the necessary management operation for execution, e.g. network configuration or adjustment.

-	Execution: The group of tasks which execute the management operations.

Editor’s Note: above categorization of the tasks in a workflow may need to be revisit based on the investigation on corresponding workflow discussed in other SDOs and SI/WI in SA5.

4.4.3	Management scope

The network autonomy can be implemented in different management scopes, the complexity of network autonomy depends on the management scope. For example, it will be more challenge for the network system to achieve the network autonomy on cross domain layer than domain layer, because more autonomy mechanism needs to be introduced for the coordination between different domains.

Following are potential scopes of network autonomy:

- Network autonomy in NE layer, which means the autonomy mechanism is executed in the NE.

- Network autonomy in domain layer, which means the autonomy mechanism is executed in the MnF(s) in domain.

- Network autonomy in cross domain layer, which means the autonomy mechanism is executed in the MnF(s) in cross domain.

Figure 4.4.3-1 Network autonomy for different management scope

5	Scenarios

Editor's note: this clause will contain typical scenarios and potential requirements for managing and operating the network and service that need autonomous support.

5.1	Fault RCA and recovery scenario example for network autonomy level

Fault root cause analysis (RCA) and recovery scenario refers to the entire workflow of network fault management, full autonomy of fault RCA and recovery can help the network operator to reduce OPEX by reducing manual involvement in such tasks and to enhance user experience by reducing the time for network fault recovery. However, full autonomy of fault RCA and recovery is a long term goal, it will be beneficial for operator to achieve this goal step by step and have clear view on which typical issues can be addressed by utilizing network autonomy mechanism in corresponding steps.

Following brief introduction is an example of classification of network autonomy for fault RCA and recovery scenario:

Classification A:

-	Alarm filtering, root cause correlation and at least one fault recovery execution task is executed by network system based on human defined RCA rules or human configured recovery instructions. At least one fault recovery evaluation task is accomplished by network system.

-	All the other tasks in the fault RCA and recovery workflow are accomplished by human.

Classification B:

-	Alarm filtering, root cause correlation and fault recovery execution tasks are executed by network system itself without human intervention and human defined rules. At least one fault recovery evaluation task is accomplished by network system. Fault discovery and recovery analysis tasks are accomplished by network system based on human defined rules.

-	Fault recovery instructions are determined by human.

Classification C:

-	All the alarm filtering, root cause correlation, fault recovery execution and fault recovery evaluation tasks are executed by network system itself without human intervention and human defined rules. Fault discovery and recovery analysis tasks are accomplished by network system based on human defined rules. At least one fault recovery instruction is determined by network system based on human defined rules.

-	At least one fault recovery instruction is determined by human.

Classification D:

-	Almost the whole close-loop of fault RCA and recovery workflow is accomplished automatically by network system with human defined fault RCA and recovery rules.

-	Fault RCA and recovery rules are specified by human.

Classification E:

-	The whole close-loop of fault RCA and recovery workflow is accomplished automatically by network system without human intervention and human defined rules.

-	Human can but not have to supervise the fault recovery decision generated by network system.

Editor's note: the detailed clarification of network autonomy for this scenario is FFS.

5.2	Coverage optimization scenario example for classification of network autonomy

Coverage is critical for good user experience, however, coverage optimization is complexity. Full autonomy of coverage optimization for whole network is a long term goal, it will be beneficial for operator to achieve this goal step by step and have clear view on which typical issues can be addressed by utilizing network autonomy mechanism in corresponding steps.

Editor's note: the detailed classification of network autonomy for this scenario is FFS.

5.3	RAN NE provisioning scenario example for classification of network autonomy

RAN NE provisioning scenario refers to the entire workflow of deploying an RAN NE, full autonomy of RAN NE provisioning can help the network operator to reduce OPEX by reducing manual involvement in such tasks. However, full autonomy of RAN NE provisioning is a long term goal, it will be beneficial for operator to achieve this goal step by step and have clear view on which typical issues can be addressed by utilizing network autonomy mechanism in corresponding steps.

Editor's note: the detailed classification of network autonomy for this scenario is FFS.

5.4	Multi-domain/layer/technology management service coordination automation scenario example

The goal of this scenario is to enable automation of end-to-end joint coordination of management services across multiple domains (e.g. AN, CN), multi-layers (packet and optical transport), and multi-technology (wireless and wireline, x-haul, mobile core, data network, physical and virtual functions).

The following functions and services are assumed to be deployed and active:

- NG-RAN, 5GC, NSI(s),

- The performance assurance (PM), fault supervision (FM) and provisioning (CM) services are available across multi-domain, multi-layer, and multi-technology.

- The provider of multi-domain, multi-layer, and multi-technology coordination of management services.

The provisioning, assurance and supervision services provide information on the multi-domain, layer, and technology, the information characterized by:

- Status details on domain/layer/technology specific network resources, e.g. NG-RAN and 5GC, packet and transport networks, physical network resources and virtual resources

- Information from one or multiple network slices and slice subnets in relation with multi-domain, layer, and technology

- For each domain, layer, and technology, information on allocated resources and their utilization, event history (e.g. recent orchestration actions taken per domain, layer, and technology), history of actions taken (e.g. recent orchestration actions taken per domain, layer, and technology)

The following scopes of network autonomy is applied to meet the objectives of automation of the multi-domain, layer, and technology coordination of management services:

- Network autonomy in NE layer

- Network autonomy in domain layer

- Network autonomy in cross domain layer

Based on the assumptions described above, the provider of multi-domain, multi-layer, and multi-technology coordination of management services is running jointly with individual coordination providers of domain, layer, and technology to automate end-to-end vertical and horizontal management coordination processes across multiple domain, layers, and technologies.

The provider of multi-domain, multi-layer, and multi-technology coordination of management services utilizes provisioning, assurance and supervision services to achieve optimal management coordination across multiple domains, layers, and technologies.

The provider of multi-domain, multi-layer, and multi-technology coordination of management services may utilize AI technologies (e.g., machine learning, deep learning algorithms, etc.) to achieve an optimal coordination.

The provider of multi-domain, multi-layer, and multi-technology coordination of management services offers management capabilities on the resource parameters configuration.

The coordination of management services consists of different levels of network autonomy as follows:

-Automation of coordination across multiple layer, technology, or domain individually

-Automation of coordination across a combination of multiple-layer and technology, multiple-layer and domain, or multiple-technology and domain

- Automation of coordination across multiple-layer, technology, and domain as a whole

6	Potential solutions

Editor's note: this clause will specify the working methodology to address the relation among the levels of network autonomy and various standardized features related to autonomous network, potential work flows for the management of autonomous network and recognize the necessary automation features to be enhanced.

6.1	Potential solutions for network autonomy level

6.1.1 	Introduction

This clause describes the potential solutions for network autonomy level.

6.1.2 	Framework approach for classification of network autonomy level

Editor's note: the text and table in clause 6.1.2 initial an example of potential framework approach for network autonomy level, which needs more investigation based on the discussion of concept and scenario.

According to the potential categorization of the tasks in a general network autonomy workflow, a framework approach for classification of network autonomy level is introduced as following.

Table 6.1.2 Framework approach for classification of network autonomy level

| Network autonomy level |  | Task categories |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Execution | Awareness | Analysis | Decision | Intent translation |
| L0 | Manual operating network | Human | Human | Human | Human | Human |
| L1 | Assisted operating network | Human & Network system | Human & Network system | Human | Human | Human |
| L2 | Preliminary autonomous network | Network system | Human & Network system | Human & Network system | Human | Human |
| L3 | Intermediate autonomous network | Network system | Network system | Human & Network system | Human & Network system | Human |
| L4 | Advanced autonomous network | Network system | Network system | Network system | Network system | Human & Network system |
| L5 | Full autonomous network | Network system | Network system | Network system | Network system | Network system |

Level 0 manual operating network: No categorization of the tasks is accomplished by network system itself.

Level 1 assisted operating network: A part of the execution and awareness tasks are accomplished automatically by network system itself based on human defined rules. At this level, network system can assist human operator to improve the execution and awareness efficiency.

Level 2 preliminary autonomous network: All the execution tasks are accomplished automatically by network system itself. A part of the awareness and analysis tasks are accomplished automatically by network system itself based on human defined rules. At this level, execution tasks do not need human intervention.

Level 3 intermediate autonomous network: All the execution and awareness tasks are accomplished automatically by network system itself. A part of the analysis and decision tasks are accomplished automatically by network system itself based on human defined rules. At this level, execution and awareness tasks do not need human intervention.

Level 4 advanced autonomous network: All the execution, awareness, analysis and decision tasks are accomplished automatically by network system itself. And intent translation tasks can be partly accomplished automatically by network system itself based on human defined rules. At this level, almost the whole close-loop of network autonomy workflow is accomplished automatically by network system without human intervention except for intent translation tasks.

Level 5 fully autonomous network: The whole close-loop of network autonomy workflow is accomplished automatically by network system without human intervention.

NOTE - Human reviewed decision have the highest authority in each level if there is any confliction between human reviewed decision and network system generated decision.

7	Conclusion and recommendation

Editor's note: this clause will be used to document the conclusions and recommendation of the study.

Annex <A> (normative):
<Normative annex for a Technical Specification>

Annex <B> (informative):
<Informative annex for a Technical Specification>

B.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. B.1.2 is formatted using Heading 2 style.

Annex <B>:
<Informative annex title for a Technical Report>

Annex <C> (informative):
Bibliography

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

<Publication>: "<Title>".

Annex <D> (informative):
Index

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | SA5#127 | S5-196391 |  |  |  | Study on concept, requirements and solutions for levels of autonomous network | 0.0.0 |
| 2019-10 | SA5#127 | S5-196323 S5-196878 S5-196740 |  |  |  | 1. pCR 28.810 Add skeleton 2. pCR 28.810 Add background 3. pCR 28.810 Add scope | 0.1.0 |
| 2019-11 | SA#128 | S5-197811 S5-197820 S5-197821 S5-197822 S5-197812 S5-197813 S5-197780 S5-197676 S5-197799 S5-197702 |  |  |  | 1. pCR 28.810 addition of multi-domain/layer/technology orchestration automation use case and requirement 2. pCR 28.810 Add concept of network autonomy 3. pCR 28.810 Add concept of network autonomy level 4. pCR 28.810 pCR 28.810 Add potential dimensions for classification of network autonomy level 5. pCR 28.810 Add coverage optimization scenario exampe for classification of network autonomy levels 6. pCR 28.810 Add NE deployment scenario exampe for classification of network autonomy levels 7. Add background of relevant study in other SDOs or industry parties 8. Add significance for levels of autonomous network 9. Add potential solution for network autonomy level 10. Add fault RCA and recovery scenario example for network autonomy level | 0.2.0 |