| 3GPP TS 28.201 V0.3.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Architecture considerations	8

4.1	High-level Description	8

4.2	Network Slice performance and analytics charging architecture	8

4.2.1	High level network slice performance and analytics architecture	8

4.2.2	Converged charging architecture	8

5	Network slice performance and analytics charging principles and scenarios	9

5.1	Network slice performance and analytics charging principles	9

5.1.1	General	9

5.1.2	Requirements	9

5.1.3	Network slice performance and analytics charging information	9

5.1.4	NWDAF discovery	10

5.1.5	CHF selection	10

5.2 	Network slice performance and analytics charging scenarios	10

5.2.1	Basic principles	10

5.2.1.1	General	10

5.2.1.2	Applicable triggers in network slice performance and analytics charging	11

5.2.2.2	Analytics and performance charging from CSIF	11

5.2.2.2.1	General	11

5.2.2.2.2	General analytics and performance charging – PEC	12

5.2.2.3	Network data analytics subscription from NWDAF	12

5.2.2.3.1	General	12

5.2.3	CDR generation	12

5.2.3.1	Introduction	12

5.2.3.2	Triggers for CHF CDR	13

5.2.3.2.1	General	13

5.2.3.2.2	Triggers for CHF CDR generation	13

5.2.4	Ga record transfer flows	13

5.2.5	Bnspa CDR file transfer	13

6	Definition of charging information	13

6.1	Data description for network slice performance and analytics charging	13

6.1.1	Message contents	13

6.1.1.1	General	13

Annex <X> (informative): Change history	14

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

1	Scope

The present document is part of a series of documents that specify charging aspects of network slicing.

This document includes the charging aspects of network slice performance and analytics, including charging scenarios, data and way of data transfer specific for the network slice.

The architecture for network slice charging follows common architecture and concepts specified in TS 32.240 [1].

The charging aspect of network slice charging on PDU session charging is specified in TS 32.255 [15].

The network slice performance and analytics charging collects and processes charging information related to performance metrics of a network slice.

All references, abbreviations, definitions, descriptions, principles and requirements, used in the present document, that are common across 3GPP TSs, are defined in TR 21.905 [100]. Charging management in 3GPP networks/domains, services or subsystems are provided in the umbrella TS 32.240 [1] and are copied into clause 3 of the present document for ease of reading. Finally, those items that are specific to the present document are defined exclusively in the present document.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles ".

[2] - [14]	Void

[15]	3GPP TS 32.255: " Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

[16] - [49]	Void.

[50]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[51]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[52] - [54]	Void.

[55]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[56]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

[57]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[58] - [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101] - [149]	Void.

[150]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[151] - [199]	Void.

[200]	3GPP TS 28.202: "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[201] - [249]	Void.

[250]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[251] - [299]	Void.

[300]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[301] - [400]	Void.

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [100] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [100].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

Bnspa	Reference point for the CDR file transfer from the network slice performance and analytics CGF to the BD.

Ga	Reference point for CDR transfer between a CDF and the CGF.

Nchf	Service based interface exhibited by CHF.

Nnwdaf 	Service based interface exhibited by NWDAF.

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [100] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [100].

CHF	Charging Function

CGF	Charging Gateway Function

PEC	Post Event Charging

NWDAF	Network Data Analytics Function

MnF	Management Function

MnS	Management Service

4	Architecture considerations

4.1	High-level Description

The architecture for network slice charging follows common architecture and concepts specified in TS 32.240 [1].

The network slice charging shall support consumption charging of network slice instance and performance and analytics based charging.

Editor’s note: The high-level description is still ffs.

4.2	Network Slice performance and analytics charging architecture

4.2.1	High level network slice performance and analytics architecture

The scope of the present document is network slice performance and analytics charging, the details for other type of network slice charging are defined in TS 32.255 [15] and TS 28.202 [200].

4.2.2	Converged charging architecture

The CSIF (Charging Subscribing and Interaction Function) for network slice performance and analytics charging has two functional blocks:

-	Charging Subscribing Function (CSF) that subscribes to events and information by consuming services exposed by other functions, in order to be able to capture the required charging information from the service and send it to the CIF for charging purposes. In this document events and information related to charging of performance and analytics for network slices.

-	CHF Interaction Function (CIF) that receives the collected information from the CSF and determines the occurrence of chargeable events based on internal or CHF provided rules. When it determines that a chargeable event has occurred it then triggers charging by consuming the Nchf services.

Editor’s Note: The naming of CSIF, CSF and CIF are ffs.

The architectural option for network slice performance and analytics converged charging interacting with Network Functions is depicted in figure 4.2.1.1.

Figure 4.2.2.1: Network slice performance and analytics converged charging architecture

For network slice performance and analytics converged charging the CSIF can consume network slice performance management service (MnS) exposed by the management functions, described in TS 28.533 [250], and/or Network Data Analytics service (Nnwdaf) exposed by the Network Data Analytics Function (NWDAF), described in TS 23.288 [150].

The general architecture components can be found in TS 32.240 [1].

Ga is described in clause 5.2.4 and Bnspa in clause 5.2.5 of this document, and Nchf is described in TS 32.290 [50].

5	Network slice performance and analytics charging principles and scenarios

5.1	Network slice performance and analytics charging principles

5.1.1	General

Performance and analytics based charging shall be achieved by subscription to the performance metrics or analytics information of a specific network slice and report them to CHF for charging purpose. The detail of performance and analytics information is specified in the clause 5.1.3.

Network slice performance and analytics charging shall support collection of network slice performance metrics related to network slice charging based on network slice performance and analytics charging requirements.

5.1.2	Requirements

The following are high-level charging requirements specific to the network slice performance and analytics charging:

-	The CSIF shall support subscription to specific network slice performance and analytics information.

-	The CSIF shall support reporting charging information based on network slice performance and analytics information.

-	The CSIF  shall support reporting performance and analytics charging information per network slice.

-	The CSIF shall support PEC scenario to report performance and analytics charging information.

Editor’s note: The requirements are not complete.

5.1.3	Network slice performance and analytics charging information

The following is a non-exhaustive list related to one or more of the performance and analytics inputs relevant to charging for network slice:

-	Latency

-	Throughput

-	Maximum utilized bandwidth

-	Maximum packet loss rate as defined in GSMA NG.116 [x]

-	Service Experience statistics information as defined in Clause 6.4, TS 23.288 [150]

-	The number of PDU sessions

-	Registered Subscribers

-	Load level information as defined in Clause 6.3, TS 23.288 [150].

Editor’s note: the list is still ffs.

5.1.4	NWDAF discovery

The NWDAF service discovery by the CSIF is specified in TS 29.510[300].

5.1.5	CHF selection

The CHF address(es) selection by the CSIF is done based on NRF based discovery. The priority order depends on Operator's policies.

Editor’s note: The details on CHF selection by the NRF is not complete.

Editor’s note: CHF selection based on local configuration is FFS.

5.2 	Network slice performance and analytics charging scenarios

5.2.1	Basic principles

5.2.1.1	General

Network slice performance and analytics charging is performed by the CSIF interacting with CHF using Nchf specified in TS 32.290 [50] and TS 32.291 [51].In order to provide the data required for the charging management activities outlined in TS 32.240 [1], the CSIF shall be able to perform converged charging for each of the following:

-	the performance and analytics information obtained from NWDAF.

-	the performance and analytics information obtained from MnS;

The CSIF shall be able to obtain performance and analytics information from the NWDAF and/or MnS as follows:

-	The Subscribe/Notify are exchanged between the CSIF and the NWDAF using the Nnwdaf service. The interactions are issued by the CSIF towards the NWDAF when certain conditions are met.

-	The Subscribe/Notify are exchanged between the CSIF and the MnS producer. The interactions are issued by the CSIF when certain conditions are met.

The CSIF shall be able to perform converged charging by interacting with CHF, for charging data related to network slice performance and analytics. The Charging Data Request and Charging Data Response are exchanged between the CSIF and the CHF, based on PEC scenarios specified in TS 32.290 [50]. The Charging Data Request is issued by the CSIF towards the CHF when certain conditions (chargeable events) are met.

The charging information is collected per Single Network Slice Selection Assistance Information (S-NSSAI).

The services provided by NWDAF are defined in the TS 23.288[150] and the services provided by MnS producer are defined in the TS 28.533[250]. The charging functionalites for network slice performance and analytics, i.e. CSIF functionality, charging events and charging procedures, are defined in the following clauses.

Whether performance and analytics charging for an S-NSSAI in the CSIF is actived or not is based on configuration. Which performance and analytics information is obtained from NWDAF or MnS by the CSIF is based on configuration.

Editor’s note: The configuration of how performance and analytics information is obtained is FFS.

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in clauses 5.2.2.

A detailed formal description of the converged charging parameters to CHF defined in the present document is can be found in TS 32.291 [51].

A detailed formal description of the CDR parameters defined in the present document is can be found in TS 32.298 [57].

5.2.1.2	Applicable triggers in network slice performance and analytics charging

Triggers for CSIF to invoke a Charging Data Request [Event] towards the CHF are defined as following trigger conditions.

When a charging event is issued towards the CHF, it includes details such as Single Network Slice Selection Assistance Information (S-NSSAI).  Table 5.2.1.2.1.1 summarizes the set of default trigger conditions and their category which shall be supported by the CSIF.

Table 5.2.1.2.1.1: Default Trigger conditions in CSIF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Performance and analytics information notify | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| Expiry of time limit per S-NSSAI | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| Threshold reached for a performance indicator (e.g. load level per S-NSSAI) | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |
| Expiry of data event limit per S-NSSAI | - | Immediate | Not Applicable | Not Applicable | Charging Data Request [Event] |

Editor’s note: the table 5.2.1.2.1.1 default trigger conditions in CSIF is FFS.

The CDR generation mechanism processed by the CHF upon receiving Charging Data Request [Event] issued by the CSIF for these chargeable events, is specified in clause 5.2.3.

5.2.2.2	Analytics and performance charging from CSIF

5.2.2.2.1	General

The clause below describes analytics and performance charging from CSIF scenarios.

5.2.2.2.2	General analytics and performance charging – PEC

The following figure 5.2.2.2.2 describes analytics and performance charging:

Figure 5.2.2.2.2: Analytics and performance reporting

1)	Collection of analytics and performance information: Collection of information related to the network slice analytics and performance is done by the CSIF.

2)	Trigger condition met: the CSIF determines based on the collected information and the trigger conditions that a report of charging information to the CHF is needed.

3)	Charging Data Request [Event]: The CSIF generates charging data related to the collected information and sends the charging data request for the CHF to process the related charging data for CDR generation purpose.

4)	 Create CDR: the CHF stores received information and creates a CDR related to the event.

5)	Charging Data Response [Event]: The CHF informs the CSIF on the result of the request.

5.2.2.3	Network data analytics subscription from NWDAF

5.2.2.3.1	General

The clause below describes network data analytics subscription scenarios.

Editor’s note: The need for specific flows or if the flows in TS 23.288 can be referenced is FFS.

5.2.3	CDR generation

5.2.3.1	Introduction

The network slice performance and analytics CHF CDRs are generated by the CHF to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

The following clauses describe in detail the conditions for generating the network slice performance and analytics CHF CDR, which shall be supported by the CHF.

5.2.3.2	Triggers for CHF CDR

5.2.3.2.1	General

A network slice performance and analytics CHF CDR is used to capture charging information related to network slice performance and analytics chargeable events for PEC.

5.2.3.2.2	Triggers for CHF CDR generation

A network slice performance and analytics CHF CDR is generated by the CHF for each received Charging Data Request [Event].

5.2.4	Ga record transfer flows

Details of the Ga protocol application are specified in TS 32.295 [55].

5.2.5	Bnspa CDR file transfer

Details of the Bnspa protocol application are specified in TS 32.297 [56].

6	Definition of charging information

6.1	Data description for network slice performance and analytics charging

6.1.1	Message contents

6.1.1.1	General

The Charging Data Request and Charging Data Response are specified in subclause 5.1.2.2.1 of TS 32.290 [50].

Table 6.1.1.1.1 describes the use of these messages for network slice performance and analytics charging.

Table 6.1.1.1.1: network slice performance and analytics charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | CSIF | CHF |
| Charging Data Response | CHF | CSIF |

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of TS 32.240 [1]

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | SA5 #127 | S5-196554 S5-196555 S5-196137 |  |  |  | Update of the Skeleton Scope Add the terms | 0.1.0 |
| 2019-11 | SA5 #128 | S5‑197516 S5-197517 S5-197518 S5‑197519 S5‑197520 S5‑197521 S5‑197523 |  |  |  | Add high level 5G System architecture to clause 4.1 Architecture considerations Add high level description Add charging requirements to clause 5.1 Add information description to clause 5.1 Add discovery and selection to clause 5.1 Add CDR generation to clause 5.2 | 0.2.0 |
| 2020-02 | SA5 #129e | S5-201432 S5-201442 S5-201443 S5-201444 S5-201445 S5-201446 S5-201450 |  |  |  | Add message content Add applicable triggers to clause 5.2 Add Basic Principles to clause 5.2 Introduction of Reference point to the Billing system Update of the Reference Add performance and analytics inputs to clause 5.1.3 Addition of CSIF and CHF flows | 0.3.0 |