3GPP TSG-SA5 Meeting #130e	S5-202277rev3

e-meeting 20-28 April 2020

| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 32.290 | CR | 0116 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add CHF-Controlled Quota Management functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Amdocs |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHFCQM |  |  |  |  |  | Date: |  |  | 2020-02-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | R16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the functionality of suspending and resuming quota management for a given rating group, controlled by CHF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add section 5.4.X |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No CHF-controlled quota management capability |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change to TS 32.290

## 5.4	Other functionalities

### 5.4.X	CHF-Controlled Quota Management

CHF may instruct NF consumer to suspend quota management for a given rating group in the current charging session within charging data reponse message

When quota management is supended for a given rating group, usage reporting for that rating group shall indicate that the quota management is temporarely suspended in Charging Data Request messages [Update].  As in this situation Quota Management triggers are not used, other existing active triggers are used for further usage reporting.

CHF may instruct NF consumer to resume quota management for a given rating group for which quota management was previously suspended by:

- granting units, indicating that quota management is resumed, in Charging Data Response message, or

- using Re-authorization procedure.

End of change