3GPP TSG-SA5 Meeting #131e 	S5-203312

# e-meeting 25th May-3rd June 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.532 | CR | 0114 | rev | 2 | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update URI for generic fault supervision management service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-04-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some issues for the URI for generic fault supervision management service, for example, clause 12.2.1.3.2.1.2, the URI is described as follows: {DN_prefix_authority_part} / {DN_prefix_remainder}/FaultMnS/v1500/alarms, and the definiation for DN_prefix_authority_part and DN_prefix_remainder refers to TS 32.158. However, there is no such definition in TS 32.158. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the URI for generic fault supervision management service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrect URI and invalid reference are used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.2.1.3.1, 12.2.1.3.2.1.2, 12.2.1.3.2.2.2, 12.2.1.3.2.3.2, 12.2.1.3.2.4.2, 12.2.1.3.2.5.2, 12.2.1.3.2.6.2, 12.2.1.3.2.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: | S5-202361 |
| --- | --- |

1st Change

12.2.1.3	Resources

12.2.1.3.1	Resource structure

Figure 12.2.1.3.1-1 shows the resource structure of the Fault Supervision MnS. The "alarms","comments" and "subscriptions" resource are collection resources.

Figure 12.2.1.3.1-1: Resource URI structure of the Fault Supervision MnS

Table 12.2.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 12.2.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method | Description |
| --- | --- | --- | --- |
| alarms | /alarms | GET | Retrieve all alarms or a filtered subset |
|  |  | POST | Add a comment to multiple alarms |
|  |  | PATCH | Clear, acknowledge or unacknowledge multiple alarms |
| alarm | /alarms/{alarmId} | PATCH | Clear, acknowledge or unacknowledge a single alarms |
|  |  | POST | Add a comment to a single alarm |
| $alarmCount | /alarms/$alarmCount | GET | Retrieve the alarm count per perceived severity |
| subscriptions | /subscriptions | POST | Create a subscription |
| subscriptions | /subscriptions | DELETE | Delete all subscriptions made with a consumerReferenceId |
| subscription | /subscriptions/{subscriptionId} | DELETE | Delete a single subscription |
| notificationSink | /notificationSink | POST | Send notifications |

12.2.1.3.2	Resource definitions

12.2.1.3.2.1	Resource "/alarms"

12.2.1.3.2.1.1	Description

This resource represents a collection of alarms.

12.2.1.3.2.1.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/alarms

The resource URI variables a defined in the following table.

Table 12.2.1.3.2.1.2-1: URI variables

| Name |  | Definition |
| --- | --- | --- |
| MnSRoot |  | See clause 4.4 of TS 32.158 [15] |

12.2.1.3.2.1.3	HTTP methods

12.2.1.3.2.1.3.1	HTTP GET

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.1.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| alarmAckState | alarmAckState-QueryType |  | O |
| href | uri-Type |  | O |
| filter | filter-QueryType |  | O |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.2.1.3.2.1.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | Description | Qualifier |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.2.1.3.2.1.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | Qualifier |
| --- | --- | --- | --- |
| alarms-Type | 200 OK | The alarms returned. | M |
| error-Type | 4xx/5xx | Returned in case of an error | O |

12.2.1.3.2.1.3.2	HTTP POST

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.1.3.2-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| alarmIdList | Array of strings | Identifies the alarms the POST method shall be applied to. | O |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.2.1.3.2.1.3.2-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | Qualifier |
| --- | --- | --- |
| comment-Type | JSON schema for the representation of a comment resource. | M |

Table 12.2.1.3.2.1.3.2-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success, the response body shall be empty. | M |
| failedAlarms-ResponseType | 4xx/5xx | In case of failure, the response body shall carry a JSON object described by the "failedAlarms-Type" format. | M |

12.2.1.3.2.1.3.3	HTTP PATCH

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.1.3.3-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| alarmIds | array (alarmId-Type) | Identifies the alarms the PATCH shall be applied to | M |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.2.1.3.2.1.3.3-2: Data structures supported by the PATCH Request Body on this resource

| Data type | Description | Qualifier |
| --- | --- | --- |
| patchAcknowledgeAlarms-Type | Patch document for acknowledging one or multiple alarms | M |
| patchClearAlarms-Type | Patch document for clearing one or multiple alarms | M |

Table 12.2.1.3.2.1.3.3-3: Data structures supported by the PATCH Response Body on this resource

| Data type | Response codes | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success the response body shall be empty. | M |
| failedAlarms-ResponseType | 4xx/5xx | In case of failure, the response body shall carry a JSON object described by the "failedAlarms-Type" format. | M |

12.2.1.3.2.2	Resource "alarms /{alarmId}"

12.2.1.3.2.2.1	Description

This resource represents an alarm.

12.2.1.3.2.2.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/alarms/{alarmId}

The resource URI variables a defined in the following table.

Table 12.2.1.3.2.2.2-1: URI variables

| Name |  | Definition |
| --- | --- | --- |
| MnSRoot |  | See clause 4.4 of TS 32.158 [15] |
| alarmId |  | String identifying an alarm |

12.2.1.3.2.2.3	HTTP methods

12.2.1.3.2.2.3.1	HTTP PATCH

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.2.3.1-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures, the response data structures and response codes specified in the following table.

Table 12.2.1.3.2.2.3.1-2: Data structures supported by the PATCH Request Body on this resource

| Data type | Description | Qualifier |
| --- | --- | --- |
| patchAcknowledgeAlarms-Type | Patch document for acknowledging an alarm | M |
| patchClearAlarms-Type | Patch document for clearing an alarm | M |

Table 12.2.1.3.2.2.3.1-3: Data structures supported by the PATCH Response Body on this resource

| Data type | Response codes | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | 200 OK | In case of success the response body shall be empty. |  |
| failedAlarms-ResponseType | 4xx/5xx | In case of failure, the response body shall carry a JSON object described by the "failedAlarms-Type" format. |  |

12.2.1.3.2.3	Resource "alarms/$alarmCount"

12.2.1.3.2.3.1	Definition

This resource holds metadata about the /alarms collection resource like the alarm count per perceived severity.

12.2.1.3.2.3.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/alarms/$alarmCount

The resource URI variables are defined in table 12.2.1.3.2.3.2-1.

Table 12.2.1.3.2.3.2-1: URI variables

| Name | Definition |  |
| --- | --- | --- |
| MnSRoot | See clause 4.4 of TS 32.158 [15] |  |

12.2.1.3.2.3.3	HTTP methods

12.2.1.3.2.3.3.1	GET

This method shall support the URI query parameters specified in table 12.2.1.3.2.3.3.1-1.

Table 12.2.1.3.2.3.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | Description | SQ |
| --- | --- | --- | --- |
| n/a |  |  |  |

This method shall support the request data structures specified in table 12.2.1.3.2.3.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.3.3.1-3.

Table 12.2.1.3.2.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |

Table 12.2.1.3.2.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| alarmsCount-Type | 200 OK | The alarm count per severity level returned. | M |
| error-Type | 4xx/5xx | Returned in case of an error | O |

12.2.1.3.2.4	Resource "alarms/{alarmId}/comments"

12.2.1.3.2.4.1	Definition

This resource is a collection resource for comments attached to an alarm.

12.2.1.3.2.4.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/alarms/{alarmId}/comments

The resource URI variables are defined in the following table.

Table 12.2.1.3.2.4.2-1: URI variables

| Name | Definition |
| --- | --- |
| root | See clause 4.4 of TS 32.158 [15] |
| alarmId | Alarm identifier |

12.2.1.3.2.4.3	HTTP methods

12.2.1.3.2.4.3.1	POST

This method shall support the URI query parameters specified in the following table.

Table 12.2.1.3.2.4.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| n/a |  |  |  |

This method shall support the request data structures, and the response data structures and response codes specified in the following tables.

Table 12.2.1.3.2.4.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | Qualifier |
| --- | --- | --- |
| comment-RequestType | The representation of the comment to be added to an alarm. | M |

Table 12.2.1.3.2.4.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| comment-ResponseType | 201 Created | In case of success, the response body shall be described by the "comment-ResponseType" format. The commentTime property shall carry the value set by the server. | M |
| failedAlarms-ResponseType | 4xx/5xx | In case of failure, the response body shall be described by the "failedAlarms-ResponseType" format. | M |

12.2.1.3.2.5	Resource "/{commentId}"

12.2.1.3.2.5.1	Definition

This resource represents a comment attached to an alarm.

12.2.1.3.2.5.2	URI

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/alarms/{alarmId}/comments/{commentId}

The resource URI variables are defined in the following table.

Table 12.2.1.3.2.4.5-1: URI variables

| Name |  | Definition |
| --- | --- | --- |
| MnSRoot |  | See clause 4.4 of TS 32.158 [15] |
| alarmed |  | Alarm identifier |
| commentId |  | Comment identifier |

12.2.1.3.2.5.3	HTTP methods

None.

12.2.1.3.2.6	Resource "/subscriptions"

12.2.1.3.2.6.1	Description

This resource is a container resource for individual subscriptions.

12.2.1.3.2.6.2	URI

The resource URI is:

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/subscriptions

The resource URI variables are defined in the following table.

Table 12.2.1.3.2.6.2-1: URI variables

| Name | Definition |
| --- | --- |
| root | See clause 4.4 of TS 32.158 [15] |

12.2.1.3.2.6.3	HTTP methods

12.2.1.3.2.6.3.1	POST

This method shall support the URI query parameters specified in table 12.2.1.3.2.6.3.1-1.

Table 12.2.1.3.2.6.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.2.1.3.2.6.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.6.3.1-3.

Table 12.2.1.3.2.6.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| subscription-RequestType | Details of the subscription to be created | M |

Table 12.2.1.3.2.6.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| subscription-ResponseType | 201 Created | In case of success the representation of the created subscription is returned. | M |
| error-Type | 4xx/5xx | In case of failure the error object is returned. | M |

12.2.1.3.2.6.3.2	DELETE

This method shall support the URI query parameters specified in table 12.2.1.3.2.6.3.2-1.

Table 12.2.1.3.2.6.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| consumerReferenceId | consumerReferenceId-QueryType | Identifies the consumer whose subscriptions shall be deleted | M |

This method shall support the request data structures specified in table 12.2.1.3.2.6.3.2-2 and the response data structures and response codes specified in table 12.2.1.3.2.6.3.2-3.

Table 12.2.1.3.2.6.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.2.1.3.2.5.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | n/a |
| error-Type | 4xx/5xx | In case of failure the error object is returned. | M |

12.2.1.3.2.7	Resource "/subscriptions/{subscriptionId}"

12.2.1.3.2.7.1	Description

This resource represents a subscription.

12.2.1.3.2.7.2	URI

The resource URI is:

Resource URI: {MnSRoot}/FaultSupervisionMnS/{MnSVersion}/subscriptions/{subscriptionId}

Table 12.2.1.3.2.7.2-1: URI variables

| Name |  | Definition |
| --- | --- | --- |
| MnSRoot |  | See clause 4.4 of TS 32.158 [15] |
| subscriptionId |  | Subscription identifier |

12.2.1.3.2.7.3	HTTP methods

12.2.1.3.2.7.3.1	DELETE

This method shall support the URI query parameters specified in table 12.2.1.3.2.7.3.1-1.

Table 12.2.1.3.2.6.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.2.1.3.2.7.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.7.3.1-3.

Table 12.2.1.3.2.7.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | Description | SQ |
| --- | --- | --- |
| n/a | n/a | n/a |

Table 12.2.1.3.2.7.3.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| error-Type | 4xx/5xx | In case of failure the error object is returned. | M |

12.2.1.3.2.8	Resource "/notificationSink"

12.2.1.3.2.8.1	Description

This resource represents a resource to which notifications are sent to.

12.2.1.3.2.8.2	URI

The resource URI is provided by the notification subscriber when creating the subscription.

12.2.1.3.2.8.3	HTTP methods

12.2.1.3.2.8.3.1	POST

This method shall support the URI query parameters specified in table 12.2.1.3.2.8.3.1-1.

Table 12.2.1.3.2.8.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | Description | Qualifier |
| --- | --- | --- | --- |
| n/a | n/a | n/a | n/a |

This method shall support the request data structures specified in table 12.2.1.3.2.8.3.1-2 and the response data structures and response codes specified in table 12.2.1.3.2.8.3.1-3.

Table 12.2.1.3.2.8.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type |  | Description |  | SQ |  |
| --- | --- | --- | --- | --- | --- |
| notifyNewAlarm-NotifType |  | Type in case a notifyNewAlarm notification is sent |  | M |  |
| notifyNewSecurityAlarm-NotifType |  | Type in case a notifyNewSecurityAlarm notification is sent |  | M |  |
| notifyAckStateChanged-NotifType |  | Type in case a notifyAckStateChanged notification is sent |  | M |  |
| notifyClearedAlarm-NotifType |  | Type in case a notifyClearedAlarm notification is sent |  | M |  |
| notifyAlarmListRebuilt-NotifType |  | Type in case a notifyAlarmListRebuilt notification is sent |  | M |  |
| notifyChangedAlarm-NotifType |  | Type in case a notifyChangedAlarm notification is sent |  | M |  |
| notifyComments-NotifType |  | Type in case a notifyComments notification is sent |  | M |  |
| notifyPotentialFaultyAlarmList-NotifType |  | Type in case a notifyPotentialFaultyAlarmList notification is sent |  | M |  |
| notifyCorrelatedNotificationChanged-NotifType |  | Type in case a notifyCorrelatedNotificationChanged notification is sent |  | M |  |
| notifyChangedAlarmGeneral-NotifType |  | Type in case a notifyChangedAlarmGeneral notification is sent |  | M |  |

Table 12.2.1.3.2.8.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | Response codes | Description | SQ |
| --- | --- | --- | --- |
| n/a | 204 No Content | In case of success no message body is returned | M |
| error-Type | 4xx/5xx | In case of failure the error object is returned. | M |

End of Change