3GPP TSG-SA5 Meeting #132e 	S5-204028

# e-meeting 17th 28th August 2020

# Source:	3GPP RAN3

# Title:	LS on propagation of user consent related information during Xn inter-PLMN handover

# Document for:	Information, Discussion

Agenda Item:	6.1

3GPP TSG-RAN WG3 Meeting #108-e	     R3-204378

E-meeting, 1 – 11 June, 2020

Title:	LS on propagation of user consent related information during Xn inter-PLMN handover

Response to:

Release:	Release 16

Work Item:	NR_SON_MDT-Core

Source:	RAN3

To:	SA3

Cc:	RAN2, SA5

Contact Person:

#### Name:	Håkon Helmers

E-mail Address:	hakon.helmers@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

According to current stage 3 agreements in the work on introduction of MDT in NG-RAN, the NG-RAN node receives user consent information per PLMN for management-based MDT from the CN in the NGAP Management Based MDT PLMN List IE only in the initial UE context setup. In case of Xn handover, in line with principle used for E-UTRAN MDT, this IE is propagated to the target NG-RAN node except for the case of inter-PLMN handover if the source NG-RAN node selects a serving PLMN in the target NG-RAN node which is not included in the Management Based MDT PLMN List.  The user consent information will be lost if the source NG-RAN selects a serving PLMN which is an equivalent PLMN but is not included in the Management Based MDT PLMN List.

RAN3 would like to ask SA3 confirmation whether SA3 would have any concern if the source NG-RAN node propagates the Management Based MDT PLMN List IE to the target NG-RAN node irrespective of whether the target PLMN is included or not included in the list.

2. Actions:

To SA3 : 	RAN3 kindly requests SA3 to provide feedback on whether the source NG-RAN node should be allowed, in case of inter-PLMN Xn handover, to propagate the Management Based MDT PLMN List IE to the target NG-RAN node irrespective of the target PLMN being included or not included in the list.

3. Dates of Next TSG-RAN WG3 Meetings:

3GPP TSG RAN WG3#109	-e	17 – 28 August, 2020		Online

3GPP TSG RAN WG3#110-e	2 – 13 November, 2020	Online