3GPP TSG-SA5 Meeting #132e 	S5-204196

# e-meeting 17th 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.422 | CR | 0338 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the procedure of SCG failure reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The report of SCG failure is introduced for dual connection scenairo in Rel-16. The procedure of SCG failure is missing in TS 32.422. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the procedure of SCG failure reporting. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure of SCG failure is missing in TS 32.422. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.9a, 4.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st of changes

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [4], 3GPP TS 32.101 [1] , 3GPP TS 23.501 [40], 3GPP TS 38.300 [42] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [4].

AS	Application Server

BGCF	Breakout Gateway Control Function

CSCF	Call Session Control Function

I-CSCF	Interrogating-CSCF

IM CN SS	IP Multimedia Core Network Subsystem

IMEI-TAC	IMEI Type Allocation Code P-CSCF	Proxy - CSCF

RCEF	RRC Connection Establishment Failure

RLF	Radio Link Failure

S-CSCF	Serving-CSCF

TAU	Tracking Area Update

SCG	Secondary Cell Group

2nd of changes

5.9a	Job type (CM)

The Job type is a conditional mandatory parameter. The condition is either MDT or RLF or RCEF data collection functionality is supported. It defines if a single trace job, a combined MDT and trace job or RLF report collection job or RCEF report collection job is activated. This parameter also defines the MDT mode. The Job type parameter is an enumerated type with the following values:

-	Immediate MDT only (0);

-	Logged MDT only (1);

-	Trace only (2);

-	Immediate MDT and Trace (3);

-	RLF reports only (4);

-	RCEF reports only (5).

-	Logged MBSFN MDT (6).

-	SCG failure reports only (7).

NOTE 1:	The Job type "RLF reports only" and "RCEF reports only" are applicable only in management based trace activation in E-UTRAN.

NOTE 2:	Logged MBSFN MDT is applicable only for activation in E-UTRAN.

NOTE 3:	The Job type "SCG failure reports only" is applicable only in management based trace activation in E-URTAN and NR.

3rd of changes

4.X	SCG failure reporting

4.X.1 Trace session activation for SCG failure reporting

SCG failure reporting is activated to an eNB/gNB, as a special Trace Session where the job type indicates SCG failure reporting only. The detailed procedure is shown in figure 4.X.1-1 where a UE experiences an SCG failure event.

When the Trace Seesion Activation for SCG failure reporting is started, the check of user consent is required by the network because of privacy and legal obligations. The steps to get user consent for management based MDT in clause 4.6.2 is reused in SCG failure reporting. If user consent is not avalible, network shall not configure UE to report location information within SCG failure report.

Figure 4.X.1-1 Example scenario for SCG failure reporting

When the eNB/gNB receives the Trace Session activation indicating SCG failure reporting only, the eNB/gNB shall start a Trace Session. This Trace Session shall collect only SCG failure reports received from the UE. The Trace Session activation message received from the management system shall contain the following information:

-	Trace Reference

-	Job type= SCG failure reporting only

-	IP address of the TCE

If a UE detects a SCG failure event, it collects certain information for SCG failure reports. Once the eNB/gNB retrieved the SCG failure report from the UE, it shall save it to the Trace Record. The Trace Record containing the SCG failure reports can be transferred to the TCE in the same mechanism as for normal subscriber and equipment trace or for MDT.

4.X.2 Trace session deactivation for SCG failure reporting

When the MN receives the indication from the Management System for trace session deactivation with the job type "SCG failure reporting only", it shall deactivate the trace session for the indicated trace reference of SCG failure reporting and stop SCG failure reporting to the TCE.

End of changes