3GPP TSG-SA5 Meeting #132e 	S5-204276

# e-meeting 17th 28th August 2020

# Source:	Huawei

# Title:	Management of stand-alone non-public networks

# Document for:	Approval

Agenda Item:	6.5.4

1	Decision/action requested

Discuss and approve on the proposal.

2	References

[1]	TS 28.557 Management of non-public networks; Stage 1 and stage 2 v0.0.0

[2]	TR 28.807 Study on management aspects of non-public networks v16.0.0

3	Rationale

It is proposed to add concept of management of stand-alone non-public networks in draft TS 28.557 [1] based on the corresponding content in TR 28.807 [2].

4	Detailed proposal

This document proposes the following changes in TS 28.557 [1].

1st Change

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[U]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[V]	5G-ACIA White paper: "5G Non-Public Networks for Industrial Scenarios", July 31, 2019.

[W]	3GPP TS 23.003: "Numbering, addressing and identification".

[X]	3GPP TS 28.530: "Concepts, use cases and requirements".

Next Change

4	Concepts and overview

4.3	Management of stand-alone non-public networks

An SNPN is deployed as an independent, isolated network. As shown in figure 4.3-1, all SNPN network functions are located inside the logical perimeter of the defined premises (e.g. factory) and the SNPN is separate from the public network. An optional connection to the public network services via the firewall, can be employed to enable access to public network services, such as voice, while within NPN coverage, see figure 1 in clause 5.2 of [V].

SNPN

Public Network Management

SNPN Management

PLMN

Figure 4.3-1: SNPN deployment and SNPN management

To manage a SNPN which is based on 3GPP-defined technologies, the standalone SNPN management system needs a dedicated NPN identifier. The combination of a PLMN ID and Network identifier (NID) is used to identify an SNPN. The NID supports two assignment models, see clause 5.30.2 of TS 23.501 [U]:

-	Locally managed NIDs are assumed to be self-managed by SNPNs (i.e. chosen individually by SNPNs) at deployment time (and may therefore not be unique) but use a different numbering space than the universally managed NIDs as defined in TS 23.003 [W].

-	Universally managed NIDs are assumed to be globally unique.

An SNPN, which includes 3GPP and non-3GPP segments, may be created for use of a NPN service customer (e.g. a private company). From management viewpoint, this means that the 3GPP and non-3GPP segments of this NPN are completely independent and separated from PLMN provided network functions. The SNPN operator has full management control over the exclusive SNPN network functions, i.e., 3GPP segment which includes non-public 5GC and/or non-public NG-RAN, and non-3GPP segment.

An SNPN, which includes 3GPP segments only, may be created for use of a NPN service customer (e.g. a private company). From management viewpoint, this means that the 3GPP segments of this NPN are completely independent and separated from PLMN provided network functions. The SNPN operator has full management control over the exclusive SNPN network functions, i.e., 3GPP segments which includes non-public 5GC and non-public NG-RAN.

The relations between SNPN related roles and the corresponding entities can be found in table 4.3-X.

-	The NG-RAN segment of the SNPN is deployed in well-defined areas (e.g. within vertical's premises, factory, along railroad tracks, etc.) to meet the vertical's service requirements.

-	The 5GC segment of the SNPN is deployed in vertical's premises. Alternatively, in case of virtualization of some 5GC network functions, the vertical may rely on one or more VISPs (see TS 28.530 [X], clause 4.8) to host its virtualized 5GC network functions, out of the vertical's premises.

-	Only the vertical's UEs are authorized to gain access to the SNPN.

Table 4.3-X Relations between SNPN related roles and corresponding entities

| Segment of SNPN | NPN operator | NPN service provider | NPN service customer |
| --- | --- | --- | --- |
| NG-RAN | Played by a vertical independently. | Played by a vertical independently. | Played by a private entity (e.g. vertical's employees, vertical's applications, or vertical's customers). |
| 5GC | Played by a vertical independently. (note 1) | Played by a vertical independently. | Played by a private entity (e.g. vertical's employees, vertical's applications, or vertical's customers). |
| NOTE 1: In case of virtualization of some 5GC network functions, Virtualization Infrastructure Service Provider (VISP) role may be played by a 3rd-party cloud service provider. NOTE 2: How dedicated licensed spectrum is obtained by the vertical to operate its SNPN is out of the scope of the present document. |  |  |  |

End of change