3GPP TSG-SA5 Meeting #132e 	S5-204529

# e-meeting 17th-28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 0015 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update the URI structure definition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2020-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | 1.URI structure for resources not representing managed object instances is defined as follows: {scheme}://{authority}/{root}/{MnSName}/{MnSVersion}/{MnSResourcePath} The following abbreviated URI structure is given for the sake of brevity, where the URI template {MnSRoot} is introduced that includes the scheme component, the two slash characters ("//") and the authority component. {MnSRoot}/{MnSName}/{MnSVersion}/{MnSResourcePath} Compared the full URI structure and the abbreviated URI, the {root} component is missing in the abbreviated URI. 2. The following description for {scheme}, only “http” and “https” are allowed, however, in TS 28.532, the allowed value for {protocal} (same as {scheme}) can be “http”, “https” and “wss”. {scheme} Scheme component "http" or "https" In addition to these changes the concept of MnsRoot is introduced in clause 4.4.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update the description for {MnSRoot} - Add allowed value “wss” for {scheme} component. - Add concept of MnsRoot to clause 4.4.2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The abbreviated URI structure is not same as full URI structure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.2, 4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-204436 |  |  |  |  |  |  |  |  |

1st Change

4.4.2	URI structure for resources representing managed object instances

URIs identifying resources representing managed object instances shall follow a structure given by

{scheme}://{URI-DN-prefix}/{root}/{MnSName}/{MnSVersion}/{URI-LDN}

with:

{scheme}		Scheme component "http" or "https"

{URI-DN-prefix}		Authority component (host identifier and optional TCP port), the host name is constructed from the DN prefix as defined in clause 4.2.3.

{root}		Part of the path component, allows specifying optional path segments for structuring the resource hierarchy on a HTTP server.

{MnSName}		Part of the path component, allows specifying an optional MnS name in a single path segment.

{MnSVersion}		Part of the path component, allows specifying an optional MnS version in a single path segment.

{URI-LDN}		Part of the path component, constructed from the LDN as defined in clause 4.2.3, one or more path segments.

As seen above, to construct the URI from a DN, it is necessary to map the "DNPrefixPlusRDNSeparator" as defined in clause 7.3 of [3], the “LocalDN” as defined in clause 7.3 of [3], and to add the additional optional path segments "/{root}/{MnSName}/{MnSVersion}".

To allow for a predictive mapping from a URI to the original DN it is necessary to specify "/{MnSName}/{MnSVersion}" in such a way that the beginning of the "LocalDN" can be unambigously identified.

Note it may be required when specifying a MnS to clearly identify the last RDN of "{URI-LDN}" and to use the following instead of "{URI-LDN}"

{URI-LDN-first-part}/{RDN}

or

{URI-LDN-first-part}/{className}={id}.

For the sake of brevity, "MnSRoot" is introduced that includes the "{scheme}" part, the two slash characters ("//"), the "{authority}" part, a single slash character ("/") and the "{root}" part. When using "{MnSRoot}" the abbreviated URI structure is given by

{MnSRoot}/{MnSName}/{MnSVersion}/{URI-LDN}

or

{MnSRoot}/{MnSName}/{MnSVersion}/{URI-LDN-first-part}/{className}={id}

It is recommended to use this abbreviated version of the URI structure when defining Management Services.

Next Change

4.4.3	URI structure for resources not representing managed object instances

URIs identifying other resources shall follow a structure given by

{scheme}://{authority}/{root}/{MnSName}/{MnSVersion}/{MnSResourcePath}

with:

{scheme}		Scheme component "http" or "https" or "wss"

{authority}	Authority component (host identifier and optional TCP port)

{root}		Part of the path component, allows specifying optional path segments for structuring the resource hierarchy on a HTTP server.

{MnSName}		Part of the path component, specifies the mandatory MnS name in a single path segment.

{MnSVersion}		Part of the path component, specifies the mandatory MnS version in a single path segment.

{MnSResourcePath}		Part of the path component, one or more path segments, specifies a resource of the MnS

For the sake of brevity, {MnSRoot} is introduced that includes the "{scheme}" part, the two slash characters ("//"), the "{authority}" part, a single slash character ("/") and the "{root}" part. When using "{MnSRoot}" the abbreviated URI structure is given by

{MnSRoot}/{MnSName}/{MnSVersion}/{MnSResourcePath}

It is recommended to use this abbreviated version of the URI structure when defining Management Services.

End of Change