3GPP TSG-SA5 Meeting #132e 	S5-204227

# e-meeting 17th 28th August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0242 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add EPC/ePDG interworking procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16, 5GS_Ph1-DCH |  |  |  |  |  | Date: |  |  | 2020-08-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | EPC/ePDG interworking with 5GS procedures are missing in TS 32.255. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Following interworking procedures are added in TS 32.255 - Handover from EPC/ePDG to 5GS - Handover from 5GS to EPC/ePDG |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The charging for EPC/ePDG and 5GS interworking scenario is not covered in TS 32.255. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.2.2.11.x			Handover from EPC/ePDG to 5GS

The following figure 5.2.2.11.x.1 describes a PDU session charging handover from EPC/ePDG to 5GS for non-roaming scenario.

Figure 5.2.2.11.x.1: PDU session charging handover from EPC/ePDG to 5GS

0. One or more PDN Connections have been established between the UE and the EPC/ePDG via untrusted non-3GPP access as specified in clauses 7.2.4 and 7.6.3 of TS 23.402 with some modification described in 4.11.4.3.3 and 4.11.4.3.5 in TS 23.502.

1. UE register to 5GC via NG RAN.

2. The UE initiates a UE requested PDU Session Establishment via 3GPP Access acording to clause 4.3.2.2 of TS 23.502 and includes the "Existing PDU Session" indication or "Existing Emergency PDU Session" and the PDU Session ID.

2ch-a. PGW-C+SMF sends Charging Data Request [Update] to CHF with RAT type change and serving node change of 5GS.

2ch-b. The CHF updates for this PDU session

2ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the PGW-C+SMF.

NOTE: The RAT type change triggers, if enabled, applied during the PDU session establishment via the 3GPP access network.

3. Step 10-14, in procedure of UE requested PDU Session Establishment as described in clause 4.3.2.2 of TS 23.502.

4. The combined PGW+SMF/UPF initiates a PDN GW initiated Resource Allocation Deactivation with GTP on S2b as described in TS 23.402 clause 7.9.2 to release the EPC and ePDG resources when S6b is used.

5.2.2.11.y			Handover from 5GS to EPC/ePDG

The following figure 5.2.2.11.y.1 describes a PDU session charging handover from 5GS to EPC/ePDG for non-roaming scenario.

Figure 5.2.2.11.y.1: PDU session charging handover from 5GS to EPC/ePDG

- One or more PDU Sessions have been established between the UE and the SMF/UPF via NG-RAN.

- UE selects ePDG as specified in TS 23.502.

- Step A1 in procedure of the UE initiates a Handover Attach procedure as described in TS 23.402 clause 8.6.2.1.

2ch-a. PGW-C+SMF sends Charging Data Request [Update] to CHF with RAT type change and serving node change of 5GS.

2ch-b. The CHF updates for this PDU session.

2ch-c. The CHF acknowledges by sending Charging Data Response [Update] to the PGW-C+SMF.

NOTE: The RAT type change triggers, if enabled, applied during the PDU session establishment via the 3GPP access network.

- Step C1, in procedure of the UE initiates a Handover Attach procedure as described in TS 23.402 clause 8.6.2.1, except step 11 of referenced figure 8.2.3-1 that corresponds to the release of resources.

- The combined PGW+SMF/UPF initiates a network requested PDU Session Release via 3GPP access as described in clause 4.11.4.2 step 3 of TS 23.502.

End of change