# 3GPP TSG SA WG5 Meeting #139e	S5-215198

# Online, , 11 Oct 2021- 20 Oct 2021

# Source:	Samsung

# Title:	pCR 28.104 Traffic projection usecase and requirements

# Document for:	Approval

Agenda Item:	6.4.18

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

The pCR provides use case for traffic projection.

# 4	Detailed proposal

First modification

# x	MDA Scenarios

## x.1	General

## x.2	MDA capabilities

### x.2.1	SLS Analysis

#### x.2.1.1	Description

This capability deal with enabling various functionalities related with SLS analysis.

#### x.2.1.2	Use cases

##### x.2.1.2.1         Network slice traffic projection

It is desirable to use uses MDAS to get the network slice traffic projections including individual traffic projections on each of the constituent network functions instances present in the slice. The individual traffic projections can be used for better resource management of the slice. For example, resources can be pre-configured considering the projected traffic on the slice.

#### x.2.1.3	Requirements

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-TRA_MDA-CON-x | The MDAS producer should have a capability allowing the authorized consumer to request the slice traffic analytics report describing traffic projection of the slice including its constituent network functions. | Network slice traffic projection |
| REQ-TRA_MDA-CON-x | The MDAS producer should have a capability to provide the slice traffic analytics report describing the traffic projections for each constituent network function instance in the slice. | Network slice traffic projection |
| REQ-TRA_MDA-CON-x | The slice traffic analytics report providing traffic projection for the slice may include the following information: - Projected uplink and downlink throughput on each User Plane Function instance (UPF) present in the slice. - Projected number of Packet Data Unit (PDU) session for each Session Management Function (SMF) instance present in the slice. - Projected number of UE or Registered subscriptions for each AMF instance present in the slice. - Projected maximum packet size for each UPF instance present in the slice. - Projected UE uplink and downlink throughput on each gNodeB (gNB) instance present in the slice. - Projected number of UE for each gNB/NR cell instance present in the slice. | Network slice traffic projection |

End of First modification