3GPP TSG-SA5 Meeting #139-e 	S5-215324

# e-meeting, 11 - 20 October 2021 															Revision of S5-20xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0354 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of QoS Monitoring to Assist URLLC Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_URLLC |  |  |  |  |  | Date: |  |  | 2021-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the QoS Monitoring to Assist URLLC Service, the SMF may report the packet delay measurement per QoS Flow per UE to CHF. The coresponding parameters should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the related parameters for QoS Monitoring to Assist URLLC Service |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Can not support the URLCC charging. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.1,6.1.6.2.2.8,6.1.8,7.2,A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.255 CR 0338 TS 32.298 CR 0881 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.1.6.1	General

This subclause specifies the application data model supported by the API.

The Nchf_ConvergedCharging Service API allows the NF consumer to consume the converged charging service from the CHF as defined in 3GPP TS 32.290 [58].

Table 6.1.6.1-1 specifies the data types defined for the ConvergedCharging service based interface protocol.

Table 6.1.6.1-1: Nchf_ ConvergedCharging specific Data Types

| Data type |  | Section defined |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ChargingDataRequest |  | 6.1.6.2.1.1 6.1.6.2.2.1 |  | Describes the attributes of Charging Data Request to CHF for initial, update and termination of the charging session. |  |  |  |
| ChargingDataResponse |  | 6.1.6.2.1.2 6.1.6.2.2.2 |  | Describes the attributes of Charging Data Response from CHF on charging session initial, update and termination. |  |  |  |
| ChargingNotifyRequest |  | 6.1.6.2.1.3 |  | Describes Notifications about events that occurred in request message. |  |  |  |
| ChargingNotifyResponse |  | 6.1.6.2.1.16 |  | Describes the response of notification. |  |  |  |

Table 6.1.6.1-2 specifies data types re-used by the Nchf_ConvergedCharging service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nchf_ConvergedCharging service based interface.

Table 6.1.6.1-2: Nchf_ConvergedCharging re-used Data Types

| Data type |  | Reference |  | Comments |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Supi |  | 3GPP TS 29.571 [371] |  | The identification of the user (i.e. IMSI, NAI, GLI, GCI). (NOTE 1) |  |  |  |
| Uint32 |  | 3GPP TS 29.571 [371] |  | Unsigned 32-bit integers |  |  |  |
| Uint64 |  | 3GPP TS 29.571 [371] |  | Unsigned 64-bit integers |  |  |  |
| PduSessionId |  | 3GPP TS 29.571 [371] |  | The identification of the PDU session. |  |  |  |
| PduSessionType |  | 3GPP TS 29.571 [371] |  | the type of a PDU session |  |  |  |
| Uri |  | 3GPP TS 29.571 [371] |  | String providing an URI |  |  |  |
| AccessType |  | 3GPP TS 29.571 [371] |  | The identification of the type of access network. |  |  |  |
| DateTime |  | 3GPP TS 29.571 [371] |  | The time. |  |  |  |
| ChargingId |  | 3GPP TS 29.571 [371] |  | Charging identifier allowing correlation of charging information |  |  |  |
| RatType |  | 3GPP TS 29.571 [371] |  | The identification of the RAT type. |  |  |  |
| RatingGroup |  | 3GPP TS 29.571 [371] |  | The identification of the rating group |  |  |  |
| Ipv4Addr |  | 3GPP TS 29.571 [371] |  | Ipv4 address. |  |  |  |
| Ipv6Prefix |  | 3GPP TS 29.571 [371] |  | The Ipv6 prefix allocated for the user. |  |  |  |
| Ipv6Addr |  | 3GPP TS 29.571 [371] |  | Ipv6 Address. |  |  |  |
| Pei |  | 3GPP TS 29.571 [371] |  | The Identification of a Permanent Equipment. |  |  |  |
| TimeZone |  | 3GPP TS 29.571 [371] |  | Time zone information |  |  |  |
| NfInstanceId |  | 3GPP TS 29.571 [371] |  | String uniquely identifying a NF instance. |  |  |  |
| Gpsi |  | 3GPP TS 29.571 [371] |  | String identifying a Gpsi |  |  |  |
| DefaultQosInformation |  | 3GPP TS 29.571 [371] |  | Identifies the information of the default QoS. |  |  |  |
| SubscribedDefaultQos |  | 3GPP TS 29.571 [371] |  | subscribed default QoS. |  |  |  |
| AuthorizedDefaultQos |  | 3GPP TS 29.512 [302] |  | Authorized default QoS. |  |  |  |
| Ambr |  | 3GPP TS 29.571 [371] |  | Aggregate Maximum Bit rate |  |  |  |
| QosData |  | 3GPP TS 29.512 [302] |  | Contains QoS parameters |  |  |  |
| UserLocation |  | 3GPP TS 29.571 [371] |  | User location information |  |  |  |
| PlmnId |  | 3GPP TS 29.571 [371] |  | PLMN id |  |  |  |
| Guami |  | 3GPP TS 29.571 [371] |  | Globally Unique AMF Identifier |  |  |  |
| DurationSec |  | 3GPP TS 29.571 [371] |  | Identifies a period of time in units of seconds. |  |  |  |
| Snssai |  | 3GPP TS 29.571 [371] |  | SNSSAI |  |  |  |
| ProblemDetails |  | 3GPP TS 29.571 [371] |  | additional details of the error |  |  |  |
| ServiceId |  | 3GPP TS 29.571 [371] |  | Identifier of service |  |  |  |
| SscMode |  | 3GPP TS 29.571 [371] |  | SSC Mode type |  |  |  |
| PresenceInfo |  | 3GPP TS 29.571 [371] |  | PRA information including PRAId, PRA element list and PRA status |  |  |  |
| Qfi |  | 3GPP TS 29.571 [371] |  | QoS flow identifier designated as "Qfi". |  |  |  |
| AmfId |  | 3GPP TS 29.571 [371] |  | AMF identifier |  |  |  |
| Dnn |  | 3GPP TS 29.571 [371] |  | Data Network Name |  |  |  |
| GroupId |  | 3GPP TS 29.571 [371] |  | Network internal Identifier for a group of IMSIs |  |  |  |
| ExternalGroupId |  | 3GPP TS 29.571 [371] |  | External Group Identifier for one or more subscriptions associated to a group of IMSIs |  |  |  |
| Bytes |  | 3GPP TS 29.571 [371] |  | String with format "byte" |  |  |  |
| Tai |  | 3GPP TS 29.571 [371] |  | Tracking Area Identifier |  |  |  |
| Area |  | 3GPP TS 29.571 [371] |  | List of TACs or Operator specific codes |  |  |  |
| CoreNetworkType |  | 3GPP TS 29.571 [371] |  | 5GC or EPC |  |  |  |
| ServiceAreaRestriction |  | 3GPP TS 29.571 [371] |  | Service Area restriction |  |  |  |
| GlobalRanNodeId |  | 3GPP TS 29.571 [371] |  | Global RAN Node Id |  |  |  |
| QosCharacteristics |  | 3GPP TS 29.512 [302] |  | Map of QoS characteristics for non standard 5QIs and non-preconfigured 5QIs. |  |  |  |
| SupportedFeatures |  | 3GPP TS 29.571 [371] |  | See 3GPP TS 29.500 [4] clause 6.6 |  |  |  |
| NsiLoadLevelInfo |  | 3GPP TS 29.520 [306] |  | Represents the load level information for an S-NSSAI and the associated network slice instance |  |  |  |
| ServiceExperienceInfo |  | 3GPP TS 29.520 [306] |  | ServiceExperience |  |  |  |
| ApplicationChargingId |  | 3GPP TS 29.571 [371] |  | Application provided charging identifier allowing correlation of charging information. |  | AF_Charging_Identifier |  |
| SharingLevel |  | 3GPP TS 28.541 [254] |  | Ressources sharing level |  |  |  |
| MobilityLevel |  | 3GPP TS 28.541 [254] |  | UE mobility Level |  |  |  |
| SsT |  | 3GPP TS 28.541 [254] |  | Slice Service type (SST) |  |  |  |
| Support |  | 3GPP TS 28.541 [254] |  | Supported, Not Supported indicator |  |  |  |
| Float |  | 3GPP TS 29.571 [371] |  | Number with format "float" |  |  |  |
| MaPduIndication |  | 3GPP TS 29.512 [302] |  | MA PDU session indication |  | ATSSS |  |
| AtsssCapability |  | 3GPP TS 29.571 [371] |  | ATSSS capabilities |  | ATSSS |  |
| SteeringFunctionality |  | 3GPP TS 29.571 [371] |  | Steering functionalities for MA PDU session |  | ATSSS |  |
| SteeringMode |  | 3GPP TS 29.512 [302] |  | Steering mode for MA PDU session |  | ATSSS |  |
| OperationalState |  | 3GPP TS 28.623 [257] |  | Operational state |  |  |  |
| AdministrativeState |  | 3GPP TS 28.623 [257] |  | Administrative state |  |  |  |
| RanNasRelCause |  | 3GPP TS 29.512 [302] |  | Indicates the RAN or NAS release cause code information. |  | EnhancedDiagnostics |  |
| Ecgi |  | 3GPP TS 29.571 [371] |  | E-UTRA Cell Id |  |  |  |
| Ncgi |  | 3GPP TS 29.571 [371] |  | NR Cell Id |  |  |  |
| QosMonitoringReport |  | 3GPP TS 29.512 [302] |  | Indicates the service data flows QoS Monitoring. |  | QoSMonitoring |  |
| NOTE 1: A SUPI containing GLI or GCI is used to support 5G-RG and FN-RG in scenarios of wireline network. |  |  |  |  |  |  |  |

Next change

6.1.6.2.2.8	Type PDUSessionInformation

Table 6.1.6.2.2.8-1: Definition of type PDUSessionInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| networkSlicingInfo | NetworkSlicingInfo | OM | 0..1 | information of network slice serving the PDU session |  |
| pduSessionID | PduSessionId | M | 1 |  |  |
| pduType | PduSessionType | OM | 0..1 | type of the PDU session |  |
| sscMode | SscMode | OC | 0..1 | information of SSC Mode type. |  |
| hPlmnId | PlmnId | OC | 0..1 | PLMN identifier of the home network |  |
| servingNetworkFunctionID | ServingNetworkFunctionID | OC | 0..1 | This field holds serving Network Function identifier. |  |
| servingCNPlmnId | PlmnId | OC | 0..1 | Serving Core Network Operator PLMN ID selected by the UE in shared networks. |  |
| ratType | RatType | OC | 0..1 | the RAT Type of the PDU session |  |
| mAPDUNon3GPPRATType | RatType | OC | 0..1 | the RAT Type of non-3GPP access for the MA PDU session | ATSSS |
| dnnId | Dnn | M | 1 | a Data Network Name |  |
| dnnSelectionMode | DnnSelectionMode | OC | 0..1 | This field indicates how the DNN was selected. |  |
| chargingCharacteristics | string | OC | 0..1 | the Charging Characteristics for this PDU session. It carries the value in hexadecimal representation Pattern: '^[0-9a-fA-F]{1,4}$' |  |
| chargingCharacteristicsSelectionMode | ChargingCharacteristicsSelectionMode | OC | 0..1 | information about how the "Charging Characteristics" was selected. |  |
| startTime | DateTime | OC | 0..1 | the UTC time which represents the start of a PDU session at the SMF |  |
| stopTime | DateTime | OC | 0..1 | the UTC time which represents the stop of a PDU session at the SMF |  |
| 3gppPSDataOffStatus | 3GPPPSDataOffStatus | OC | 0..1 | This field holds the 3GPP Data off Status when UE’s 3GPP Data Off status is Activated or Deactivated. |  |
| sessionStopIndicator | boolean | OC | 0..1 | This field indicates to the CHF that the PDU session has been terminated. |  |
| pduAddress | PDUAddress | OC | 0..1 | Group of user ip address/prefix |  |
| diagnostics | Diagnostics | OC | 0..1 | provides a detailed cause value from SMF. |  |
| enhancedDiagnostics | EnhancedDiagnostics5G | OC | 0..N | provides a more detailed cause value from SMF. | EnhancedDiagnostics |
| authorizedQoSInformation | AuthorizedDefaultQos | OC | 0..1 | This field holds the authorized QoS applied to PDU session. |  |
| subscribedQoSInformation | SubscribedDefaultQos | OC | 0..1 | This field holds the subscribed Default QoS |  |
| authorizedSessionAMBR | Ambr | OC | 0..1 | This field holds the authorized session-AMBR. |  |
| subscribedSessionAMBR | Ambr | OC | 0..1 | This field holds the subscribed session-AMBR. |  |
| mAPDUSessionInformation | MAPDUSessionInformation | OC | 0..1 | This field holds the MA PDU session information. | ATSSS |
| redundantTransmissionType | RedundantTransmissionType | OC | 0..1 | Indicates the redundant transmission type. If this field isn’t present, it should be seen as a non-redundant transmission. | URLLC |
| qosMonitoringInformation | array(QosMonitoringReport) | OC | 0..1 | This field holds QoS Monitoring reporting information. It may be present when the URLLC is supported. | QoSMonitoring |
| pDUSessionPairID | Uint32 | OC | 0..1 | This field identifies the two redundant PDU Sessions that belong together for dual connectivity based end to end redundant user plane paths type. | URLLC |

Next change

6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nchf_ConvergedCharging API. They shall be negotiated using the extensibility mechanism defined in subclause 6.6 of 3GPP TS 29.500 [299].

Table 6.1.8-1: Supported Features

| Feature number |  | Feature Name |  | Description |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | CHFCQM |  | CHF-controlled quota management i.e. support for temporary offline |  |
| 2 |  | AF_Charging_Identifier |  | Indicates the support of long character strings as charging identifiers. |  |
| 3 |  | 5GIEPC_CH |  | 5GS interworking with EPC |  |
| 4 |  | ATSSS |  | This feature indicates support of Access Traffic Steering, Switching, Splitting (ATSSS). |  |
| 5 |  | ETSUN |  | This feature indicates support of Enhancing Topology of SMF and UPF in 5G Networks (ETSUN). |  |
| 6 |  | EnhancedDiagnostics |  | Support the enhanced diagnostics |  |
| 7 |  | AMF_subs_PRA |  | PRA(s) subscription by CHF in AMF |  |
| 8 |  | FilterRuleList |  | Support of multiple filter rules in the final unit indication |  |
| 9 |  | TEI17_NIESGU |  | This feature indicates support of GERAN/UTRAN access |  |
| 10 |  | IMS |  | This feature indicates support of IMS. |  |
| 11 |  | QoSMonitoring |  | This feature indicates support of QoS Monitoring |  |

Next change

7.2	Bindings for 5G data connectivity

Table 7.2-1: Bindings of 5G data connectivity CDR field, Information Element and Resource Attribute

| Information Element |  | CDR Field |  | Resource Attribute |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | ChargingDataRequest |  |
| Supported Features |  | - |  | /supportedFeatures |  |
| Multiple Unit Usage |  | List of Multiple Unit Usage |  | /multipleUnitUsage |  |
| UPF ID |  | UPF ID |  | /multipleUnitUsage/uPFID |  |
| multi-homed PDU address |  | Multi-homed PDU address |  | /multipleUnitUsage/multihomedPDUAddress |  |
| Used Unit Container |  | Used Unit Container |  | /multipleUnitUsage/usedUnitContainer |  |
| PDU Container Information |  | PDU Container Information |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation |  |
| Time of First Usage |  | Time of First Usage |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/timeofFirstUsage |  |
| Time of Last Usage |  | Time of Last Usage |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/timeofLastUsage |  |
| QoS Information |  | QoS Information |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/qoSInformation |  |
| QoS Characteristics |  | QoS Characteristics |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/qoSCharacteristics |  |
| AF Charging Identifier |  | AF Charging Identifier |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/afChargingIdentifier |  |
| AF Charging Id String |  | AF Charging Id String |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/afChargingIdString |  |
| User Location Information |  | User Location Information |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/userLocationInformation |  |
| UE Time Zone |  | UE Time Zone |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/uetimeZone |  |
| RAT Type |  | RAT Type |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/rATType |  |
| Serving Network Function ID |  | Serving Network Function ID |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/servingNodeID |  |
| Presence Reporting Area Information |  | Presence Reporting Area Information |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/ presenceReportingAreaInformation |  |
| 3GPP PS Data Off Status |  | 3GPP PS Data Off Status |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/3gppPSDataOffStatus |  |
| MA PDU Steering functionality |  | MA PDU Steering functionality |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/mAPDUSteeringFunctionality |  |
| MA PDU Steering mode |  | MA PDU Steering mode |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/mAPDUSteeringMode |  |
| Sponsor Identity |  | Sponsor Identity |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/sponsorIdentity |  |
| Application Service Provider Identity |  | Application Service Provider Identity |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/applicationserviceProviderIdentity |  |
| Charging Rule Base Name |  | Charging Rule Base Name |  | /multipleUnitUsage/usedUnitContainer/pDUContainerInformation/chargingRuleBaseName |  |
| PDU Session Charging Information |  | PDU Session Charging Information |  | /pDUSessionChargingInformation |  |
| Charging Id |  | Charging Id |  | /pDUSessionChargingInformation/chargingId |  |
| Home Provided ChargingId |  | Home Provided ChargingId |  | /pDUSessionChargingInformation/ homeProvidedChargingId |  |
| User Information |  | User Information |  | /pDUSessionChargingInformation/ userInformation |  |
| User Identifier |  | User Identifier |  | /pDUSessionChargingInformation/userInformation/servedGPSI |  |
| User Equipment Info |  | User Equipment Info |  | /pDUSessionChargingInformation/userInformation/ servedPEI |  |
| Unauthenticated Flag |  | Unauthenticated Flag |  | /pDUSessionChargingInformation/userInformation/ unauthenticatedFlag |  |
| Roamer In Out |  | Roamer In Out |  | /pDUSessionChargingInformation/userInformation/ roamerInOut |  |
| User Location Info |  | User Location Info |  | /pDUSessionChargingInformation/ userLocationinfo |  |
| MA PDU Non 3GPP User Location info |  | MA PDU Non 3GPP User Location info |  | /pDUSessionChargingInformation/mAPDUNon3GPPUserLocationInfo |  |
| UE Time Zone |  | UE Time Zone |  | /pDUSessionChargingInformation/uEtimeZone |  |
| Presence Reporting Area Information |  | Presence Reporting Area Information |  | /pDUSessionChargingInformation/ presenceReportingAreaInformation |  |
| PDU Session Information |  | PDU Session Information |  | /pDUSessionChargingInformation/pduSessionInformation |  |
| PDU Session ID |  | PDU Session ID |  | /pDUSessionChargingInformation/pduSessionInformation/pduSessionID |  |
| Network Slice Instance Identifier |  | Network Slice Instance Identifier |  | /pDUSessionChargingInformation/pduSessionInformation/networkSlicingInfo |  |
| PDU Type |  | PDU Type |  | /pDUSessionChargingInformation /pduSessionInformation/pduType |  |
| PDU Address |  | PDU Address |  | /pDUSessionChargingInformation /pduSessionInformation/pduAddress |  |
| PDU IPv4 Address |  | PDU IPv4 Address |  | /pDUSessionChargingInformation/pduSessionInformation/pduAddress/pduIPv4Address |  |
| PDU IPv6 Address with prefix |  | PDU IPv6 Address with prefix |  | /pDUSessionChargingInformation/pduSessionInformation/pduAddress/pduIPv6Addresswithprefix |  |
| PDU Address prefix length |  | PDU Address prefix length |  | /pDUSessionChargingInformation /pduSessionInformation/pduAddress/pduAddressprefixlength |  |
| IPv4 Dynamic Address Flag |  | IPv4 Dynamic Address Flag |  | /pDUSessionChargingInformation /pduSessionInformation/pduAddress/ iPv4dynamicAddressFlag |  |
| IPv6 Dynamic Address Flag |  | IPv6 Dynamic Prefix Flag |  | pDUSessionChargingInformation /pduSessionInformation/pduAddress/ iPv6dynamicPrefixFlag |  |
| Additional PDU IPv6 Prefixes |  | Additional PDU IPv6 Prefixes |  | pDUSessionChargingInformation /pduSessionInformation/pduAddress/ addIpv6AddrPrefixes |  |
| SSC Mode |  | SSC Mode |  | /pDUSessionChargingInformation /pduSessionInformation/sscMode |  |
| MA PDU session information |  | MA PDU session information |  | /pDUSessionChargingInformation /pduSessionInformation/mAPDUSessionInformation |  |
| MA PDU session indicator |  | MA PDU session indicator |  | /pDUSessionChargingInformation /pduSessionInformation/mAPDUSessionInformation/mAPDUSessionIndicator |  |
| ATSSS capability |  | ATSSS capability |  | /pDUSessionChargingInformation /pduSessionInformation/mAPDUSessionInformation/aTSSSCapability |  |
| SUPI PLMN ID |  | SUPI PLMN ID |  | /pDUSessionChargingInformation /pduSessionInformation/hPlmnId |  |
| Serving Network Function ID |  | Serving Network Function ID |  | /pDUSessionChargingInformation /pduSessionInformation/ servingNetworkFunctionID |  |
| Serving CN PLMN ID |  | Serving CN PLMN ID |  | /pDUSessionChargingInformation/pduSessionInformation/servingCNPlmnId |  |
| RAT Type |  | RAT Type |  | /pDUSessionChargingInformation /pduSessionInformation/ratType |  |
| MA PDU Non 3GPP RAT Type |  | MA PDU Non 3GPP RAT Type |  | /pDUSessionChargingInformation /pduSessionInformation/mAPDUNon3GPPRATType |  |
| Data Network Name Identifier |  | Data Network Name Identifier |  | /pDUSessionChargingInformation /pduSessionInformation/dnnid |  |
| DNN Selection Mode |  | DNN Selection Mode |  | /pDUSessionChargingInformation /pduSessionInformation/dNNselectionMode |  |
| Authorized QoS information |  | Authorized Qos Information |  | /pDUSessionChargingInformation /pduSessionInformation/authorized qoSInformation |  |
| Subscribed QoS Information |  | Subscribed QoS Information |  | /pDUSessionChargingInformation /pduSessionInformation/subscribedQoSInformation |  |
| Authorized Session-AMBR |  | Authorized Session-AMBR |  | /pDUSessionChargingInformation /pduSessionInformation/authorizedSessionAMBR |  |
| Subscribed Session-AMBR |  | Subscribed Session-AMBR |  | /pDUSessionChargingInformation /pduSessionInformation/subscribedSessionAMBR |  |
| Charging Characteristics |  | Charging Characteristics |  | /pDUSessionChargingInformation /pduSessionInformation/ chargingCharacteristics |  |
| Charging Characteristics Selection Mode |  | Charging Characteristics Selection Mode |  | /pDUSessionChargingInformation /pduSessionInformation/chargingCharacteristicsSelectionMode |  |
| PDU session start Time |  | PDU session start Time |  | /pDUSessionChargingInformation /pduSessionInformation/startTime |  |
| PDU session stop Time |  | PDU session stop Time |  | /pDUSessionChargingInformation /pduSessionInformation/stopTime |  |
| Diagnostics |  | Diagnostics |  | /pDUSessionChargingInformation /pduSessionInformation/diagnostics |  |
| Enhanced Diagnostics |  | Enhanced Diagnostics |  | /pDUSessionChargingInformation /pduSessionInformation/enhancedDiagnostics |  |
| 3GPP PS Data Off Status |  | 3GPP PS Data Off Status |  | /pDUSessionChargingInformation /pduSessionInformation/3gppPSDataOffStatus |  |
| Session Stop Indicator |  | Session Stop Indicator |  | /pDUSessionChargingInformation /pduSessionInformation/sessionStopIndicator |  |
| Redundant Transmission Type |  | Redundant Transmission Type |  | /pDUSessionChargingInformation /pduSessionInformation/redundantTransmissionType |  |
| PDU Session Pair ID |  | PDU Session Pair ID |  | /pDUSessionChargingInformation /pduSessionInformation/pDUSessionPairID |  |
| Qos Monitoring Information |  | Qos Monitoring Information |  | /pDUSessionChargingInformation /pduSessionInformation/qosMonitoringInformation |  |
| Unit Count Inactivity Timer |  | - |  | /pDUSessionChargingInformation/unitCountInactivityTimer |  |
| RAN Secondary RAT Usage Report |  | RAN Secondary RAT Usage Report |  | /pDUSessionChargingInformation/rANSecondaryRATUsageReport |  |
| NG RAN Secondary RAT Type |  | NG RAN Secondary RAT Type |  | /pDUSessionChargingInformation/rANSecondaryRATUsageReport/rANSecondaryRATType |  |
| Qos Flows Usage Reports |  | Qos Flows Usage Reports |  | /pDUSessionChargingInformation/rANSecondaryRATUsageReport/qosFlowsUsageReports |  |
| Roaming QBC information |  | Roaming QBC information |  | /roamingQBCInformation |  |
| Multiple QFI container |  | Multiple QFI container |  | /roamingQBCInformation/multipleQFIcontainer |  |
| Triggers |  | Triggers |  | /roamingQBCInformation/multipleQFIcontainer/triggers |  |
| Trigger Timestamp |  | Trigger Timestamp |  | /roamingQBCInformation/multipleQFIcontainer/triggerTimestamp |  |
| Time |  | Time |  | /roamingQBCInformation/multipleQFIcontainer/time |  |
| Total Volume |  | Total Volume |  | /roamingQBCInformation/multipleQFIcontainer/totalVolume |  |
| Uplink Volume |  | Uplink Volume |  | /roamingQBCInformation/multipleQFIcontainer/uplinkVolume |  |
| Downlink Volume |  | Downlink Volume |  | /roamingQBCInformation/multipleQFIcontainer/downlinkVolume |  |
| Local Sequence Number |  | Local Sequence Number |  | /roamingQBCInformation/multipleQFIcontainer/localSequenceNumber |  |
| QFI Container information |  | QFI Container information |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation |  |
| QoS Flow Id |  | QoS Flow Id |  | /roamingQBCInformation/multipleQFIcontainer/ qFIContainerInformation/qFI |  |
| Time of First Usage |  | Time of First Usage |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/ timeofFirstUsage |  |
| Time of Last Usage |  | Time of Last Usage |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/timeofLastUsage |  |
| QoS Information |  | QoS Information |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/qoSInformation |  |
| QoS Characteristics |  | QoS Characteristics |  | /roamingQBCInformation/multipleQFIcontainer/ qFIContainerInformation/qoSCharacteristics |  |
| User Location Information |  | User Location Information |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/ userLocationInformation |  |
| UE Time Zone |  | UE Time Zone |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/uetimeZone |  |
| Presence Reporting Area Information |  | Presence Reporting Area Information |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/presenceReportingAreaInformation |  |
| RAT Type |  | RAT Type |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/rATType |  |
| Report Time |  | Report Time |  | /roamingQBCInformation/multipleQFIcontainer/ qFIContainerInformation/reportTime |  |
| Serving Network Function ID |  | Serving Network Function ID |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/ servingNetworkFunctionID |  |
| 3GPP PS Data Off Status |  | 3GPP PS Data Off Status |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/3gppPSDataOffStatus |  |
| EPS bearer Charging Id |  | EPS bearer Charging Id |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/3gppChargingId |  |
| Diagnostics |  | Diagnostics |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/diagnostics |  |
| Enhanced Diagnostics |  | Enhanced Diagnostics |  | /roamingQBCInformation/multipleQFIcontainer/qFIContainerInformation/enhancedDiagnostics |  |
| UPF ID |  | UPF ID |  | /roamingQBCInformation/uPFID |  |
| Roaming Charging Profile |  | Roaming Charging Profile |  | /roamingQBCInformation/roamingChargingProfile |  |
| Trigger |  | Trigger |  | /roamingQBCInformationroamingChargingProfile/trigger |  |
| Partial record method |  | Partial record method |  | /roamingQBCInformation/roamingChargingProfile/partialRecordMethod |  |
|  |  |  |  | ChargingDataResponse |  |
| Supported Features |  | - |  | /supportedFeatures |  |
| Multiple Unit information |  | - |  | /multipleUnitInformation |  |
| UPF ID |  | - |  | /multipleUnitInformation/uPFID |  |
| PDU Session Charging Information |  | - |  | /pDUSessionChargingInformation |  |
| Presence Reporting Area Information |  | - |  | /pDUSessionChargingInformation/ presenceReportingAreaInformation |  |
| Unit Count Inactivity Timer |  | - |  | /pDUSessionChargingInformation/unitCountInactivityTimer |  |
| Roaming QBC information |  | - |  | /roamingQBCInformation |  |
| Roaming Charging Profile |  | - |  | /roamingQBCInformation/roamingChargingProfile |  |

Next change

A.2	Nchf_ConvergedCharging API

openapi: 3.0.0

info:

title: Nchf_ConvergedCharging

version: 3.1.0-alpha.1

description: |

ConvergedCharging Service    © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: >

3GPP TS 32.291 V17.0.0: Telecommunication management; Charging management;

5G system, charging service; Stage 3.

url: 'http://www.3gpp.org/ftp/Specs/archive/32_series/32.291/'

servers:

- url: '{apiRoot}/nchf-convergedcharging/v3'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in subclause 4.4 of 3GPP TS 29.501.

security:

- {}

- oAuth2ClientCredentials:

- nchf-convergedcharging

paths:

/chargingdata:

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

responses:

'201':

description: Created

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataResponse'

'400':

description: Bad request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

chargingNotification:

'{$request.body#/notifyUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingNotifyRequest'

responses:

'204':

description: 'No Content, Notification was succesfull'

'400':

description: Bad request

content:

application/problem+json:

schema:

$ref: >-

TS29571_CommonData.yaml#/components/schemas/ProblemDetails

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/chargingdata/{ChargingDataRef}/update':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

parameters:

- name: ChargingDataRef

in: path

description: a unique identifier for a charging data resource in a PLMN

required: true

schema:

type: string

responses:

'200':

description: OK. Updated Charging Data resource is returned

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataResponse'

'400':

description: Bad request

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'403':

description: Forbidden

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/chargingdata/{ChargingDataRef}/release':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ChargingDataRequest'

parameters:

- name: ChargingDataRef

in: path

description: a unique identifier for a charging data resource in a PLMN

required: true

schema:

type: string

responses:

'204':

description: No Content.

'404':

description: Not Found

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'410':

$ref: 'TS29571_CommonData.yaml#/components/responses/410'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nchf-convergedcharging: Access to the Nchf_ConvergedCharging API

schemas:

ChargingDataRequest:

type: object

properties:

subscriberIdentifier:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

tenantIdentifier:

type: string

chargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ChargingId'

mnSConsumerIdentifier:

type: string

nfConsumerIdentification:

$ref: '#/components/schemas/NFIdentification'

invocationTimeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

invocationSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

retransmissionIndicator:

type: boolean

oneTimeEvent:

type: boolean

oneTimeEventType:

$ref: '#/components/schemas/oneTimeEventType'

notifyUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

serviceSpecificationInfo:

type: string

multipleUnitUsage:

type: array

items:

$ref: '#/components/schemas/MultipleUnitUsage'

minItems: 0

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

pDUSessionChargingInformation:

$ref: '#/components/schemas/PDUSessionChargingInformation'

roamingQBCInformation:

$ref: '#/components/schemas/RoamingQBCInformation'

sMSChargingInformation:

$ref: '#/components/schemas/SMSChargingInformation'

nEFChargingInformation:

$ref: '#/components/schemas/NEFChargingInformation'

registrationChargingInformation:

$ref: '#/components/schemas/RegistrationChargingInformation'

n2ConnectionChargingInformation:

$ref: '#/components/schemas/N2ConnectionChargingInformation'

locationReportingChargingInformation:

$ref: '#/components/schemas/LocationReportingChargingInformation'

nSPAChargingInformation:

$ref: '#/components/schemas/NSPAChargingInformation'

nSMChargingInformation:

$ref: '#/components/schemas/NSMChargingInformation'

required:

- nfConsumerIdentification

- invocationTimeStamp

- invocationSequenceNumber

ChargingDataResponse:

type: object

properties:

invocationTimeStamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

invocationSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

invocationResult:

$ref: '#/components/schemas/InvocationResult'

sessionFailover:

$ref: '#/components/schemas/SessionFailover'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

multipleUnitInformation:

type: array

items:

$ref: '#/components/schemas/MultipleUnitInformation'

minItems: 0

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

pDUSessionChargingInformation:

$ref: '#/components/schemas/PDUSessionChargingInformation'

roamingQBCInformation:

$ref: '#/components/schemas/RoamingQBCInformation'

locationReportingChargingInformation:

$ref: '#/components/schemas/LocationReportingChargingInformation'

required:

- invocationTimeStamp

- invocationSequenceNumber

ChargingNotifyRequest:

type: object

properties:

notificationType:

$ref: '#/components/schemas/NotificationType'

reauthorizationDetails:

type: array

items:

$ref: '#/components/schemas/ReauthorizationDetails'

minItems: 0

required:

- notificationType

ChargingNotifyResponse:

type: object

properties:

invocationResult:

$ref: '#/components/schemas/InvocationResult'

NFIdentification:

type: object

properties:

nFName:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nFIPv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

nFIPv6Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

nFPLMNID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

nodeFunctionality:

$ref: '#/components/schemas/NodeFunctionality'

nFFqdn:

type: string

required:

- nodeFunctionality

MultipleUnitUsage:

type: object

properties:

ratingGroup:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatingGroup'

requestedUnit:

$ref: '#/components/schemas/RequestedUnit'

usedUnitContainer:

type: array

items:

$ref: '#/components/schemas/UsedUnitContainer'

minItems: 0

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

multihomedPDUAddress:

$ref: '#/components/schemas/PDUAddress'

required:

- ratingGroup

InvocationResult:

type: object

properties:

error:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

failureHandling:

$ref: '#/components/schemas/FailureHandling'

Trigger:

type: object

properties:

triggerType:

$ref: '#/components/schemas/TriggerType'

triggerCategory:

$ref: '#/components/schemas/TriggerCategory'

timeLimit:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

volumeLimit:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

volumeLimit64:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

eventLimit:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

maxNumberOfccc:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

tariffTimeChange:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

required:

- triggerType

- triggerCategory

MultipleUnitInformation:

type: object

properties:

resultCode:

$ref: '#/components/schemas/ResultCode'

ratingGroup:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatingGroup'

grantedUnit:

$ref: '#/components/schemas/GrantedUnit'

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

validityTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

quotaHoldingTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'

finalUnitIndication:

$ref: '#/components/schemas/FinalUnitIndication'

timeQuotaThreshold:

type: integer

volumeQuotaThreshold:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

unitQuotaThreshold:

type: integer

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

required:

- ratingGroup

RequestedUnit:

type: object

properties:

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

UsedUnitContainer:

type: object

properties:

serviceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceId'

quotaManagementIndicator:

$ref: '#/components/schemas/QuotaManagementIndicator'

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

triggerTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

eventTimeStamps:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

minItems: 0

localSequenceNumber:

type: integer

pDUContainerInformation:

$ref: '#/components/schemas/PDUContainerInformation'

nSPAContainerInformation:

$ref: '#/components/schemas/NSPAContainerInformation'

required:

- localSequenceNumber

GrantedUnit:

type: object

properties:

tariffTimeChange:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

serviceSpecificUnits:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

FinalUnitIndication:

type: object

properties:

finalUnitAction:

$ref: '#/components/schemas/FinalUnitAction'

restrictionFilterRule:

$ref: '#/components/schemas/IPFilterRule'

restrictionFilterRuleList:

type: array

items:

$ref: '#/components/schemas/IPFilterRule'

minItems: 1

filterId:

type: string

filterIdList:

type: array

items:

type: string

minItems: 1

redirectServer:

$ref: '#/components/schemas/RedirectServer'

required:

- finalUnitAction

RedirectServer:

type: object

properties:

redirectAddressType:

$ref: '#/components/schemas/RedirectAddressType'

redirectServerAddress:

type: string

required:

- redirectAddressType

- redirectServerAddress

ReauthorizationDetails:

type: object

properties:

serviceId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceId'

ratingGroup:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatingGroup'

quotaManagementIndicator:

$ref: '#/components/schemas/QuotaManagementIndicator'

PDUSessionChargingInformation:

type: object

properties:

chargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ChargingId'

homeProvidedChargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ChargingId'

userInformation:

$ref: '#/components/schemas/UserInformation'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

mAPDUNon3GPPUserLocationInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

pduSessionInformation:

$ref: '#/components/schemas/PDUSessionInformation'

unitCountInactivityTimer:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DurationSec'
        rANSecondaryRATUsageReport:

$ref: '#/components/schemas/RANSecondaryRATUsageReport'

UserInformation:

type: object

properties:

servedGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

servedPEI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

unauthenticatedFlag:

type: boolean

roamerInOut:

$ref: '#/components/schemas/RoamerInOut'

PDUSessionInformation:

type: object

properties:

networkSlicingInfo:

$ref: '#/components/schemas/NetworkSlicingInfo'

pduSessionID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionId'

pduType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PduSessionType'

sscMode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SscMode'

hPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

servingNetworkFunctionID:

$ref: '#/components/schemas/ServingNetworkFunctionID'

ratType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

mAPDUNon3GPPRATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

dnnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Dnn'

dnnSelectionMode:

$ref: '#/components/schemas/dnnSelectionMode'

chargingCharacteristics:

type: string

pattern: '^[0-9a-fA-F]{1,4}$'

chargingCharacteristicsSelectionMode:

$ref: '#/components/schemas/ChargingCharacteristicsSelectionMode'

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

stopTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

sessionStopIndicator:

type: boolean

pduAddress:

$ref: '#/components/schemas/PDUAddress'

diagnostics:

$ref: '#/components/schemas/Diagnostics'

authorizedQoSInformation:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/AuthorizedDefaultQos'

subscribedQoSInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SubscribedDefaultQos'

authorizedSessionAMBR:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

subscribedSessionAMBR:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ambr'

servingCNPlmnId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

mAPDUSessionInformation:

$ref: '#/components/schemas/MAPDUSessionInformation'

enhancedDiagnostics:

$ref: '#/components/schemas/EnhancedDiagnostics5G'

redundantTransmissionType:

$ref: '#/components/schemas/RedundantTransmissionType'

pDUSessionPairID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

qosMonReports:

type: array

items:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosMonitoringReport'

minItems: 1

required:

- pduSessionID

- dnnId

PDUContainerInformation:

type: object

properties:

timeofFirstUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofLastUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qoSInformation:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosData'

qoSCharacteristics:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosCharacteristics'

afChargingIdentifier:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ChargingId'

afChargingIdString:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ApplicationChargingId'

userLocationInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

servingNodeID:

type: array

items:

$ref: '#/components/schemas/ServingNetworkFunctionID'

minItems: 0

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

sponsorIdentity:

type: string

applicationserviceProviderIdentity:

type: string

chargingRuleBaseName:

type: string

mAPDUSteeringFunctionality:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/SteeringFunctionality'

mAPDUSteeringMode:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/SteeringMode'

NSPAContainerInformation:

type: object

properties:

latency:

type: integer

throughput:

$ref: '#/components/schemas/Throughput'

maximumPacketLossRate:

type: string

serviceExperienceStatisticsData:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/ServiceExperienceInfo'

theNumberOfPDUSessions:

type: integer

theNumberOfRegisteredSubscribers:

type: integer

loadLevel:

$ref: 'TS29520_Nnwdaf_EventsSubscription.yaml#/components/schemas/NsiLoadLevelInfo'

NSPAChargingInformation:

type: object

properties:

singleNSSAI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- singleNSSAI

NetworkSlicingInfo:

type: object

properties:

sNSSAI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- sNSSAI

PDUAddress:

type: object

properties:

pduIPv4Address:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv4Addr'

pduIPv6AddresswithPrefix:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Addr'

pduAddressprefixlength:

type: integer

iPv4dynamicAddressFlag:

type: boolean

iPv6dynamicPrefixFlag:

type: boolean

addIpv6AddrPrefixes:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ipv6Prefix'

ServingNetworkFunctionID:

type: object

properties:

servingNetworkFunctionInformation:

$ref: '#/components/schemas/NFIdentification'

aMFId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmfId'

required:

- servingNetworkFunctionInformation

RoamingQBCInformation:

type: object

properties:

multipleQFIcontainer:

type: array

items:

$ref: '#/components/schemas/MultipleQFIcontainer'

minItems: 0

uPFID:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

roamingChargingProfile:

$ref: '#/components/schemas/RoamingChargingProfile'

MultipleQFIcontainer:

type: object

properties:

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

triggerTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

time:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

totalVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

localSequenceNumber:

type: integer

qFIContainerInformation:

$ref: '#/components/schemas/QFIContainerInformation'

required:

- localSequenceNumber

QFIContainerInformation:

type: object

properties:

qFI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

reportTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofFirstUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

timeofLastUsage:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

qoSInformation:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosData'

qoSCharacteristics:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/QosCharacteristics'

userLocationInformation:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

servingNetworkFunctionID:

type: array

items:

$ref: '#/components/schemas/ServingNetworkFunctionID'

minItems: 0

3gppPSDataOffStatus:

$ref: '#/components/schemas/3GPPPSDataOffStatus'

3gppChargingId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ChargingId'

diagnostics:

$ref: '#/components/schemas/Diagnostics'

enhancedDiagnostics:

type: array

items:

type: string

required:

- reportTime

RoamingChargingProfile:

type: object

properties:

triggers:

type: array

items:

$ref: '#/components/schemas/Trigger'

minItems: 0

partialRecordMethod:

$ref: '#/components/schemas/PartialRecordMethod'

SMSChargingInformation:

type: object

properties:

originatorInfo:

$ref: '#/components/schemas/OriginatorInfo'

recipientInfo:

type: array

items:

$ref: '#/components/schemas/RecipientInfo'

minItems: 0

userEquipmentInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

roamerInOut:

$ref: '#/components/schemas/RoamerInOut'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

sMSCAddress:

type: string

sMDataCodingScheme:

type: integer

sMMessageType:

$ref: '#/components/schemas/SMMessageType'

sMReplyPathRequested:

$ref: '#/components/schemas/ReplyPathRequested'

sMUserDataHeader:

type: string

sMStatus:

type: string

pattern: '^[0-7]?[0-9a-fA-F]$'

sMDischargeTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

numberofMessagesSent:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

sMServiceType:

$ref: '#/components/schemas/SMServiceType'

sMSequenceNumber:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

sMSresult:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

submissionTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

sMPriority:

$ref: '#/components/schemas/SMPriority'

messageReference:

type: string

messageSize:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

messageClass:

$ref: '#/components/schemas/MessageClass'

deliveryReportRequested:

$ref: '#/components/schemas/DeliveryReportRequested'

OriginatorInfo:

type: object

properties:

originatorSUPI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

originatorGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

originatorOtherAddress:

$ref: '#/components/schemas/SMAddressInfo'

originatorReceivedAddress:

$ref: '#/components/schemas/SMAddressInfo'

originatorSCCPAddress:

type: string

sMOriginatorInterface:

$ref: '#/components/schemas/SMInterface'

sMOriginatorProtocolId:

type: string

RecipientInfo:

type: object

properties:

recipientSUPI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

recipientGPSI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

recipientOtherAddress:

$ref: '#/components/schemas/SMAddressInfo'

recipientReceivedAddress:

$ref: '#/components/schemas/SMAddressInfo'

recipientSCCPAddress:

type: string

sMDestinationInterface:

$ref: '#/components/schemas/SMInterface'

sMrecipientProtocolId:

type: string

SMAddressInfo:

type: object

properties:

sMaddressType:

$ref: '#/components/schemas/SMAddressType'

sMaddressData:

type: string

sMaddressDomain:

$ref: '#/components/schemas/SMAddressDomain'

RecipientAddress:

type: object

properties:

recipientAddressInfo:

$ref: '#/components/schemas/SMAddressInfo'

sMaddresseeType:

$ref: '#/components/schemas/SMAddresseeType'

MessageClass:

type: object

properties:

classIdentifier:

$ref: '#/components/schemas/ClassIdentifier'

tokenText:

type: string

SMAddressDomain:

type: object

properties:

domainName:

type: string

3GPPIMSIMCCMNC:

type: string

SMInterface:

type: object

properties:

interfaceId:

type: string

interfaceText:

type: string

interfacePort:

type: string

interfaceType:

$ref: '#/components/schemas/InterfaceType'

RANSecondaryRATUsageReport:

type: object

properties:

rANSecondaryRATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

qosFlowsUsageReports:

type: array

items:

$ref: '#/components/schemas/QosFlowsUsageReport'

Diagnostics:

type: integer

IPFilterRule:

type: string

QosFlowsUsageReport:

type: object

properties:

qFI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Qfi'

startTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

uplinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

downlinkVolume:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint64'

NEFChargingInformation:

type: object

properties:

externalIndividualIdentifier:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

externalGroupIdentifier:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ExternalGroupId'

groupIdentifier:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GroupId'

aPIDirection:

$ref: '#/components/schemas/APIDirection'

aPITargetNetworkFunction:

$ref: '#/components/schemas/NFIdentification'

aPIResultCode:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uint32'

aPIName:

type: string

aPIReference:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

aPIContent:

type: string

required:

- aPIName

RegistrationChargingInformation:

type: object

properties:

registrationMessagetype:

$ref: '#/components/schemas/RegistrationMessageType'

userInformation:

$ref: '#/components/schemas/UserInformation'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

pSCellInformation:

$ref: '#/components/schemas/PSCellInformation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

5GMMCapability:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Bytes'

mICOModeIndication:

$ref: '#/components/schemas/MICOModeIndication'

smsIndication:

$ref: '#/components/schemas/SmsIndication'

taiList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

minItems: 0

serviceAreaRestriction:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceAreaRestriction'

minItems: 0

requestedNSSAI:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 0

allowedNSSAI:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 0

rejectedNSSAI:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 0

nSSAIMapList:

type: array

items:

$ref: '#/components/schemas/NSSAIMap'

minItems: 0

amfUeNgapId:

type: integer

ranUeNgapId:

type: integer

ranNodeId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GlobalRanNodeId'

required:

- registrationMessagetype

PSCellInformation:

type: object

properties:

nrcgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

NSSAIMap:

type: object

properties:

servingSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

homeSnssai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

required:

- servingSnssai

- homeSnssai

N2ConnectionChargingInformation:

type: object

properties:

n2ConnectionMessageType:

$ref: '#/components/schemas/N2ConnectionMessageType'

userInformation:

$ref: '#/components/schemas/UserInformation'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

pSCellInformation:

$ref: '#/components/schemas/PSCellInformation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

amfUeNgapId:

type: integer

ranUeNgapId:

type: integer

ranNodeId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/GlobalRanNodeId'

restrictedRatList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

minItems: 0

forbiddenAreaList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Area'

minItems: 0

serviceAreaRestriction:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ServiceAreaRestriction'

minItems: 0

restrictedCnList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/CoreNetworkType'

minItems: 0

allowedNSSAI:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 0

rrcEstCause:

type: string

pattern: '^[0-9a-fA-F]+$'

required:

- n2ConnectionMessageType

LocationReportingChargingInformation:

type: object

properties:

locationReportingMessageType:

$ref: '#/components/schemas/LocationReportingMessageType'

userInformation:

$ref: '#/components/schemas/UserInformation'

userLocationinfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/UserLocation'

pSCellInformation:

$ref: '#/components/schemas/PSCellInformation'

uetimeZone:

$ref: 'TS29571_CommonData.yaml#/components/schemas/TimeZone'

rATType:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RatType'

presenceReportingAreaInformation:

type: object

additionalProperties:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PresenceInfo'

minProperties: 0

required:

- locationReportingMessageType

N2ConnectionMessageType:

type: integer

LocationReportingMessageType:

type: integer

NSMChargingInformation:

type: object

properties:

managementOperation:

$ref: '#/components/schemas/ManagementOperation'

idNetworkSliceInstance:

type: string

listOfserviceProfileChargingInformation:

type: array

items:

$ref: '#/components/schemas/ServiceProfileChargingInformation'

minItems: 0

managementOperationStatus:

$ref: '#/components/schemas/ManagementOperationStatus'

# To be introduced once the reference to 'generic.yaml is resolved

#        managementOperationalState:

#          $ref: 'genericNrm.yaml#/components/schemas/OperationalState'

#        managementAdministrativeState:

#          $ref: 'genericNrm.yaml#/components/schemas/AdministrativeState'

required:

- managementOperation

ServiceProfileChargingInformation:

type: object

properties:

serviceProfileIdentifier:

type: string

sNSSAIList:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Snssai'

minItems: 0

# To be introduced once the reference to 'nrNrm.yaml is resolved

#         sST:

#           $ref: 'nrNrm.yaml#/components/schemas/Sst'

latency:

type: integer

availability:

type: number

# To be introduced once the reference to sliceNrm.yaml is resolved

#         resourceSharingLevel:

#           $ref: 'sliceNrm.yaml#/components/schemas/SharingLevel'

jitter:

type: integer

reliability:

type: string

maxNumberofUEs:

type: integer

coverageArea:

type: string

# To be introduced once the reference to sliceNrm.yaml is resolved

#        uEMobilityLevel:

#          $ref: 'sliceNrm.yaml#/components/schemas/MobilityLevel'

#        delayToleranceIndicator:

#          $ref: 'sliceNrm.yaml#/components/schemas/Support'

dLThptPerSlice:

$ref: '#/components/schemas/Throughput'

dLThptPerUE:

$ref: '#/components/schemas/Throughput'

uLThptPerSlice:

$ref: '#/components/schemas/Throughput'

uLThptPerUE:

$ref: '#/components/schemas/Throughput'

maxNumberofPDUsessions:

type: integer

kPIMonitoringList:

type: string

supportedAccessTechnology:

type: integer

# To be introduced once the reference to sliceNrm.yaml is resolved

#        v2XCommunicationModeIndicator:

#          $ref: 'sliceNrm.yaml#/components/schemas/Support'

addServiceProfileInfo:

type: string

Throughput:

type: object

properties:

guaranteedThpt:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

maximumThpt:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Float'

MAPDUSessionInformation:

type: object

properties:

mAPDUSessionIndicator:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/MaPduIndication'

aTSSSCapability:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AtsssCapability'

EnhancedDiagnostics5G:

$ref: '#/components/schemas/RanNasCauseList'

RanNasCauseList:

type: array

items:

$ref: 'TS29512_Npcf_SMPolicyControl.yaml#/components/schemas/RanNasRelCause'

NotificationType:

anyOf:

- type: string

enum:

- REAUTHORIZATION

- ABORT_CHARGING

- type: string

NodeFunctionality:

anyOf:

- type: string

enum:

- AMF

- SMF

- SMS

- PGW_C_SMF

- NEFF # Included for backwards compatibility, shall not be used

- SGW

- I_SMF

- ePDG

- CEF

- NEF

- MnS_Producer

- SGSN

- type: string

ChargingCharacteristicsSelectionMode:

anyOf:

- type: string

enum:

- HOME_DEFAULT

- ROAMING_DEFAULT

- VISITING_DEFAULT

- type: string

TriggerType:

anyOf:

- type: string

enum:

- QUOTA_THRESHOLD

- QHT

- FINAL

- QUOTA_EXHAUSTED

- VALIDITY_TIME

- OTHER_QUOTA_TYPE

- FORCED_REAUTHORISATION

- UNUSED_QUOTA_TIMER # Included for backwards compatibility, shall not be used

- UNIT_COUNT_INACTIVITY_TIMER

- ABNORMAL_RELEASE

- QOS_CHANGE

- VOLUME_LIMIT

- TIME_LIMIT

- EVENT_LIMIT

- PLMN_CHANGE

- USER_LOCATION_CHANGE

- RAT_CHANGE

- SESSION_AMBR_CHANGE

- UE_TIMEZONE_CHANGE

- TARIFF_TIME_CHANGE

- MAX_NUMBER_OF_CHANGES_IN_CHARGING_CONDITIONS

- MANAGEMENT_INTERVENTION

- CHANGE_OF_UE_PRESENCE_IN_PRESENCE_REPORTING_AREA

- CHANGE_OF_3GPP_PS_DATA_OFF_STATUS

- SERVING_NODE_CHANGE

- REMOVAL_OF_UPF

- ADDITION_OF_UPF

- INSERTION_OF_ISMF

- REMOVAL_OF_ISMF

- CHANGE_OF_ISMF

- START_OF_SERVICE_DATA_FLOW

- ECGI_CHANGE

- TAI_CHANGE

- HANDOVER_CANCEL

- HANDOVER_START

- HANDOVER_COMPLETE

- GFBR_GUARANTEED_STATUS_CHANGE

- ADDITION_OF_ACCESS

- REMOVAL_OF_ACCESS

- START_OF_SDF_ADDITIONAL_ACCESS

- REDUNDANT_TRANSMISSION_CHANGE

- CGI_SAI_CHANGE

- RAI_CHANGE

- type: string

FinalUnitAction:

anyOf:

- type: string

enum:

- TERMINATE

- REDIRECT

- RESTRICT_ACCESS

- type: string

RedirectAddressType:

anyOf:

- type: string

enum:

- IPV4

- IPV6

- URL

- type: string

TriggerCategory:

anyOf:

- type: string

enum:

- IMMEDIATE_REPORT

- DEFERRED_REPORT

- type: string

QuotaManagementIndicator:

anyOf:

- type: string

enum:

- ONLINE_CHARGING

- OFFLINE_CHARGING

- QUOTA_MANAGEMENT_SUSPENDED

- type: string

FailureHandling:

anyOf:

- type: string

enum:

- TERMINATE

- CONTINUE

- RETRY_AND_TERMINATE

- type: string

SessionFailover:

anyOf:

- type: string

enum:

- FAILOVER_NOT_SUPPORTED

- FAILOVER_SUPPORTED

- type: string

3GPPPSDataOffStatus:

anyOf:

- type: string

enum:

- ACTIVE

- INACTIVE

- type: string

ResultCode:

anyOf:

- type: string

enum:

- SUCCESS

- END_USER_SERVICE_DENIED

- QUOTA_MANAGEMENT_NOT_APPLICABLE

- QUOTA_LIMIT_REACHED

- END_USER_SERVICE_REJECTED

- USER_UNKNOWN

- RATING_FAILED

- QUOTA_MANAGEMENT

- type: string

PartialRecordMethod:

anyOf:

- type: string

enum:

- DEFAULT

- INDIVIDUAL

- type: string

RoamerInOut:

anyOf:

- type: string

enum:

- IN_BOUND

- OUT_BOUND

- type: string

SMMessageType:

anyOf:

- type: string

enum:

- SUBMISSION

- DELIVERY_REPORT

- SM_SERVICE_REQUEST

- DELIVERY

- type: string

SMPriority:

anyOf:

- type: string

enum:

- LOW

- NORMAL

- HIGH

- type: string

DeliveryReportRequested:

anyOf:

- type: string

enum:

- YES

- NO

- type: string

InterfaceType:

anyOf:

- type: string

enum:

- UNKNOWN

- MOBILE_ORIGINATING

- MOBILE_TERMINATING

- APPLICATION_ORIGINATING

- APPLICATION_TERMINATING

- type: string

ClassIdentifier:

anyOf:

- type: string

enum:

- PERSONAL

- ADVERTISEMENT

- INFORMATIONAL

- AUTO

- type: string

SMAddressType:

anyOf:

- type: string

enum:

- EMAIL_ADDRESS

- MSISDN

- IPV4_ADDRESS

- IPV6_ADDRESS

- NUMERIC_SHORTCODE

- ALPHANUMERIC_SHORTCODE

- OTHER

- IMSI

- type: string

SMAddresseeType:

anyOf:

- type: string

enum:

- TO

- CC

- BCC

- type: string

SMServiceType:

anyOf:

- type: string

enum:

- VAS4SMS_SHORT_MESSAGE_CONTENT_PROCESSING

- VAS4SMS_SHORT_MESSAGE_FORWARDING

- VAS4SMS_SHORT_MESSAGE_FORWARDING_MULTIPLE_SUBSCRIPTIONS

- VAS4SMS_SHORT_MESSAGE_FILTERING

- VAS4SMS_SHORT_MESSAGE_RECEIPT

- VAS4SMS_SHORT_MESSAGE_NETWORK_STORAGE

- VAS4SMS_SHORT_MESSAGE_TO_MULTIPLE_DESTINATIONS

- VAS4SMS_SHORT_MESSAGE_VIRTUAL_PRIVATE_NETWORK(VPN)

- VAS4SMS_SHORT_MESSAGE_AUTO_REPLY

- VAS4SMS_SHORT_MESSAGE_PERSONAL_SIGNATURE

- VAS4SMS_SHORT_MESSAGE_DEFERRED_DELIVERY

- type: string

ReplyPathRequested:

anyOf:

- type: string

enum:

- NO_REPLY_PATH_SET

- REPLY_PATH_SET

- type: string

oneTimeEventType:

anyOf:

- type: string

enum:

- IEC

- PEC

- type: string

dnnSelectionMode:

anyOf:

- type: string

enum:

- VERIFIED

- UE_DNN_NOT_VERIFIED

- NW_DNN_NOT_VERIFIED

- type: string

APIDirection:

anyOf:

- type: string

enum:

- INVOCATION

- NOTIFICATION

- type: string

RegistrationMessageType:

anyOf:

- type: string

enum:

- INITIAL

- MOBILITY

- PERIODIC

- EMERGENCY

- DEREGISTRATION

- type: string

MICOModeIndication:

anyOf:

- type: string

enum:

- MICO_MODE

- NO_MICO_MODE

- type: string

SmsIndication:

anyOf:

- type: string

enum:

- SMS_SUPPORTED

- SMS_NOT_SUPPORTED

- type: string

ManagementOperation:

anyOf:

- type: string

enum:

- CreateMOI

- ModifyMOIAttributes

- DeleteMOI

- type: string

ManagementOperationStatus:

anyOf:

- type: string

enum:

- OPERATION_SUCCEEDED

- OPERATION_FAILED

- type: string

RedundantTransmissionType:

anyOf:

- type: string

enum:

- NON_TRANSMISSION

- END_TO_END_USER_PLANE_PATHS

- N3/N9

- TRANSPORT_LAYER

- type: string

End of change