3GPP TSG-SA5 Meeting #140-e	S5-216398

# Online, , 15th Nov 2021 - 24th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.633 | CR | 0009 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | YANG Solution Set for Inventory Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSA_SBMA |  |  |  |  |  | Date: |  |  | 2021-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A YANG solution set is needed for inventory management. Currently only an XML solution is available. XML solution set was deprecated/removed from other parts of Rel-17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A YANG solution set is provided for inventory management. The YANG solution set is a mapping of the existing stage 2 in 28.632. As some users need inventory not just about HW, but also about HW, SW and Licenses alt.2 is also included. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No usable solution set for inventory management. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3, 4, X, X.1, X.2, X.2.1, X.2.2, X.2.3, X.2.4, X.2.5, X.2.6, X.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link : https://forge.3gpp.org/rep/sa5/MnS/tree/S5-215110_Rel-17_CR_28.663_YANG_Solution_Set_for_Inventory_Management Related to "S5-216092 Rel-17 CR 28.632 Update Inventory stage2 to support SBMA". Independent whether classes are IOCs or SupportIOCs for the XML and the YANG solution sets the mapping is the same. Resubmitted based on the October Rapporteurs call. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 28.632: " Telecommunication management; Inventory Management (IM) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[2]	3GPP TS 32.612: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Information Service (IS)".

[3]	3GPP TS 32.616: "Telecommunication management; Configuration Management (CM); Bulk CM Integration Reference Point (IRP): Solution Set (SS) definitions".

[4]	W3C REC-xml11-20060816: "Extensible Markup Language (XML) 1.1 (Second Edition)".

[5]	Void

[6]	W3C XML Schema Definition Language (XSD) 1.1 Part 1: Structures.

[7]	W3C XML Schema Definition Language (XSD) 1.1 Part 2: Datatypes.

[8]	W3C REC-xml-names-20060816: "Namespaces in XML 1.1 (Second Edition)".

[9]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[10]	3GPP TS 32.342: "Telecommunication management; File Transfer (FT) Integration Reference Point (IRP): Information Service (IS)".

[X]	3GPP TS 28.623: “Generic network resources Integration Reference Point (IRP); Solution Set (SS) definition”.

[X+1]	3GPP TS 32.160: "Management and orchestration; Management Service Template".

Next change

# 4	Solution Set definitions

This specification defines the following 3GPP Inventory Management (IM) Network Resource Model (NRM)  Solution Set (SS) definitions:

-	3GPP Inventory Management (IM) Network Resource Model (NRM) IRP XML definitions (Annex A)

-	3GPP Inventory Management (IM) Network Resource Model (NRM) YANG based definitions (Annex X) for SBMA based systems.

CORBA Solution Set is not present in the current version of this specification.

Next change

# Annex X (normative):
YANG definitions

## X.1	General

This annex contains the YANG definitions for Inventory Management NRM, in accordance with information model definitions specified	3GPP TS 28.632 [1]. The mapping follows 3GPP TS 32.160 [X+1] clause 6.2.

## X.2	Modules

### X.2.1 module _3gpp-inv-inventoryunit.yang

<CODE BEGINS>

module _3gpp-inv-inventoryunit {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-inv-inventoryunit";

prefix inv3gpp;

import ietf-yang-types { prefix yang; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This model defines a YANG mapping of the inventory data IOC for

Alternative 1 of the IM NRM, TS 28.632.";

reference "3GPP TS 28.633

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.632

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2021-10-01 { reference "CR-0009"; }

grouping InventoryUnitGrp {

description "Represents the InventoryUnit IOC.";

leaf inventoryUnitType {

description "Type of inventory unit.";

reference "3GPP TS 32.690";

type string;

config false;

}

leaf vendorUnitFamilyType {

description "Mnemonic of inventory unit family type (e.g. Fan, PSU)

assigned by vendor.";

type string;

config false;

}

leaf vendorUnitTypeNumber {

description "A vendor/manufacturer defined and assigned number which

uniquely identifies the unit type and optionally for backward

compatibility reasons only, also version (used for replacing hardware

units, spares).";

type string;

config false;

}

leaf versionNumber {

description "The version information related to vendorUnitTypeNumber.";

type string;

config false;

}

leaf vendorName {

description "Name of inventory unit vendor (or vendors may provide

manufacturer name)";

type string;

config false;

}

leaf serialNumber {

description "Serial number of inventory unit.";

type string;

config false;

}

leaf dateOfManufacture {

description "Date of manufacture of inventory unit";

type yang:date-and-time;

config false;

}

leaf dateOfLastService {

description "Date of last service or repair of inventory unit";

type yang:date-and-time;

config false;

}

leaf unitPosition {

description "Position of inventory unit (e.g. Rack, shelf, slot, etc.).

Depending on the implementation of the inventory unit in the managed

system, the value and meaning of this attribute may vary.

For example, if a system has three levels and types of inventory units

representing Rack, Shelf and Slot respectively (i.e. the Managed

Element contains multiple Rack inventory units, each Rack inventory

unit contains multiple Shelf inventory units and each Shelf inventory

unit contains multiple Slot inventory units), then for this example:

- for the Inventory Unit representing a Rack, the Frame

Identification code may be used as the value of this attribute;

- for the Inventory Unit representing a Shelf, the Rack Shelf code may

be used as the value of this attribute;

- for the Inventory Unit representing a Slot, the position code may be

used as the value of this attribute.";

type string;

config false;

}

leaf manufacturerData {

description "Manufacturer specific data of inventory unit.";

type string;

config false;

}

leaf-list relatedFunction {

description "DN of related ManagedFunction.";

type types3gpp:DistinguishedName;

config false;

}

}

augment "/me3gpp:ManagedElement" {

list InventoryUnit {

uses top3gpp:Top_Grp;

key id;

description "Represents inventory information for an Inventory Unit.";

container attributes {

uses InventoryUnitGrp;

// Data nodes that follow provide recursive containment

leaf-list parents {

type leafref {

path "../../../InventoryUnit/id";

}

description

"Reference to all containing InventoryUnit instances in

strict order from the root down to the immediate

parent (i.e. the full path from the top level InventoryUnit

and downward).

If InventoryUnit instances form a containment hierarchy this is

modeled using references between the child InventoryUnits

and the parent InventoryUnit.

This reference MUST NOT be present for the top level

InventoryUnit and MUST be present for all InventoryUnits below

it.";

}

leaf-list containedChildren {

type leafref {

path "../../../InventoryUnit/id";

}

description

"Reference to all directly contained InventoryUnit instances.

If InventoryUnit instances form a containment hierarchy

this is modeled using references between the child

InventoryUnits and the parent InventoryUnit.";

}

}

}

}

}

<CODE ENDS>

### X.2.2 module _3gpp-inv-inventoryunitne.yang

<CODE BEGINS>

module _3gpp-inv-inventoryunitne {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-inv-inventoryunitne";

prefix invne3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This model defines a YANG mapping of the NE inventory data

IOC for Alternative 2 of the IM NRM, TS 28.632.";

reference "3GPP TS 28.633

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.632

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2021-10-01 { reference CR-0009 ; }

grouping InventoryUnitNEGrp {

description "Represents the InventoryUnitNE IOC.";

leaf neId {

description "Vendor defined unique identifier of a logical or physical

network element unit.";

type string;

config false;

}

leaf customerIdentifier {

description "Unique identification of a vendor's customer.";

type string;

config false;

}

leaf productName {

description "NE name classifying a vendor's product family or function.";

type string;

config false;

}

leaf vendorName {

description "Name of inventory unit vendor (or vendors may provide

manufacturer name)";

type string;

config false;

}

leaf productType {

description "Identifier of the e.g. platform, in case the product can be

based on different HW/SW platforms (not used for logical NEs).";

type string;

config false;

}

leaf salesUniqueId {

description "Unique identifier used by vendor (used e.g. for ordering a

new unit).";

type string;

config false;

}

leaf operatorUniqueName {

description "Unique NE identifier used by operator.";

type string;

}

leaf siteId {

description "NE site in customer network.";

type string;

config false;

}

leaf additionalInformation {

description "Supplementary information about inventory data (if any).";

type string;

config false;

}

leaf mFunction {

description "DN of related ManagedFunction.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list lICList {

description "Set of DNs of related InventoryUnitLic IOCs.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list hWList {

description "Set of DNs of related InventoryUnitHw IOCs.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list sWList {

description "Set of DNs of related InventoryUnitSw IOCs.";

type types3gpp:DistinguishedName;

config false;

}

}

augment "/me3gpp:ManagedElement" {

list InventoryUnitNE {

uses top3gpp:Top_Grp;

key id;

description "Represents the logical and physical structure of the NE

(Network Element).";

container attributes {

uses InventoryUnitNEGrp;

// Data nodes that follow provide recursive containment

leaf-list parents {

type leafref {

path "../../../InventoryUnitNE/id";

}

description

"Reference to all containing InventoryUnitNE instances in

strict order from the root down to the immediate

parent (i.e. the full path from the top level InventoryUnitNE

and downward).

If InventoryUnitNE instances form a containment hierarchy this is

modeled using references between the child InventoryUnitNEs

and the parent InventoryUnitNE.

This reference MUST NOT be present for the top level

InventoryUnitNE and MUST be present for all InventoryUnitNEs below

it.";

}

leaf-list containedChildren {

type leafref {

path "../../../InventoryUnitNE/id";

}

description

"Reference to all directly contained InventoryUnitNE instances.

If InventoryUnitNE instances form a containment hierarchy

this is modeled using references between the child

InventoryUnitNEs and the parent InventoryUnitNE.";

}

}

}

}

}

<CODE ENDS>

### X.2.3 module _3gpp-inv-inventoryunithw.yang

<CODE BEGINS>

module _3gpp-inv-inventoryunithw {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-inv-inventoryunithw";

prefix invhw3gpp;

import ietf-yang-types { prefix yang; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This model defines a YANG mapping of the hardware inventory data

IOC for Alternative 2 of the IM NRM, TS 28.632.";

reference "3GPP TS 28.633

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.632

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2021-10-01 { reference "CR-0009"; }

typedef ManualDataEntry {

description "Non-extensible enumeration indicating whether a unit is

'passive', i.e. only supporting manual data entry, or 'active', i.e.

self-reporting";

type enumeration {

enum PASSIVE {

description "Manual insertion of inventory data is needed.";

}

enum ACTIVE {

description "Inventory data can be read from the unit.";

}

}

}

grouping InventoryUnitHwGrp {

description "Represents the InventoryUnitHw IOC.";

leaf hwId {

description "Hardware identifier allocated by the vendor, e.g. the serial

number.";

type string;

config false;

}

leaf hwType {

description "Type of the Hardware unit e.g. equipment holder, carriage.";

type string;

config false;

}

leaf hwName {

description "Mnemonic of Hardware inventory unit family type (e.g. Fan,

PSU) assigned by vendor.";

type string;

config false;

}

leaf hwVersion {

description "Version / revision number of current unit e.g. firmware

version.";

type string;

config false;

}

leaf vendorName {

description "Name of inventory unit vendor (or vendors may provide

manufacturer name)";

type string;

config false;

}

leaf salesUniqueId {

description "Unique identifier used by vendor (used e.g. for ordering a

new unit).";

type string;

config false;

}

leaf hwUnitLocation {

description "Unique physical / logical location identifier within NE.";

type string;

config false;

}

leaf model {

description "Equipment configuration, e.g. standard hardware unit or a

variant that may contain additional disk capacity.";

type string;

config false;

}

leaf hwCapability {

description "Hardware capability e.g. capacity, size.";

type string;

config false;

}

leaf modificationDate {

description "Date/time stamp of last change (e.g. repair action).";

type yang:date-and-time;

config false;

}

leaf manualDataEntry {

description "Indicates whether unit is passive (manual insertion of

inventory data is needed) or active (inventory data can be read from

the unit).";

type ManualDataEntry;

config false;

}

leaf additionalInformation {

description "Supplementary information about inventory data (if any).";

type string;

config false;

}

leaf mFunction {

description "DN of related ManagedFunction.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list lICList {

description "Set of DNs of related InventoryUnitLic IOCs.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list nEList {

description "Set of DNs of related InventoryUnitNE IOCs.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list sWList {

description "Set of DNs of related InventoryUnitSw IOCs.";

type types3gpp:DistinguishedName;

config false;

}

}

augment "/me3gpp:ManagedElement" {

list InventoryUnitHw {

uses top3gpp:Top_Grp;

key id;

description "Represents the hardware components.";

container attributes {

uses InventoryUnitHwGrp;

// Data nodes that follow provide recursive containment

leaf-list parents {

type leafref {

path "../../../InventoryUnitHw/id";

}

description

"Reference to all containing InventoryUnitHw instances in

strict order from the root down to the immediate

parent (i.e. the full path from the top level InventoryUnitHw

and downward).

If InventoryUnitHw instances form a containment hierarchy this is

modeled using references between the child InventoryUnitHws

and the parent InventoryUnitHw.

This reference MUST NOT be present for the top level

InventoryUnitHw and MUST be present for all InventoryUnitHws below

it.";

}

leaf-list containedChildren {

type leafref {

path "../../../InventoryUnitHw/id";

}

description

"Reference to all directly contained InventoryUnitHw instances.

If InventoryUnitHw instances form a containment hierarchy

this is modeled using references between the child

InventoryUnitHws and the parent InventoryUnitHw.";

}

}

}

}

}

<CODE ENDS>

### X.2.4 module _3gpp-inv-inventoryunitsw.yang

<CODE BEGINS>

module _3gpp-inv-inventoryunitsw {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-inv-inventoryunitsw";

prefix invsw3gpp;

import ietf-yang-types { prefix yang; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This model defines a YANG mapping of the software inventory data

IOC for Alternative 2 of the IM NRM, TS 28.632.";

reference "3GPP TS 28.633

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.632

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2021-10-01 { reference CR-0009 ; }

grouping InventoryUnitSwGrp {

description "Represents the InventoryUnitSw IOC.";

leaf swId {

description "Unique identifier of a SW unit.";

type string;

config false;

}

leaf swName {

description "Software release name used.";

type string;

config false;

}

leaf swVersion {

description "Version identifier of the software unit.";

type string;

config false;

}

leaf vendorName {

description "Name of inventory unit vendor (or vendors may provide

manufacturer name)";

type string;

config false;

}

leaf salesUniqueId {

description "Unique identifier used by vendor (used e.g. for ordering a

new unit).";

type string;

config false;

}

leaf classification {

description "Name of installed SW (e.g. SW release, SW build, SW

patches).";

type string;

config false;

}

leaf swStatus {

description "Status of the SW unit (e.g. installed, archived).";

type string;

config false;

}

leaf swInstallationTime {

description "Date and time when the software installation process ended

and the software was installed.";

type yang:date-and-time;

config false;

}

leaf swActivationTime {

description "Date and time when the software was activated.";

type yang:date-and-time;

config false;

}

leaf additionalInformation {

description "Supplementary information about inventory data (if any).";

type string;

config false;

}

leaf mFunction {

description "DN of related ManagedFunction.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list lICList {

description "Set of DNs of related InventoryUnitLic IOCs.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list nEList {

description "Set of DNs of related InventoryUnitNE IOCs.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list hWList {

description "Set of DNs of related InventoryUnitHw IOCs.";

type types3gpp:DistinguishedName;

config false;

}

}

augment "/me3gpp:ManagedElement" {

list InventoryUnitSw {

uses top3gpp:Top_Grp;

key id;

description "Represents the software components.";

container attributes {

uses InventoryUnitSwGrp;

// Data nodes that follow provide recursive containment

leaf-list parents {

type leafref {

path "../../../InventoryUnitSw/id";

}

description

"Reference to all containing InventoryUnitSw instances in

strict order from the root down to the immediate

parent (i.e. the full path from the top level InventoryUnitSw and

downward).

If InventoryUnitSw instances form a containment hierarchy this is

modeled using references between the child InventoryUnitSws

and the parent InventoryUnitSw.

This reference MUST NOT be present for the top level

InventoryUnitSw and MUST be present for all InventoryUnitSws below

it.";

}

leaf-list containedChildren {

type leafref {

path "../../../InventoryUnitSw/id";

}

description

"Reference to all directly contained InventoryUnitSw instances.

If InventoryUnitSw instances form a containment hierarchy

this is modeled using references between the child

InventoryUnitSws and the parent InventoryUnitSw.";

}

}

}

}

}

<CODE ENDS>

### X.2.5 module _3gpp-inv-inventoryunitlic.yang

<CODE BEGINS>

module _3gpp-inv-inventoryunitlic {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-inv-inventoryunitlic";

prefix invlic3gpp;

import ietf-yang-types { prefix yang; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This model defines a YANG mapping of the license inventory data

IOC for Alternative 2 of the IM NRM, TS 28.632.";

reference "3GPP TS 28.633

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.632

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2021-10-01 { reference CR-0009 ; }

grouping InventoryUnitLicGrp {

description "Represents the InventoryUnitLic IOC.";

leaf licId {

description "Unique identifier of a license (e.g. name, code).";

type string;

config false;

}

leaf licType {

description "Describing type of current license (e.g. capacity,

particular feature, number of subscribers).";

type string;

config false;

}

leaf vendorName {

description "Name of inventory unit vendor (or vendors may provide

manufacturer name).";

type string;

config false;

}

leaf validity {

description "License validity which may include one of the elements

duration, end (expiration date) or forever.";

type string;

config false;

}

leaf key {

description "License activation key according to the used licensing

system.";

type string;

config false;

}

leaf licStatus {

description "License status - applicable only for managed licenses (e.g.

scheduled, valid, expired, invalid, capacity violated).";

type string;

config false;

}

leaf licActivationTime {

description "Date and time when the license was activated.";

type yang:date-and-time;

config false;

}

leaf salesUniqueId {

description "Unique identifier used by vendor (used e.g. for ordering a

new unit).";

type string;

config false;

}

leaf additionalInformation {

description "Supplementary information about inventory data (if any).";

type string;

config false;

}

leaf mFunction {

description "DN of related ManagedFunction.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list sWList {

description "Set of DNs of related InventoryUnitSw IOCs.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list nEList {

description "Set of DNs of related InventoryUnitNE IOCs.";

type types3gpp:DistinguishedName;

config false;

}

leaf-list hWList {

description "Set of DNs of related InventoryUnitHw IOCs.";

type types3gpp:DistinguishedName;

config false;

}

}

augment "/me3gpp:ManagedElement" {

list InventoryUnitLic {

uses top3gpp:Top_Grp;

key id;

description "Represents the InventoryUnitLic SupportIOC.";

container attributes {

uses InventoryUnitLicGrp;

// Data nodes that follow provide recursive containment

leaf-list parents {

type leafref {

path "../../../InventoryUnitLic/id";

}

description

"Reference to all containing InventoryUnitLic instances in

strict order from the root down to the immediate

parent (i.e. the full path from the top level InventoryUnitLic and

downward).

If InventoryUnitLic instances form a containment hierarchy this is

modeled using references between the child InventoryUnitLics

and the parent InventoryUnitLic.

This reference MUST NOT be present for the top level

InventoryUnitLic and MUST be present for all InventoryUnitLics

below it.";

}

leaf-list containedChildren {

type leafref {

path "../../../InventoryUnitLic/id";

}

description

"Reference to all directly contained InventoryUnitLic instances.

If InventoryUnitLic instances form a containment hierarchy

this is modeled using references between the child

InventoryUnitLics and the parent InventoryUnitLic.";

}

}

}

}

}

<CODE ENDS>

### X.2.6 module _3gpp-inv-tmainventoryunit.yang

<CODE BEGINS>

module _3gpp-inv-tmainventoryunit {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-inv-tmainventoryunit";

prefix tmainv3gpp;

import ietf-yang-types { prefix yang; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-inv-inventoryunit { prefix inv3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This model defines a YANG mapping of the Tower Mounted

Amplifier (TMA) inventory data IOC for Alternative 1 of the IM NRM, TS

28.632.";

reference "3GPP TS 28.633

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.632

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2021-10-01 { reference CR-0009 ; }

grouping TmaInventoryUnitGrp {

description "Represents the TmaInventoryUnit IOC.";

uses inv3gpp:InventoryUnitGrp;

leaf tmaNumberOfNon-LinearGainValues {

description "Number of non linear gain values supported (N).";

reference "Defined in 3GPP TS 37.466";

type uint32;

config false;

}

leaf tmaNon-LinearGainValue {

description "Non linear gain supported value number (expressed in

dB/4).";

reference "Defined in 3GPP TS 37.466";

type uint32;

}

leaf tmaAdditionalDataFieldNumber {

description "Identifies a standard data field which has no operational

impact. Used by the procedures SetDeviceData and GetDevicedata.";

reference "Data field defined in Table B.3 of 3GPP TS 37.466";

type uint32;

}

leaf tmaAntennaModelNumber {

description "Antenna model number.";

reference "Data field defined in Table B.3 of 3GPP TS 37.466";

type string;

}

leaf tmaAntennaOperatingBands {

description "Antenna operating band(s). Not supported in this release.";

reference "Data field defined in Table B.3 of 3GPP TS 37.466";

type uint32;

}

leaf-list tmaBeamwidthForEachOpBandInBandOrder {

description "Beamwidth for each operating band in band order (deg),

beginning with lowest band. The lowest band is transmitted within the

first 16-bit value. Not supported in this release.";

// If the leaf-list is present it MUST have exactly four entries.

reference "Data field defined in Table B.3 of 3GPP TS 37.466";

type uint16;

max-elements 4;

}

leaf-list tmaGainForEachOpBandInBandOrder {

description "Gain [dBi] for each operating band in band order, expressed

in gain value times 10, beginning with the lowest band. The lowest band

is transmitted within the first 8-bit value. Not supported in this

release.";

// If the leaf-list is present it MUST have exactly four entries.

reference "Data field defined in Table B.3 of 3GPP TS 37.466";

type uint8;

max-elements 4;

}

leaf tmaInstallationDate {

description "Installation date.";

reference "Data field defined in Table B.3 of 3GPP TS 37.466";

type yang:date-and-time;

}

leaf tmaInstallersId {

description "Installer's ID.";

reference "Data field defined in Table B.3 of 3GPP TS 37.466";

type string;

}

leaf tmaMaxSupportedGain {

description "Maximum supported gain given as gain figure expressed in

dB/4.";

reference "Defined in 3GPP TS 37.466";

type uint32;

}

leaf tmaMinSupportedGain {

description "Minimum supported gain given as gain figure expressed in

dB/4.";

reference "Defined in 3GPP TS 37.466";

type uint32;

}

}

augment "/me3gpp:ManagedElement" {

list TmaInventoryUnit {

uses top3gpp:Top_Grp;

key id;

description "Represents inventory information for a Tower Mounted

Amplifier Unit.";

container attributes {

uses TmaInventoryUnitGrp;

// Data nodes that follow provide recursive containment

leaf-list parents {

type leafref {

path "../../../TmaInventoryUnit/id";

}

description

"Reference to all containing TmaInventoryUnit instances in

strict order from the root down to the immediate parent (i.e.

the full path from the top level TmaInventoryUnit and downward).

If TmaInventoryUnit instances form a containment hierarchy this is

modeled using references between the child TmaInventoryUnits

and the parent InventoryUnit.

This reference MUST NOT be present for the top level

TmaInventoryUnit and MUST be present for all TmaInventoryUnits

below it.";

}

leaf-list containedChildren {

type leafref {

path "../../../TmaInventoryUnit/id";

}

description

"Reference to all directly contained TmaInventoryUnit instances.

If TmaInventoryUnit instances form a containment hierarchy

this is modeled using references between the child

TmaInventoryUnits and the parent TmaInventoryUnit.";

}

}

}

}

}

<CODE ENDS>

### X.2.7 module _3gpp-inv-antennainventoryunit.yang

<CODE BEGINS>

module _3gpp-inv-antennainventoryunit {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-inv-antennainventoryunit";

prefix "antinv3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-inv-inventoryunit { prefix inv3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This model defines a YANG mapping of the antenna inventory data

IOC for Alternative 1 of the IM NRM, TS 28.632.";

reference "3GPP TS 28.633

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Solution Set (SS) definitions

3GPP TS 28.632

Inventory Management (IM)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)";

revision 2021-10-01 { reference CR-0009 ; }

grouping AntennaInventoryUnitGrp {

description "Represents the AntennaInventoryUnit IOC.";

reference "3GPP TS 28.633";

uses inv3gpp:InventoryUnitGrp;

leaf maxTiltValue {

description "The maximum amount of electrical tilt the RET system can

support, expressed in tilt value times 10.";

reference "See 'Maximum supported tilt' in Ref. 3GPP TS 37.466";

type types3gpp:TenthOfDegrees;

}

leaf minTiltValue {

description "The minimum amount of electrical tilt the RET system can

support, expressed in tilt value times 10.";

reference "See 'Minimum supported tilt' in Ref. 3GPP TS 37.466";

type types3gpp:TenthOfDegrees;

}

leaf mechanicalOffset {

description "Non-adjustable tilt value, which is imparted to the antenna

due to the physical installation. The actual tilt at any point in time

is the summation of mechanicalOffset and retTiltValue.

Note: The value of this attribute has no operational impact on the

network, e.g. the NE behavior is not affected by the value setting of

this attribute.  Note as well that this attribute is not supported

over the Iuant interface according to 3GPP TS 37.466.";

type types3gpp:TenthOfDegrees;

}

leaf baseElevation {

description "The elevation in meters above sea level at the base of the

antenna structure. This value, when subtracted from height (see TS

28.662), provides the height of the antenna above the ground.

An integral value representing a number of meters in 0.1 meter

increments.

Note: The value of this attribute has no operational impact on the

network, e.g. the NE behavior is not affected by the value setting of

this attribute.  Note as well that this attribute is not supported

over the Iuant interface according to 3GPP TS 37.466.";

type uint32;

units decimeter;

}

leaf latitude {

description "The latitude of the antenna location based on World

Geodetic System (1984 version) global reference frame (WGS 84).

Positive values correspond to the northern hemisphere.

Note: The value of this attribute has no operational impact on the

network, e.g. the NE behavior is not affected by the value setting of

this attribute.  Note as well that this attribute is not supported

over the Iuant interface according to 3GPP TS 37.466.";

reference "Valid values described in 3GPP TS 23.032";

type types3gpp:Latitude;

}

leaf longitude {

description "The longitude of the antenna location based on World

Geodetic System (1984 version) global reference frame (WGS 84).

Positive values correspond to degrees east of 0 degrees longitude.

Note: The value of this attribute has no operational impact on the

network, e.g. the NE behavior is not affected by the value setting of

this attribute.  Note as well that this attribute is not supported

over the Iuant interface according to 3GPP TS 37.466.";

reference "Valid values described in 3GPP TS 23.032";

type types3gpp:Longitude;

}

leaf patternLabel {

description "The pattern name is a textual, alpha-numeric string to

allow identification of the antenna pattern along with the antenna

vendor information such as model number, etc.

Note: The value of this attribute has no operational impact on the

network, e.g. the NE behavior is not affected by the value setting of

this attribute.  Note as well that this attribute is not supported

over the Iuant interface according to 3GPP TS 37.466.";

type string { length "0..80"; }

reference "3GPP TS 37.466";

}

}

augment "/me3gpp:ManagedElement" {

list AntennaInventoryUnit {

uses top3gpp:Top_Grp;

key id;

description "Represents inventory information for an Antenna Unit.";

container attributes {

uses AntennaInventoryUnitGrp;

// Data nodes that follow provide recursive containment

leaf-list parents {

type leafref {

path "../../../AntennaInventoryUnit/id";

}

description

"Reference to all containing AntennaInventoryUnit instances in

strict order from the root down to the immediate parent (i.e. the

full path from the top level AntennaInventoryUnit and downward).

If AntennaInventoryUnit instances form a containment hierarchy

this is modeled using references between the child

AntennaInventoryUnits and the parent AntennaInventoryUnit.

This reference MUST NOT be present for the top level

AntennaInventoryUnit and MUST be present for all

AntennaInventoryUnits below it.";

}

leaf-list containedChildren {

type leafref {

path "../../../AntennaInventoryUnit/id";

}

description

"Reference to all directly contained AntennaInventoryUnit

instances. If AntennaInventoryUnit instances form a containment

hierarchy this is modeled using references between the child

AntennaInventoryUnits and the parent AntennaInventoryUnit.";

}

}

}

}

}

<CODE ENDS>

End of  changes