| 3GPP TR 28.819 V0.4.0 (2021-11) |  |
| --- | --- |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and Orchestration; Study on continuous integration continuous delivery support for 3GPP NFs (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

This report studies multi-vendor CI-CD chains for 3GPP NFs	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Related Work in Other SDOs	8

4.1	ETSI-TST	8

4.2	NGMN	10

5	Concepts	10

5.1	Roles Relevant to this Study	10

5.2 	Single and Multiple NF Suppliers CI-CD	10

5.3	Classification of tests	11

6	Use cases and solutions for automated delivery chains	12

6.1	Scenario X: The operator is notified of the delivery of a new NF version from the NF supplier	12

6.1.1	Description	12

6.1.2	Potential Requirements	13

6.1.3 Possible solutions	13

6.1.3.1 Alternate 1	13

6.2	 Operational testing	13

6.2.1 Description	13

6.2.2 Requirements	14

6.2.3 	Possible Solutions	15

6.2.3.1 Alternate 1 – Test utilizing slicing	15

6.2.3.1.1 	Overview	15

6.2.3.1.2 	Details	15

6.2.4 Solution for automated roll-out	15

6.3 Feature selection	16

6.3.2 Requirements	16

6.3.3 Possible Solutions	16

6.3.3.1 Alternate 1	16

6.4	Test task and test case management	16

6.4.1	Description	16

6.4.2	Potential Requirements	16

6.4.3 Possible Solutions	17

6.4.3.1 Alternate 1	17

6.5	Feedback to vendor	17

6.5.1	Description	17

6.5.2	Potential Requirements	17

6.5.3	Solution Alternate 1	17

6.6      Environment Data Collection	18

6.6.1	Description	18

6.6.2	Potential Requirements	18

6.6.3 Possible Solutions	18

6.6.3.1 Alternate 1	18

6.7      Test orchestration	18

6.7.1	Description	18

6.7.2	Potential Requirements	18

6.7.3 Possible Solutions	19

6.7.3.1 Alternate 1	19

6.8	Scenario 8	19

6.8.1	Description	19

6.8.2	Potential Requirements	19

6.9      Test data analysis	19

6.9.1	Description	19

6.9.2	Potential Requirements	19

6.9.3 Possible Solutions	19

6.9.3.1 Alternate 1	19

7. Proposed Process for Multi-Vendor CI-CD	20

8	Conclusions and Recommendations	20

Annex <A> (normative): <Normative annex for a Technical Specification>	21

Annex <B> (informative): <Informative annex for a Technical Specification>	22

B.1	Heading levels in an annex	22

Annex <X> (informative): Change history	23

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# This report studies multi-vendor CI-CD chains for 3GPP NFs

# 1	Scope

The present document studies key issues in automating delivery of 3GPP NFs and associated software artefacts to the operator’s operation environment. The document describes the following details:

- The information delivered together with the NFs to the operator.

- The type of tests performed by the operator and the expectations towards the testing pipeline.

- Information relevant to the NF stored in operator’s software inventory.

- Procedure from software deployment to operations.

For each of the above-mentioned aspects key issues and corresponding solutions are defined.

Editor’s note: This is an initial version of the scope. After discussions, further modifications based on the issues discussed will be added.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	NGMN – “Continuous Delivery in Telecommunication Network Environments” Version 1, 2019

[3]	3GPP TS 28.530: 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Concepts, use cases and requirements

[4]		ETSI GR NFV-TST 006 v1.1.1 (2020-01): “Network Functions Virtualisation (NFV); Testing; Report on CICD and DevOps”.

[5]	ETSI GR NFV-TST 011 v1.1.1 (2019-03): “Network Functions Virtualisation (NFV); Testing; Test Domain and Description Language Recommendations”.

[6]		ETSI GR NFV-TST 002 v1.1.1 (2016-10): “Network Functions Virtualisation (NFV); Testing Methodology; Report on NFV Interoperability Testing Methodology”.

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

Test task:  A test task is an executable unit containing a set of test cases to be executed,  outputting the execution results of test cases within the test task.Test orchestration: Determine the execution order of test cases in one test task, allocate/deallocate and reseve/release resources to support testing.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Related Work in Other SDOs

## 4.1	ETSI-TST

The ETSI GR NFV TST 006[4] provides guidance and recommendations on how to leverage DevOps and CI/CD techniques across the boundary from VNF provider to service provider, or any combination of developer, installation and operational entities. The goal of the ETSI report is to establish a DevOps Joint Pipeline between VNF provider to service provider.

- Exploring use cases

  - Single vendor to single operator: This scenario can be understood as splitting the CICD process of a single product into different organizations. Development, building, and testing are in the vendor part, and deployment and operation are in the operator part. And analysed the delivery and feedback method between the two organizations.

  - Multiple vendors to single operator: This scenario analyses the interval of delivery by multiple vendors and the timing of integration of multiple vendor products in the Operator part, and points out that the integrated test is not on a component but on the combined integrated VNF or NS.

  - Based on the analysis of use cases, two components of the DevOps process are recommended:

DevOps server: Stage and operate for the operator part of DevOps process.

Data handling component: Used to process sensitive information in feedback data to the vendor.

- Defining the test steps in the DevOps process:

  - Step 1: Test Definition

  - Step 2: Code/VNF Package Shipment

  - Step 3: Automated Test Execution

  - Step 4: Moving to Production

  - Step 5: Collecting operational data

- Providing recommendations on implementations

  - Test code/test function/description included in VNF Package

VNF package is recommended to contain a testing section with various information concerning testing and DevOps. VNF Package is recommended to include a description of the acceptance test, the test code, and a framework or test VNF that automates the execution of the test code.

  - Implementation of automated test execution

Option 1: Package the test function that automates the test execution as part of the VNF Package: for example, as test VNFC.

Option 2: Implement the test function that automates the test execution as separate test VNF: for example, a test Network Service consists of test VNFs and the VNF under test.

  - Test feedback to VNF vendor

It is recommended that a requirement be specified for the VNF to be capable to provide the information as feedback data.

It is recommended that a requirement be specified for the OSS to be capable to receive the feedback from the VNF

Besides, there is a revision version of TST 006(TST006ed121) which is still working in progress now. This revision will extend the scope of the report to analyse and provide recommendations on how to enhance the support for joint delivery pipeline, including:

- Defining the key components in the DevOps process

  - DevOps server：Analyse which specific NFV components are involved in related operations of DevOps server, and discuss related requirements for NFV MANO APIs.

  - Test Framework: Analyse which specific NFV components are involved in related operations of Test Framework, and discuss related requirements for NFV MANO APIs.

- Analyze implementation of automated test execution

  - Leverage a standard test case description file which will be defined in NFV TST013 Spec, Test Frame work will parse this machine-readable file to obtain information for automated test execution.

- Defining the CI-CD process in more detail based on the above analysis.

ETSI NFV has described a general framework to be used in CICD, with the following major components:

- As described in ETSI GR NFV-TST 006 [4], DevOps server is responsible for pre-checks of the NFVI, triggering the different testing phases, evaluating the testing phases, post event health checks of the VNF(s), sending feedback to the VNF Provider.

- As described in ETSI GR NFV-TST 011 [x], a Test Execution Platform is responsible for managing the execution of test cases and managing all resources outside the System Under Test.

- As described in ETSI GR NFV-TST 002 [y], the System Under Test includes the Virtual Network Functions, the NFV Infrastructure, and the associated management/orchestration and descriptors.

As introduced above, there are already some related works in ETSI. So, it is helpful to use the results of the above ETSI GRs in this 3GPP study.

## 4.2	NGMN

The work done in NGMN on “Continuous Deliver in Telecommunication Network environments” [2] highlights five main aspects that are to be considered for standardization:

1.	The release model is the complete set of items required for instantiating a software-based function. In terms of 3GPP NF this would be the information and the supporting artefacts required instantiate the 3GPP NF

2.	The environment model describes the environment to which releases could be deployed. This refers to the available software and hardware artefacts in that environment. Release models may be specific to environment models

3.	Use of version control in the operator environment

4.	Automated deployment steps that include testing the software artefacts delivered from across vendors in a staging environment

5.	Clear separation of development, testing and operations environment for security reasons.

NGMN works acts as a guiding structure to the work in this study item. Some concepts from the NGMN work, particularly, release models are reused in this work.

Editor’s note: Concluding remarks to be updated before finalizing the TR

# 5	Concepts

## 5.1	Roles Relevant to this Study

This report uses the roles as specified in item 4.8 of TS28.530[3]. The roles that are particularly important for this document are:

- Network Operator (NOP)

- the NFVI Supplier

- The Network equipment supplier: For the purposes of this report the network equipment supplier is referred to as 3GPP NF supplier (or the VNF supplier if the NF is virtualized).

## 5.2 	Single and Multiple NF Suppliers CI-CD

Figure 1 Typical CI-CD Chain

A typical CI-CD chain consists of stages as in Figure 1: Development Phase, Staging Phase and Operational Phase. Starting from the Development Phase, the developers write the software, and the test code used to test the software. When the software is finished and committed, continuous integration environment will automatically trigger the building and testing.  At the step of testing, the integrity of software artifacts is  checked and the functional/non-functional acceptance testing are performed. At the Operational phase, the software is deployed in the production environment with the real traffic/user load loaded. The software delivered in such typical CI-CD chain is usually provided by a single supplier.

Figure 2 Multiple NF supplier to single operator

In telco environments (see Figure 2) the Dev and initial testing part of the CI-CD chain is typically at the vendor premises and the testing and deployment part is the operator part. The Operator part hosts the CI-CD tools which will be responsible for fetching delivered artifacts, triggering tests, collecting test results and data, deploying software to staging/production environment, collecting operational data and sending feedback to the vendor.

Figure 3: Vendor and Operator internal processes

In vendor part, the vendor has its internal CI-CD process including coding, testing, packaging as shown in Figure 3. Additionally, an operator receives software artefacts (3GPP NFs) from multiple vendors (shown in Figure 2) and may follow its own internal procedures (see Figure 3) as a part of the CI-CD chain before deploying the NF to an operational platform. The internal aspects of the NF delivery process are not a subject of standardization and are therefore not considered in this study.

## 5.3	Classification of tests

Figure X: Test classification

When a new NF is delivered to the operator, the operator expects to perform certain tests. Tests can normally be classified into three basic types (see Figure X)

- Unit and acceptance tests: These tests involve testing the independent functioning of the NF. The tests may be both functional and logistical in nature and include vertical testing. Vertical testing tests for aspects of the new NF such as claimed compatibility with the underlying hardware, software or virtualization platforms. These tests can be provided to the operator by the 3GPP NF supplier. In addition, the operators may specify their own set of unit and acceptance tests for each NF.

- Integration tests: Integration tests refer typically to limited functionality horizontal test with other NF. For example, when a new SMF is delivered, it may be tested for expected functionality against existing AMFs. This naturally requires the combination of AMF and SMF to be tested together vertically with the underlying virtualization system (e.g. the ETSI NFV network service (NS) supporting the NFs).

- System tests: In the context of 3GPP NFs a system test could refer to testing the NFs functionining as part of a complete NSSI, NSI or communication service instance.

- Operational tests: Once all the above test (1-3) are completed to the operator’s requirement the network operator may still roll out the new version of the NF into the operational environment slowly in a step wise process. At every step more responsibility (for example: more UEs, more geographical area) is assigned to the new version of the NF in comparison with the old one. Eventually, if the new NF version performs well and all functions of the old NF version may be assigned to the new NF, effectively replacing the old NF.

This classification of tests represents conceptual points on the continuum of tests’ complexity for the purposes of this document. A network operator may define his own tests anywhere along this continuum.

# 6	Use cases and solutions for automated delivery chains

## 6.1	Scenario X: The operator is notified of the delivery of a new NF version from the NF supplier

### 6.1.1	Description

Figure 1 A New NF is released

The NF supplier publishes a new release of an NF used by the operator in Figure 1. When a new release is published the operator’s 3GPP management system should become aware of its availability and fetch the newly released version of the NF. There are two possibilities for this use case

- The 3GPP management system fetches then new release of the NF from a location provided by the NF supplier

- The 3GPP management system receives the newly released NF from the NF supplier

Both cases should be supported.

### 6.1.2	Potential Requirements

REQ-New_NF_CICD-1: The 3GPP Management system should be aware of the locations where respective 3GPP NF suppliers provide new releases of the NFs used by the 3GPP management system.

REQ-New_NF_CICD-2: The 3GPP Management system should be able to receive the new NF from an authorized NF supplier.

REQ-New_NF_CICD-3: The 3GPP Management system should be able to fetch the new NF from the NF supplier

### 6.1.3 Possible solutions

#### 6.1.3.1 Alternate 1

As part of the business agreement to purchase the license to use the NF the NF supplier indicates to the operator where to subscribe to receive NF software updates. The operators 3GPP Management system subscribes to the said address to receive notifications of new NF versions. When the vendor internal processes are completed, and an update is available for the operator to test and deploy the vendor provides a notification at the said address. The notification may include additional information such as authentication, payment details and the requests of data for feedback in addition to the location on where to fetch the new NF version and the corresponding information about the NF. This information is the primary release model for the new NF and may include details such as:

- The NF type - example: AMF, SMF.

- An globally unique identifier for the NF and the corresponding release model information

- An identifier of the NF it replaces, if any.

- The new list of optional features and the corresponding variables to enable/disable those features

- The tests to be run on the NF as provided by the vendor – with an an address to fetch those tests

- Vendor information

- An address where to provide NF fault and performance feedback

.

Editor’s note: Which MnS is responsible for subscribing depends on the discussion paper outcome and is FFS.

## 6.2	 Operational testing

### 6.2.1 Description

The Network Slicing feature in 5G provides telecom operators the capability to support multiple logical networks on the common physical network infrastructure. Each of these Logical networks will cater to different services and might have different operational requirements spanning across domains. Therefore, in addition to the 5G feature, it is important for operators to have the flexibility to manage these logical networks independently. Cloud Native based platform and tools which are widely used in the IT world has inbuilt frameworks for agile and flexible deployment. These platforms are increasingly becoming the de-facto choice for 5G Telecom Products. This highlights the importance of bringing many Cloud Native benefits like CI-CD that are essential for automation and flexibility of 5G networks. CI-CD automates and integrates the development phase, testing phase and deployment phase to improve feature velocity. Enabling CI-CD in 5G networks which includes the most transaction-intensive and time-sensitive RAN network functions is still a challenge. Dynamic software upgrades and live testing of 5G software components in production environment without service interruptions remains the main challenge that need to be solved for enabling CI-CD in 5G networks.

The ability to test the upgraded network nodes in the operational network will increase the agility of 5G networks. Slicing and virtualization can be used to assist in live testing. A test network slice instance or a test NF can be created/deployed, using the upgraded network functions, for testing purpose. The basic principles of the operational testing are:

- Test network slice instance or test NF creation with appropriate information e.g., test cases, test duration, target network nodes etc.

- Automatic UE selection: The set of UEs to be assigned to the test network slice instance or the test NF instance can be selected and updated over time to minimize the adverse effects of testing. The UEs can be selected based on any network or UE characteristics, such as current load of the network, location of the network nodes, UE mobility or dual connectivity etc.

- Deployment location selection for the upgraded node: The location of the upgraded node can be selected to minimize the adverse impact of testing. The location of the Networks nodes that needs to be upgraded and tested can be selected based on load conditions of the Network nodes at a given location. The location of the node can also be based on the availability of UEs for testing in each location.

Furthermore, such operational testing could be further automated. When a new version of NF is delivered to the operator that has passed all tests and can now be rolled out into the operational network the roll out could be combined with operational testing. This means that the new NF could be installed in the operational network without being initially being used at all. If the installation proceeds correctly then a small portion of the cases (examples: UEs or events or requests) that were handled by the old NF version can now automatically be assigned to the new NF version. If the new NF performs well and no failures occur further entities could incrementally be assigned to the new NF till eventually the old NF has no cases to handle and is rendered redundant. At this point the old NF could be uninstalled and removed from the operational network. In case of error or insufficient performance the system rolls-back to the old NF.

### 6.2.2 Requirements

REQ-CICD-OP-FUN-1 The 3GPP Management system shall be able to support testing the upgraded software in the live network.

REQ-CICD-OP-FUN-2 The 3GPP Management system shall be able to support creating/deploying a slice for testing the upgraded network functions in the live network.

REQ-CICD-OP-FUN-3 The 3GPP Management system shall be able to support selecting appropriate UE(s) to be assigned to the test slice minimizing the service impacts, if any.

REQ-CICD-OP-FUN-4 The 3GPP Management system shall be able to support selecting appropriate location for the upgrade network functions deployment minimizing the service impacts, if any.

REQ-CICD-OP-FUN-5: The 3GPP Management system should support configurations that enable the automated roll-out of new NFs into the operational environment while performing operational tests.

REQ-CICD-OP-FUN-6: The 3GPP Management system should support the roll-back of new NFs to the previous version in an operation environment.

### 6.2.3 	Possible Solutions

#### 6.2.3.1 Alternate 1 – Test utilizing slicing

##### 6.2.3.1.1 	Overview

The solution involves creating/instantiating a test entity (NSI, NSSI or a NF) exclusively for testing purpose triggered by a software upgrade procedure. The test entity will include information related with the testing including Managed Function to be tested, test duration, test case etc.

The appropriate UE(s) are assigned to this test entity minimizing the service impacts. UEs can be selected based on the location of the network nodes where the upgraded software is to be deployed. UEs can also be selected based on the UE characteristics like UE mobility and UE capabilities like dual connectivity or can just be percentage of available UEs

The location of the networks nodes that needs to be upgraded and tested can be selected based on load conditions of the Network nodes at a given time. The location of the node can also be based on (not limited too) the availability of UEs for testing in each location.

##### 6.2.3.1.2 	Details

The solution requires the ability to indicate that a particular test entity instance is meant for testing purpose. This will include enabling information like list of nodes to be tested, test duration, procedures (e.g., rrc connection setup, handover etc) to be tested etc.

Once the test network entity is instantiated, the UE(s) can be assigned to this entity. The UEs selected for testing can be:

- Based on UE capability

- Carrier aggregation, Dual connectivity capability.

- Based on UE characteristics

- Based on User Mobility (which can be determined based on UEmobilityLevel).

- Based on UE RRC_State (which can be determined based on the UE history information stored with the operator)

- Any Percentage of available UE

The testing location of the new node can be critical in terms of minimizing the service experience and maintain better use experience. Selection of deployment location of the test software in the field can be based on

- the availability of radio and cloud resources;

- the availability of UE selected; and

- any other operator defined restrictions.

### 6.2.4 Solution for automated roll-out

The operator may configure a “roll-out plan” for any new version of a 3GPP NF. The roll-out plan may contain the following information

- The increments to assign cases (example: UEs, requests etc) to the new version of the NF. For example, in case a new version of NSSF is available: As a first increment 10% of the request for slice selection could go to the new version of the NSSF, in a second increment this could be increased to 20%.

- The conditions or thresholds to be met by the system to change from on increment to the next. For example: if there are no errors in the New version of the NSSF for the next 30 days move the increment 2.

- Other performance KPIs

- Conditions that signal a failure and may require a roll-back of the new NF. For example: Significantly higher drop rate for UEs assigned by the new version of the NSSF.

Based on such a roll-out plan and on being triggered by the conditions in the network the 3GPP management may configure the cases in the core network. For example: the selection of NSSF probability could be configured in the NRF.

## 6.3 Feature selection

An NF upgrade may provide the operator with several new features in the new version of the NF. In such a case it should be possible for the network operator to switch on or off the features that the operator doesn’t wish to use or features that cannot be supported in the operator network due to technical considerations. For example, if Network function gNB-CU is upgraded with features X, Y, Z but feature Y has dependency on the NG interface with 5GC. In this case, only feature X, Z should be tested and enabled in the operational network till the 5GC is also upgraded with feature Y.

### 6.3.2 Requirements

REQ-CICD-FS-FUN-1 The 3GPP Management system shall be able to support partial feature testing of the upgraded network functions.

REQ-CICD-FS-FUN-2 The 3GPP Management system shall be able to activate and deactivate certain features of an NF in an operational network.

### 6.3.3 Possible Solutions

#### 6.3.3.1 Alternate 1

Each NF has a version number already represented in its NRM. In addition, and associated to the version number, is list of optional features that the appropriate management service (example: provisioning service in an operational environment or the test management service in the testing environment is able to enable or disable). This association can be a part of the NF NRM with a binary state of enabled or disabled on each per feature in the feature list

.

## 6.4	Test task and test case management

### 6.4.1	Description

Service provider may want to test all new delivered NFs . The test could be any set of unit and acceptance tests, integration or sytems tests which are carried out automatically when a new version of NF is delivered to the service provider. Service provider may arrange relevant test cases in one test task, and should have the possibility of adding, reading, updating and deleting tests tasks, or adding, reading, updating and deleting test cases in one test task to its internal process. These tests are run automatically over every new version of the NF delivered to the service provider.

### 6.4.2	Potential Requirements

REQ-CICD_TT-FUN-1: The 3GPP Management system should have the ability to allow CRUD operations on test tasks and test cases on a per NF basis to the authorized consumer.

REQ-CICD_TT-FUN-2: The 3GPP Management system should have the ability to automatically run tests associated to an NF by an authorized consumer when a new version of the said NF is delivered to the service provider. Editor’s note: Need to check and replace “Operator" with the general term in the whole document.

### 6.4.3 Possible Solutions

#### 6.4.3.1 Alternate 1

To allow for CRUD operations of test. A solution is to add a test management service that can associate tests to a particular or yet to come NF version in a test inventory. When the new NF version is delivered, the tests that are assigned to the NF version are automatically executed in the testing environment. The order of test execution may be specified for enabling the correct orchestration of tests.

## 6.5	Feedback to vendor

### 6.5.1	Description

The network operator may conduct different procedures, like verification, tesing, deployment and so on after receiving the delivered NF from 3GPP NF supplier. During each procedure there may be problems (e.g. testing failure) or other information that need to feedback to the NF supplier. The network operator will make some analysis combining the reported errors and netwok operation data to diagnose problem and propose requirements for NF supplier to do improvements. The network operator can also feedback other related information, like problem description, logs and so on to assist the improvement.

### 6.5.2	Potential Requirements

REQ-CICD_VF-FUN-1: The 3GPP Management system should have the ability to  feedback the certain adjustment requirements, NF software problem description and related environment information to the 3GPP NF supplier.

REQ-CICD_VF-FUN-2: The 3GPP Management system should have the ability to feedback data according to NF supplier’s request.

### 6.5.3	Solution Alternate 1

To finish the feedback to vendor, 3GPP management system needs to do:

After analysing and diagnose problems, 3GPP management system sorts out the analysis results and reports, filters and extracts the NF related information, does user specific information removal and other needed data processing procedures, then deliver to NF suppliers through a new MnS.

NF supplier can subscribe the notification of the MnS, so that it can know when there has a feedback. The notification  is the feedback information, it can be the NF identifier (e.g. NF version id, NF instance id), description of problem, time stamp (e.g. the time when the problem occurred, the duration of the problem), analysis report, the address of logs, related network data and so on. If the NF supplier provides data requirements for feedback, then it should deliver information according to the requirements.In summary, it is recommended that a new MnS is introduced in 3GPP management system with the following capabilities:

- Obtain the analysis results and reports, filter and extract the NF related information

- Accomplish sensitive data removal, data deduplication and other necessary data processing for the NF related information

- Generate notification with feedback to vendors after data processing

## 6.6      Environment Data Collection

### 6.6.1	Description

When conducting testing or in NF operation and maintenance, there may happen test failure, fault or other problems, the network operator will sustain to monitor network running status, collect network data, and use collected environment data to analyze and diagnose problems. The environment data can be obtained from fault supervision, performance management, NWDA, MDA and etc. To fulfil the automatic CICD chains, the network operator should filter the related data and may feedback them to the NF supplier according to analysis results or NF supplier’s requests. This function can interact with 3GPP other MnFs to get data.

### 6.6.2	Potential Requirements

REQ-CICD_EDC-FUN-1: The 3GPP Management system should have the ability to collect test-related information.

REQ-CICD_EDC-FUN-2: The 3GPP Management system should have the ability to provide data to external CICD related systems..

### 6.6.3 Possible Solutions

#### 6.6.3.1 Alternate 1

The 3GPP management system needs to collect and manage test-related data for supporting CICD automation, such as test status, test results. When the test fails, 3GPP management system can use the data to position problem and analyse root cause. After tests passed, the 3GPP management system will involve the NF as part of operational network and keep continuous monitoring and collecting performance data and fault data from fault supervision, performance management service.

It is recommended that 3GPP management system has a new MnS for the following responsibilities:

- Collecting test related data

- Providing the collected test data to the analysis management service (i.e. MDAS) for positioning problem and analysing root cause if needed.

- Obtaining logs, configurations, fault information, performance information and othe information that are related to a specific NF from fault supervision, performance management service and etc.

- Providing data to external CICD related system when it requests the test data or other environment information.

## 6.7      Test orchestration

### 6.7.1	Description

After the NFs are delivered, there have mulitiple kinds of test cases (functional testing, performance testing and so on) to be conducted, the network operator will construct test tasks, determine the execution order of test cases, allocate and reseve resources to support testing based on test types and requirements.

### 6.7.2	Potential Requirements

REQ-CICD_TO-FUN-1: The 3GPP Management system should have the ability to provide network resource status and relevant information to external CICD related systems for assisting test orchestration.

### 6.7.3 Possible Solutions

#### 6.7.3.1 Alternate 1

## 6.8	Scenario 8

### 6.8.1	Description

While system tests are expensive to perform, they do not represent the real operational network. Therefore, the gap between a system test and an operational network, especially in telecommunication network, maybe be a rather large one.

To overcome this the operator may perform system testing in a (partially) simulated environment of the live operational network or a recording of the operational network based on specific events that the operator may want to test against. A new version of the NF may replace the old NF in the simulated environment and help the operator simulate how the new NF would perform in the operational network.

To be able to do so the 3GPP management system should be able to periodically provide configurable state and other relevant information to help maintain the simulated environment which is as close as possible to the operational network.

### 6.8.2	Potential Requirements

REQ-CICD-SE_FUN-1: The 3GPP management should provide to authorized consumer the capability to configure the level of details required to create a simulation of the operational network.

REQ-CICD-SE_FUN-2: The 3GPP management system should be able to periodically provide to authorized consumers the information required to simulate the   operational network.

REQ-CICD-SE_FUN-3: The 3GPP management system should be able to record the information required to create a simulation of the operational network.

## 6.9      Test data analysis

### 6.9.1	Description

During the conduct of testing in operational environment, there will have related network operations data and other environment data along with test results. When test fails (e.g. a performance testing under a test slice), 3GPP system can use these data to prelimary analyse where the problem is. If the analysis results show it is the problem of the NF, the network operator can send the improvement suggestions, problem description and relevant environment data to the NF supplier.

### 6.9.2	Potential Requirements

REQ-CICD_TDA-FUN-1: The 3GPP Management system should have the ability to analyse test related data and provide analysis results.

### 6.9.3 Possible Solutions

#### 6.9.3.1 Alternate 1

The 3GPP management system analyses test-related data, this solution suggets to define a new MDAS capability to do the test data analysis.

# 7. Proposed Process for Multi-Vendor CI-CD

# 8	Conclusions and Recommendations

The process described in Section 7 puts several requirements on the NF as well as the 3GPP management system which should be further standardized to enable cross-vendor automation of the CI-CD and testing pipelines.

Recommendation 1: A newly delivered NF should have significant standardized supporting information to help the 3GPP management system identify the prioritization of tests relevant  to the new NF  and how to include the tested NF in  operations. Examples of this information include:

- The NF types

- The previous NF it replaces

- The new list of optional features and the corresponding variables to enable/disable those features

- The tests to be run on the NF as proposed by the vendor

- Vendor information

- Feedback address

Further work is required to specify an exhaustive list of such information.

Recommendation 2: For R18 3GPP SA5 needs to study software management aspects. Examples include:

- What information is needed in a software inventory

- How and where are NF versions stored

- How are they maintained – for example: updated and replaced

- ?

Recommendation 3: 3GPP SA5 should work of specifying the following management functions/capabilities for management system:

- indicate a prioritization of tests for the CI-CD and operational testing and maintain information such as which tests are relevant for which NF

- Test task and test case management, test orchestration

- Uniformly collect and handle testing related data, be responsible for data fiter and processing to support the feedback to vendor

- Collec an maintain information that may need to be provided to the 3GPP NF supplier, examples include:

  - NF identifier

  - Test results

  - Description of problem

  - Time stamp (e.g. the time when the problem occurred, the duration of the problem)

  - Analysis report

  - Address of logs

Further work is required to specify an exhaustive list of such information.

Recommendation 4: 3GPP SA5 should work of specifying  enablers and solutions to perform and automate operational testing and enable automatic rollout of the new NF version into the operational environment.

Editor’s note: Further recommendations to be added prior to finalization based on further discussions.

## Annex <A> (normative):
<Normative annex for a Technical Specification>

## Annex <B> (informative):
<Informative annex for a Technical Specification>

# B.1	Heading levels in an annex

Heading levels within an annex are used as in the main document, but for Heading level selection, the "A.", "B.", etc. are ignored. e.g. B.1.2 is formatted using Heading 2 style.

## Annex <X> (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-05 | SA5-137e | S5-213059 |  |  |  | Initial Skeleton | 0.0.0 |
| 2021-05 | SA5-137e | S5-213568 |  |  |  | pCT 28.819 Add scope |  |
| 2021-05 | SA5-137e | S5-213144 |  |  |  | pCR 28.819 Add NGMN Background |  |
| 2021-05 | SA5-137e | S5-213569 |  |  |  | pCR 28.819 Add Explanation for related work in ETSI |  |
| 2021-05 | SA5-137e | S5-213570 |  |  |  | pCR 28.819 Add Roles relevant to the study |  |
| 2021-05 | SA5-137e | S5-213216 |  |  |  | Add Scenario on Supplier NF delivery |  |
| 2021-05 | SA5-137e | S5-213571 |  |  |  | pCR 28.819 Usecase and requirements for CI-CD |  |
| 2021-05 | SA5-137e | S5-213291 |  |  |  | pCR 28.819 Add Explanation for Single and Multiple NF Suppliers CI-CD |  |
| 2021-05 | SA5-137e | S5-213670 |  |  |  | Latest draft TR 28.819 | 0.1.0 |
| 2021-09 | SA5-138e | S5-214498 |  |  |  | pCR TR 28.819 Add test classifications |  |
| 2021-09 | SA5-138e | S5-214647 |  |  |  | pCR 28.819 add use case on tests CRUD |  |
| 2021-09 | SA5-138e | S5-214648 |  |  |  | pCR 28.819 Split operational testing scenario |  |
| 2021-09 | SA5-138e | S5-214500 |  |  |  | pCR 28.819 add use case to provide vendor feedback |  |
| 2021-09 | SA5-138e | S5-214502 |  |  |  | pcr 28.819 correct title |  |
| 2021-09 | SA5-138e | S5-214751 |  |  |  | Latest draft TR28.819 | 0.2.0 |
| 2021-10 | SA5-139e | S5-215527 |  |  |  | pCR 28.819 solution for Dynamic software update and live testing |  |
| 2021-10 | SA5-139e | S5-215528 |  |  |  | Add test task CRUD in 6.4 Tests’ management |  |
| 2021-10 | SA5-139e | S5-215529 |  |  |  | pCR 28.819 Definition of test orchestration and test task |  |
| 2021-10 | SA5-139e | S5-215615 |  |  |  | pCR 28.819 Use case of environment data collection |  |
| 2021-10 | SA5-139e | S5-215614 |  |  |  | pCR 28.819 Supplement to the use case of providing vendor feedback |  |
| 2021-10 | SA5-139e | S5-215616 |  |  |  | pCR 28.819 Use case of test orchestration |  |
| 2021-10 | SA5-139e | S5-215413 |  |  |  | pCR - 28.819 solution for new NF delivery |  |
| 2021-10 | SA5-139e | S5-215414 |  |  |  | pCR - 28.819 solution for feature selection |  |
| 2021-10 | SA5-139e | S5-215415 |  |  |  | pCR - 28.819 Add test management service |  |
| 2021-10 | SA5-139e | S5-215416 |  |  |  | pCR - 28.819 add operational testing in classification |  |
| 2021-10 | SA5-139e | S5-215634 |  |  |  | Latest Draft TR28.819 | 0.3.0 |
| 2021-11 | SA5-140e | S5-216587 |  |  |  | pCR 28.819 Providing detailed information for testing an NF |  |
| 2021-11 | SA5-140e | S5-216588 |  |  |  | pCR 28.819 Automated operational testing |  |
| 2021-11 | SA5-140e | S5-216249 |  |  |  | pCR 28.819 Supplement for notification data |  |
| 2021-11 | SA5-140e | S5-216379 |  |  |  | pCR 28.819 Add release model details |  |
| 2021-11 | SA5-140e | S5-216589 |  |  |  | pCR 28.819 Solution of providing vendor feedback |  |
| 2021-11 | SA5-140e | S5-216590 |  |  |  | pCR 28.819 Solution of environment data collection |  |
| 2021-11 | SA5-140e | S5-216592 |  |  |  | pCR 28.819 Use case of test data analysis |  |
| 2021-11 | SA5-140e | S5-216366 |  |  |  | pCR 28.819 Rapp Cleanup |  |
| 2021-11 | SA5-140e | S5-216593 |  |  |  | pCR - 28.819 Add intial recomendations |  |
| 2021-11 | SA5-140e | S5-216626 |  |  |  | pCR 28.819 Describe ETSI NFV testing framework |  |
| 2021-11 | SA5-140e | S5-216609 |  |  |  | Latest Draft 28.819 | 0.4.0 |