# 3GPP TSG SA WG5 Meeting #141e	S5-221384

# Online, , 15 Nov 2021- 24 Nov 2021

# Source:	Samsung

# Title:	pCR 28.312 ServiceDeploymentExpectation definition

# Document for:	Approval

Agenda Item:	6.4.10

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

None

# 3	Rationale

This contribution provides the concrete ServiceDeploymentExpectation definition.

# 4	Detailed proposal

1st modified section

#### 6.4.1 	Service Deployment Expectation

6.4.1.1 		Definition

Service Deployment Expectation is an IntentExpectation which can be used to represent MnS consumer's expectations for service deployment.

The Service Deployemnt Expectation is defined utilizing the constructs of the generic IntentExpectation <<dataType>>. It includes the following targets as attributes.

6.4.1.2		ExpectationTargets

Following provides the concrete ExpectationTargets for Service Deployment Expectation based on the common structure of ExpectationTarget

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| serviceTargets | M | T | T | F | F |
| serviceStartTimeTarget | O | T | T | F | F |
| serviceEndTimeTarget | O | T | T | F | F |
| edgeIdenfiticationIdTarget | CM | T | T | F | F |
| edgeIdenfiticationLocTarget | CM | T | T | F | F |

#### 6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| serviceTargets | This provides various service requirements in form of ServiceProfile[x]. Service requirements will be translate into resource requirements for underlying Network Slice Instance/Network Slice Subnet Instancesto satisfy the service requirements. Following are the allowed values: -targetName: "serviceTargets" -targetCondition: "is equal than" -targetValueRange: a list of ServiceProfile attribute as defined in TS 28.541. | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| serviceStartTimeTarget | This describes the start time at which the service shall be available. This contributes to the selection of the appropriate edge data network to be used for service deployment. Following are the allowed values: -targetName: "serviceStartTimeTarget " -targetCondition: "is equal than" -targetValueRange: start time stamp | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| serviceEndTimeTarget | This describes the end time after which the service shall not be available. This contributes to the selection of the appropriate edge data network to be used for service deployment. Following are the allowed values: -targetName: "serviceEndTimeTarget " -targetCondition: "is equal than" -targetValueRange: end time stamp | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| edgeIdenfiticationIdTarget | This identifies the edge network where the service need to be deployed. This should be used when the edge identification is known to the consumer Following are the allowed values: -targetName: "edgeIdentificationTarget" -targetCondition: "is equal than" -targetValueRange: EDNidentifier as defined in 28.538. | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |
| edgeIdenfiticationLocTarget | This identifies the location where the service need to be deployed. This should be used when the edge identification is not known to the consumer Following are the allowed values: -targetName: "edgeIdentificationTarget" -targetCondition: "is equal than" -targetValueRange: geographical target location. This will take a form of either single latitude & longitude or a TAI | type: ExpectationTarget multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: False isNullable: True |

End of 1st modified section