3GPP TSG-SA5 Meeting #141-e 	S5-221217

# e-meeting, 17 - 26 January 2022

# Source:	Huawei

# Title:	pCR 28.104 Add description of coordinated analysis

# Document for:	Approval

Agenda Item:	6.4.15

# 1	Decision/action requested

For approval

# 2	References

[1]	3GPP TR 28.104 V0.3.0 Management and orchestration; Management Data Analytics (MDA)

# 3	Rationale

Clause 7.2.2.2.2 implies that the MDAS producers are responsible for assuring the throughput performance. This is changed to make it clear that the MDAS producers are responsible for analysing the throughput performance.

Clause 7.2.2.2.2 describes that “The two levels of MDAS producers worked in a coordinated way to assure the throughput performance”, but there is no detail on the meaning of “in a coordinated way”. It is proposed to add text to clause 7.1 to explain the meaning of “in a coordinated way”. This is added to clause 7.1 because the concept is valid for multiple use cases.

# 4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st change

## 7.1	General

Management data analysis can be for a specific domain or for cross-domain. The two levels of MDAS producers work in a coordinated way to analyse the management data. Domain-level MDAS producers analyse the management data within a specific domain (CN or RAN). Cross-domain MDAS producer may use the domain-level analysis as input to end-to-end analysis.

2nd change

##### 7.2.2.2.2	Use case

Throughput is of great importance which represents the end users' experiences and also reflects the network problems, e.g., low UE throughput may be caused by the resource shortage. In order to satisfy the requirements of dL/ulThptPerSlice in the ServiceProfile, MDAS may be utilized for throughput related analysis/predictions for network slice instance.

MDAS producer should have the capability to receive the request from the consumer to analyse the network slice throughput related issues and identify the root cause to assist throughput assurance. Network slice throughput analysis can be for a specific domain or for cross-domain. The two levels of MDAS producers work in a coordinated way to analyse the throughput performance. The producer of MDAS is able to provide the MDA report including the network slice throughput analytics output.

End of changes