3GPP TSG-SA5 Meeting #142e	S5-222060rev2

# April 04 – 12, 2022, e-Meeting								       		 					      s5-2abcde

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.538 | CR | xxxx | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add the query procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AsiaInfo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2022-04-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the query procedures of EAS, ECS, and EES that is missing in clause 7.1 of TS 28.538. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add query procedures |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Clause 7.1 is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.Y (new) 7.1.3.Y (new) 7.1.4.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

1st Change

# 7	Procedural Flows

Editors Note: This section will contain the procedures for different edge computing management capabilities e.g provisioning/LCM, performance assurance etc.

## 7.1	Lifecycle management

### 7.1.1	Description

The clause contains procedures associated with lifecycle management.

### 7.1.2	EAS lifecycle management

#### 7.1.2. Y	EAS VNF query

Figure 7.1.2.Y-1 depicts a procedure that describes how an ASP can consume provisioning MnS query the EAS. It is assumed that both ASP and ECSP consumers have subscribed to the producer of provisioning MnS to receive notifications.

Figure 7.1.2.Y-1: EAS query procedure

1. ECSP provisioning MnS Producer receives a query request (this will use getMOIAttributes operation defined in 3GPP TS 28.532[5]) with objectInstance of the existing EAS MOI, scope, and list of attributes of EAS IOC. The list of attributes identifies the attributes to be returned by this operation.

2. Based on the request, ECSP provisioning MnS producer queries the concrete EAS MOI

3. MnS Producer sends a response to the MnS consumer with objectClass, objectInstance, status (e.g. fulfillStatus and other status), and list of [Attribute, Value] related to EAS instance as defined in clause 6.4 (e.g. eASAddress).

### 7.1.3	ECS lifecycle management

#### 7.1.3.Y 	ECS query

Figure 7.1.3.Y-1 shows that the PLMN operator or ECSP as the consumer requests the ECS query via the provisioning MnS.

Figure 7.1.3.Y-1: ECS query procedure

1. ECSP provisioning MnS Producer receives a query request (this will use getMOIAttributes operation defined in 3GPP TS 28.532[5]) with objectInstance of the existing ECS MOI, scope, and list of attributes of ECS IOC. The list of attributes identifies the attributes to be returned by this operation.

2. Based on the request, ECSP provisioning MnS producer queries the concrete ECS MOI

3. MnS Producer sends a response to the MnS consumer with objectClass, objectInstance, status (e.g. fulfillStatus and other status), and list of [Attribute, Value] related to ECS instance which is defined in clause 6.4(e.g. providerIdentifier).

7.1.4	ECS lifecycle management

#### 7.1.4.Y 	EES query

Figure 7.1.4.Y-1 shows that the PLMN operator or ECSP as the consumer requests the EES query via the provisioning MnS.

Figure 7.1.4.Y-1: EES query procedure

1. ECSP provisioning MnS Producer receives a query request (this will use getMOIAttributes operation defined in 3GPP TS 28.532[5]) with objectInstance of the existing EES MOI, scope, and list of attributes of EES IOC. The list of attributes identifies the attributes to be returned by this operation.

2. Based on the request, ECSP provisioning MnS producer queries the concrete EES MOI

3. MnS Producer sends a response to the MnS consumer with objectClass, objectInstance, status (e.g. fulfillStatus and other status), and list of [Attribute, Value] related to EES instance which is defined in clause 6.4(e.g. eESservingLocation).

End of changes