3GPP TSG-SA5 Meeting #142-e 	S5-222378rev2

# e-meeting, 4-12 April 2022

# Source:	China Mobile

# Title:	pCR 28.104 add inter-gNB beam selection optimization solution

# Document for:	Approval

Agenda Item:	6.6.5

1	Decision/action requested

The group is asked to discuss and approval.

2	References

[1]	3GPP TS 28.104 “Management and orchestration; Management Data Analytics (MDA)”.

[2]	   3GPP TR 28.809 “Management and orchestration; Study on enhancement of Management Data Analytics (MDA)”.

[3]    3GPP TS 28.552 “Management and orchestration; 5G performance measurements”

[4]    3GPP TS 32.422 “Telecommunication management; Subscriber and equipment trace; Trace control and configuration management”

[5]    3GPP TS 32.423 “Telecommunication management; Subscriber and equipment trace; Trace data definition and management”

3	Rationale

This pCR is to add the solution of inter-gNB beam selection optimization, which is based on the clause 6.5.2 in TR 28.809[2].

4	Detailed proposal

It proposes to make the following changes to TS 28.104[1].

1st Modified Section

8.4.X	MDA assisted mobility management

8.4.X.Y	Inter-gNB beam selection optimization

8.4.X.Y.1	MDA type

The MDA type for inter-gNB beam selection optimization is:  MDAMobilityManagement.BeamSelectionOptimization.

8.4.X.Y.2	Enabling data

The enabling data for inter-gNB beam selection optimization is provided in table 8.4.X.Y.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.X.Y.2-1: Enabling data for inter-gNB beam selection optimization

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | SS-RSRP distribution per SSB (beam) of serving NR cell | SS-RSRP distribution per SSB (clause 5.1.1.22.1 of TS 28.552 [3]). |
|  | SS-RSRQ distribution per SSB (beam) of serving NR cell | Clause 5.1.1.31, TS 28.552[3] |
|  | The transmitted PDCP uplink/downlink data volume. | Clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [3] |
|  | Beam level measurements: CSI-RS, SSB beam related measurements | Clause 5.1.1.28, TS 28.552 [3]; |
| MDT reports | The RSRPs of UE measurements. | RSRPs of M1 measurements in TS 32.422 [4] and TS 32.423 [5]. |
|  | The RSRQs of UE measuremnets. | RSRQs of M1 measurements in TS 32.422 [4] and TS 32.423 [5]. |
|  | The UE location information with latitude and longitude. | UE location of M1 measurements in TS 32.422 [4] and TS 32.423 [5]. |

8.4.X.Y.3	Analytics output

The specific information elements of the analytics output for inter-gNB beam selection optimization, in addition to the common information elements of the analytics output (see clause 8.3), are provided in table 8.4.X.Y.3-1.

Table 8.4.X.Y.3-1: Analytics output for inter-gNB beam selection optimization

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| PredictedHandoverSuccessRate | Predicted handover success rate per beam pair. |  | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| TimePeriod | Predicted time period used for the handover per beam pair. | M | type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| gNBsList | Objects involved: gNB(s) and cells of gNBs | M | type: DN multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

End of Modified Sections