3GPP TSG-SA5 Meeting #142-e 	S5-222426rev2

# e-meeting, 4th – 12th April 2022														Revision of S5-20xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0 | rev | 1 | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional charging message flow for LBO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | CHROAM |  |  |  |  |  | Date: |  |  | 2022-04-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the support of local breakout roaming scenario charging, the general description about local breakout roaming charging message flows should be introduced. There are two options for the interaction between V-SMF and CHF(V-CHF and H-CHF). - Option 1:V-SMF triggers and sends the charging data request to the V-CHF and H-CHF at the same time. - Option 2:V-SMF triggers and sends the charging data request to V-CHF firstly and then sends the charging data request to H-CHF. If the Roaming Charing Profile is only used for the QBC triggers setting, the option 1 and opion 2 are both applicable. If the Roaming Charging Profile is used for negotiation and QBC triggers setting, the option 2 is preferred. If the V-CHF may reject the charging data request, the option 2 is preferred, otherwise, the SMF should terminate the charging session again with the H-CHF. If the H-CHF may reject the charging data request, in the option 1 and option 2, the SMF should terminate the charging session with the V-CHF. The message flow describes the option 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the charging message flows for the support of local breakout roaming scenario charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The support of the local breakout roaming scenario charging is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.X (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.2.2.X	PDU session charging for roaming in Local breakout scenario

5.2.2.X.1	General

The clause below describes PDU session charging in roaming with local breakout scenarios.

5.2.2.X.2	PDU session establishment

The following figure 5.2.2.X.2-1 describes a PDU session establishment charging, based on figure 4.3.2.2.1-1 UE-requested PDU Session Establishment for non-roaming and roaming with local breakout TS 23.502 [202] description:

Figure 5.2.2.X.2-1: PDU session establishment

9ch-a. The UE is identified as a roamer (PLMN ID of the received SUPI is different from VPLMN PLMN ID), the V-CHF and H-CHF are selected accordingly.

9ch-b1. The Charging Data Request [Initial] is sent to V-CHF, indicating "in-bound roamer" for authorization for the subscriber to start the PDU session which is triggered by start of PDU session charging event.

9ch-c1. The V-CHF opens a CDR (indicating "in-bound roamer")

9ch-d1. The V-CHF acknowledges by sending Charging Data Response [Initial] to the V-SMF and optionally supplies a "Roaming Charging Profile" to the V-SMF which overrides the default one.

9ch-b2. A Charging Data Request [Initial] is sent to H-CHF, with charging id and the "Roaming Charging Profile" form V-CHF.

This step may request the quota from V-SMF, H-CHF can grant the quota in the step 9ch-h.

9ch-c2. The H-CHF opens a CDR .

9ch-d2. The H-CHF acknowledges by sending Charging Data Response [Initial] to the V-SMF and optionally supplies the HPLMN selected "Roaming Charging Profile" to the V-SMF.

10ch-a1. The Charging Data Request [Update] is sent to V-CHF, when triggers for QBC is armed with the HPLMN selected "Roaming Charging Profile"

10ch-b1. The V-CHF update the CDR.

10ch-c1. The V-CHF acknowledges by sending Charging Data Response [Update] to the V-SMF.

10ch-a2. A Charging Data Request [update] is sent to H-CHF, when the FBC or QBC triggers specified in the clause 5.2.1 is armed.

This step may occur in case "start of service data flow" needs quota from H-CHF, for the V-SMF to request quota.

10ch-b2. The H-CHF update a CDR.

10ch-c2. The H-CHF acknowledges by sending Charging Data Response [Initial] to the V-SMF.

5.2.2.X.3	PDU Session Modification

The following figure 5.2.2.X.3-1 describes the PDU session modification charging, based on figure 4.3.3.2-1 UE or network requested PDU Session Modification (for non-roaming and roaming with local breakout) TS 23.502 [202] description:

Figure 5.2.2.X.3-1: PDU Session Modification

2ch-a1. The Charging Data Request [Update] is sent to V-CHF for reporting the charging information when the corresponding trigger for QBC specified in the clause 5.2.1 is armed.

2ch-b1. The V-CHF update the CDR.

2ch-c1. The V-CHF acknowledges by sending Charging Data Response [Update] to the V-SMF.

2ch-a2. A Charging Data Request [update] is sent to H-CHF, when the FBC or QBC triggers specified in the clause 5.2.1 is armed.

This step may occur in case "start of service data flow" needs quota from H-CHF, for the V-SMF to request quota.

2ch-b2. The H-CHF update a CDR.

2ch-c2. The H-CHF acknowledges by sending Charging Data Response [Initial] to the V-SMF.

5.2.2.x.4	PDU Session Release

The following figure 5.2.2.x.4-1 describes the PDU session release charging, based on figure 4.3.4.2-1 UE or network requested PDU Session Release for non-roaming and roaming with local breakout TS 23.502 [202] description:

Figure 5.2.2.X.4-1: PDU Session Release

2ch-a1. The Charging Data Request [Termination] is sent to V-CHF.

2ch-b1. The V-CHF close the CDR.

2ch-c1. The V-CHF acknowledges by sending Charging Data Response [Termination] to the V-SMF.

2ch-a2. A Charging Data Request [Termination] is sent to H-CHF.

2ch-b2. The H-CHF close a CDR.

2ch-c2. The H-CHF acknowledges by sending Charging Data Response [Termination] to the V-SMF.

End of change