3GPP TSG-SA5 Meeting #144-e 	S5-224166

# e-meeting, 27 June – 1 July 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | draftCR | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 draftCR 28.552 Add beam specific inter-system handover counters related to MRO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2022-06-17 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Add new PMs to support analytics per beam-cell-pair related to inter-system mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce new PMs related to handover failure per beam-cell-pair related to MRO for inter-system mobility. - Clarification of Inter-system too early handover and Inter-system too late handover failure - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Use case for beam handover inter-system mobility cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.25.1, 5.1.1.25.2, 5.1.1.25.3, 5.1.1.25.4, 5.1.1.25.x, 5.1.1.25.y, 5.1.1.25.z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.313 draftCR S5-224168 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Next changes

#### 5.1.1.25	Measurements related to MRO

##### 5.1.1.25.1	Handover failures related to MRO for intra-system mobility

a)	This measurement provides the number of handover failure events related to MRO detected during the intra-system mobility within 5GS, see TS 38.300 [49] clause 15.5.2. The measurement includes separate counters for various handover failure types, classified as "Intra-system too early handover", "Intra-system too late handover" and "Intra-system handover to wrong cell".

b)	CC.

c)	The measurements of too early handovers, too late handovers and handover to wrong cell events are obtained respectively by accumulating the number of failure events detected by gNB during the intra-system mobility within 5GS.

d)	Each measurement is an integer value.

e)	HO.IntraSys.TooEarly
HO.IntraSys.TooLate

HO.IntraSys.ToWrongCell

f)	NRCellCU

NRCellRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.2	Handover failures related to MRO for inter-system mobility

a)	This measurement provides the number of handover failure events delated to MRO detected during the inter-system mobility from EPS to 5GS or from 5GS to EPS, see TS 38.300 [49] clause 15.5.2. The measurement includes separate counters for various handover failure types, classified as "Inter-system too early handover" (inter-system mobility from EPS to 5GS) and "Inter-system too late handover" (inter-system mobility from EPS to 5GS).

b)	CC.

c)	The measurements of too early handovers and too late handovers events are obtained respectively by accumulating the number of failure events detected by gNB during the inter-system mobility from EPS to 5GS or from 5GS to EPS.

d)	Each measurement is an integer value.

e)	HO.InterSys.TooEarly

HO.InterSys.TooLate

f)	NRCellCU

EutranRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.3	Unnecessary handovers for inter-system mobility

a)	This measurement provides the number of unnecessary handover events detected during the inter-system mobility from 5GS to EPS, see TS 38.300 [49] clause 15.5.2. An example of unnecessary handover occurred when a UE handed over from NG-RAN to other system (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient.

b)	CC.

c)	The measurement of unnecessary handovers is obtained by accumulating the number of failure events detected gNB during the inter-system mobility from 5GS to EPS.

d)	Each measurement is an integer value.

e)	HO.InterSys.Unnecessary

f)	NRCellCU

EutranRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.4	Handover ping-pong for inter-system mobility

a)	This measurement provides the number of handover ping-pong events detected during the inter-system mobility from 5GS to EPS, see TS 38.300 [49] clause 15.5.2. An example of handover ping-pong occurred when a UE is handed over from a cell in a source system (e.g. NG-RAN) to a cell in a target system different from the source system (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source system, while the coverage of the source system was sufficient for the service used by the UE.

b)	CC.

c)	The measurement of handover ping-pong events is obtained by accumulating the number of failure events detected by gNB during the inter-system mobility from 5GS to EPS.

d)	Each measurement is an integer value.

e)	HO.InterSys.PingPong

f)	NRCellCU

EutranRelation

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.5	Handover failures per beam-cell pair related to MRO for intra-system mobility

a)	This measurement provides the number of handover failure events per beam-cell pair (source beam, i.e., the last beam before failure, and target cell) related to MRO detected during the intra-system mobility within 5GS. The measurement includes separate counters for various handover failure types, classified as "Intra-system too early handover per beam”, "Intra-system too late handover per beam " and "Intra-system handover to wrong cell per beam ". The handovers considered are inter-cell handovers.

b)	CC.

c)	The measurements of too early handovers for the beam per adjacent cell, too late handovers for the beam per adjacent cell and handover to wrong cell for the beam per adjacent cell events are obtained respectively by accumulating the number of failure events detected by gNB during the intra-system mobility within 5GS, where adjacent cells are identified by their NR Cell Identity (NCI).

d)	Each measurement is an integer value.

e)	HO.IntraSys.bTooEarly.NCI
HO.IntraSys.bTooLate.NCI
HO.IntraSys.bToWrongCell.NCI

f)	Beam

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.x	Handover failures per beam-cell pair related to MRO for inter-system mobility

a)	This measurement provides the number of handover failure events per beam-cell pair (source beam, i.e., the last beam before failure, and target cell) related to MRO detected during the inter-system mobility from 5GS to EPS. The measurement counter is classified as handover failure type "Inter-system too late handover".

b)	CC.

c)	The measurements of too late handovers for the beam per adjacent cell events are obtained respectively by accumulating the number of failure events detected by gNB during the inter-system mobility from 5GS to EPS, where adjacent cells are identified by their NR Cell Identity (NCI).

d)	Each measurement is an integer value.

e)	HO.InterSys.bTooLate.NCI

f)	Beam

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.y	Unnecessary handovers per beam-cell pair for inter-system mobility

a)	This measurement provides the number of unnecessary handover events per beam-cell pair (source beam, i.e., the last beam before failure, and target cell) detected during the inter-system mobility from 5GS to EPS. An example of unnecessary handover occurred when a UE handed over from NG-RAN to other system (e.g. UTRAN) even though quality of the NG-RAN coverage was sufficient.

b)	CC.

c)	The measurement of unnecessary handovers for the beam per adjacent cell is obtained by accumulating the number of failure events detected gNB during the inter-system mobility from 5GS to EPS, where adjacent cells are identified by their NR Cell Identity (NCI).

d)	Each measurement is an integer value.

e)	HO.InterSys.bUnnecessary.NCI

f)	Beam

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

##### 5.1.1.25.z	Handover ping-pong per beam-cell pair for inter-system mobility

a)	This measurement provides the number of handover ping-pong events per beam-cell pair (source beam, i.e., the last beam before failure, and target cell) detected during the inter-system mobility from 5GS to EPS. An example of handover ping-pong occurred when a UE is handed over from a cell in a source system (e.g. NG-RAN) to a cell in a target system different from the source system (e.g. E-UTRAN), then within a predefined limited time the UE is handed over back to a cell in the source system, while the coverage of the source system was sufficient for the service used by the UE.

b)	CC.

c)	The measurement of handover ping-pong events for the beam per adjacent cell is obtained by accumulating the number of failure events detected by gNB during the inter-system mobility from 5GS to EPS, where adjacent cells are identified by their NR Cell Identity (NCI).

d)	Each measurement is an integer value.

e)	HO.InterSys.bPingPong.NCI

f)	Beam

g)	Valid for packet switched traffic.

h)	5GS.

i)	One usage of this measurement is to support MRO (see TS 28.313 [30]).

End of changes