3GPP TSG-SA5 Meeting #145-e 	S5-225566rev1

# e-meeting, 15 - 24 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | 0006 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR TS 28.105 Clarifications and corrections into the Class definitions and Attribute properties |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC, Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS |  |  |  |  |  | Date: |  |  | 2022-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | An editor’s note remains unsolved and other corrections are also needed to the class definition and attribute properties. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections to the attributes properties including a resolution to the Editor’s note. Some corrections and clarifications are aslo added to the class definition. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification remain incomplete leading ambiguties leading to misunderstanding and potentially wrong and non-interoperable implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2.1, 7.3.4.1, 7.4.3.2, 7.5.1, B.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link for the stage 3 update: https://forge.3gpp.org/rep/sa5/MnS/-/tree/TS28.105_CR%230006_Clarifications_and_corrections_into_the_Class_definitions_and_Attribute_properties |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 7.3	Class definitions

### 7.3.2	AIMLTrainingRequest

#### 7.3.2.1	Definition

The IOC AIMLTrainingRequest represents the AI/ML model training request that is created by the AI/ML training MnS consumer.

The AIMLTrainingRequest MOI is contained under one AIMLTrainingFunction MOI. Each AIMLTrainingRequest is associated to at least one AIMLEntity.

The AIMLTrainingRequest may have a source to identify where it is coming from, and which may be used to prioritize the training resources for different sources. The sources may be for example the network functions, operator roles, or other functional differentiations.

Each AIMLTrainingRequest may indicate the expectedRunTimeContext that describes the specific conditions for which the AIMLEntity (either AIML Model or AIML-enabled function) should be trained for.

In case the request is accepted, the AI/ML training MnS producer decides when to start the AI/ML training. Once the MnS producer decides to start the training based on the request, the AI/ML training MnS producer instantiates one or more AI/MLTrainingProcess MOI(s) that are responsible to perform the followings:

-	collects (more) data for training, if the training data are not available or the data are available but not sufficient for the training;

-	prepares and selects the required training data, with consideration of the consumer’s request provided candidate training data if any. The AI/ML training MnS producer may examine the consumer's provided candidate training data and select none, some or all of them for training. In addition, the AI/ML training MnS producer may select some other training data that are available in order to meet the consumer’s requirements for the MLentity training;

-	trains the AIMLEntity using the selected and prepared training data.

The AIMLTrainingRequest may have a requestStatus field to represent the status of the specific AIMLTrainingRequest:

-	The attribute values are "NOT_STARTED", "TRAINING_IN_PROGRESS", "SUSPENDED", "FINISHED", and "CANCELLED".

-	When value turns to "TrainingInProcess", the AI/ML training MnS producer instantiates one or more AIMLTrainingProcess MOI(s) representing the training process(es) being performed per the request and notifies the MLT MnS consumer(s) who subscribed to the notification.

When all of the training process associated to this request are completed, the value turns to "FINISHED.

### 7.3.4	AIMLTrainingProcess

#### 7.3.4.1	Definition

The IOC AIMLTrainingProcess represents the AI/ML training process.

One AIMLTrainingProcess MOI may be instantiated for each AIMLTrainingRequest MOI or a set of AIMLTrainingRequest MOIs.

For each AIMLEntity under training, a AIMLTrainingProcess is instantiated, i.e. an AIMLTrainingProcess is associated with exactly one AIMLEntity. The AIMLTrainingProcess may be associated with one or more AIMLTrainingRequest MOI.

The AIMLTrainingProcess does not have to correspond to a specific AIMLTrainingRequest, i.e. a AIMLTrainingRequest does not have to be associated to a specific AIMLTrainingProcess. The AIMLTrainingProcess may be managed separately from the AIMLTrainingRequest MOIs, e.g. the AIMLTrainingRequest MOI may come from consumers which are network functions while the operator may wish to manage the AIMLTrainingProcess that is instantiated following the requests. Thus, the AIMLTrainingProcess may be associated to either one or more AIMLTrainingRequest MOI.

Each AIMLTrainingProcess instance needs to be managed differently from the related AIMLEntity, although the AIMLTrainingProcess may be associated to only one AIMLEntity. For example, the AIMLTrainingProcess may be triggered to start with a specific version of the AIMLEntity and multiple AIMLTrainingProcess instances may be triggered for different versions of the AIMLEntity. In either case the AIMLTrainingProcesse instances are still associated with the same AIMLEntity but are managed separately from the AIMLEntity.

Each AIMLTrainingProcess has a priority that may be used to prioritize the execution of different AIMLTrainingProcesse instances. By default, the priority of the AIMLTrainingProcess may be related in a 1:1 manner with the priority of the AIMLTrainingRequest for which the AIMLTrainingProcess is instantiated.

Each AIMLTrainingProcess may have one or more termination conditions used to define the points at which the AIMLTrainingProcess may terminate.

The "ProgressStatus" attribute represents the status of the AI/ML model training and includes information the MnS consumer can use to monitor the progress and results. The data type of this attribute is "ProcessMonitor" (see 3GPP TS 28.622 [11]). The following specializations are provided for this data type for the AI/ML training process:

The "ProgressStatus" attribute represents the status of the AI/ML model training and includes information the AI/ML training MnS consumer can use to monitor the progress and results. The data type of this attribute is "ProcessMonitor" (see 3GPP TS 28.622 [12]). The following specializations are provided for this data type for the AI/ML training process:

-	The "status" attribute values are "RUNNING", "CANCELLING", "SUSPENDED", "FINISHED", and "CANCELLED". The other values are not used.

-	The "timer" attribute is not used.

-	When the "status" is equal to "RUNNING" the "progressStateInfo" attribute shall indicate one of the following states: "COLLECTING_DATA", "PREPARING_TRAINING_DATA", "TRAINING".

-	No specifications are provided for the "resultStateInfo" attribute. Vendor specific information may be provided though.

When the training is completed with "status" equal to "FINISHED", the MLT MnS producer provides the training report, by creating an AIMLTrainingReport MOI, to the MLT MnS consumer.

#### 7.3.4.2	Attributes

Table 7.3.4.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| aIMLTrainingProcessId | M | T | T | F | T |
| priority | M | T | T | F | T |
| terminationConditions | M | T | T | F | T |
| progressStatus | M | T | F | F | T |
| cancelProcess | O | T | T | F | T |
| suspendProcess | O | T | T | F | T |
| Attribute related to role |  |  |  |  |  |
| trainingRequestRef | CM | T | F | F | T |
| trainingReportRef | M | T | F | F | T |

### 7.4.3	AIMLContext <<dataType>>

#### 7.4.3.1	Definition

The AIMLContext represents the status and conditions related to the AIMLEntity. Specially it may be one of three types of context - the ExpectedRunTimeContext, the  TrainingContext and  the RunTimeContext.

#### 7.4.3.2	Attributes

Table 7.4.3.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceEntityRef | M | T | F | F | F |
| dataProviderRef | M | T | F | F | F |

#### 7.4.3.3	Attribute constraints

None.

#### 7.4.3.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

## 7.5	Attribute definitions

### 7.5.1	Attribute properties

Table 7.5.1-1

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| aIMLEntityId | It identifies the AI/ML entity. It is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| candidateTraingDataSource | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| inferenceType | It indicates the type of inference that the AI/ML model supports. allowedValues: the values of the MDA type (see 3GPP TS 28.104 [2]), Analytics ID(s) of NWDAF (see 3GPP TS 23.288 [3]), types of inference for RAN-intelligence, and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areConsumerTrainingDataUsed | It indicates whether the consumer provided training data have been used for the AI/ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| usedConsumerTrainingData | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the AI/ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingRequestRef | It is the DN(s) of the related AIMLTrainingRequest MOI(s). allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingReportRef | It is the DN of the AIMLTrainingReport MOI that represents the reports of the AI/ML training. allowedValues: DN. | type: DN (see 3GPP TS 32.156 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lastTrainingRef | It is the DN of the AIMLTrainingReport MOI that represents the reports for the last training of the AI/ML model. allowedValues: DN. | type: DN (see 3GPP TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| confidenceIndication | It indicates the confidence (in unit of percentage) that the AI/ML model would perform for inference on the data with the same distribution as training data. allowedValues: { 0..100 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLEntityList | It describes the list of AIMLEntity. | type: AIMLEntity multiplicity: * isOrdered: False isUnique: N/True defaultValue: None isNullable: False |
| trainingRequestSource | It describes the entity that requested to instantiate the AIMLTrainingRequest MOI. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| requestStatus | It describes the status of a particular AI/ML training request. T. allowedValues: NOT_STARTED, TRAINING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLTrainingProcessId | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| priority | It indicates the priority of the training process. The priority may be used by the AI/ML training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions | It indicates the conditions to be considered by the AIMLTraining MnS producer to terminate a specific training process. allowedValues: MODEL UPDATED_IN_INFERENCE_FUNCTION, INFERENCE FUNCTION_TERMINATED, INFERENCE FUNCTION_UPGRADED, INFERENCE_CONTEXT_CHANGED.. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus | It indicates the status of the AI/ML training process. allowedValues: N/A. | type: ProcessMonitor (see TS 28.622 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLEntityVersion | It indicates the version number of the AI/ML entity. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements | It indicates the expected performance for a trained AI/ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| performanceTraining | It indicates the performance score of the AI/ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| aIMLTrainingProcess.progressStatus.progressStateInfo | It provides the following specialization for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "AIMLTrainingProcess". When the AI/ML training is in progress, and the "status" is equal to " RUNNING" it provides the more detailed progress information. allowedValues for "status" = "RUNNING": - COLLECTING_DATA - PREPARING_TRAINING_DATA - TRAINING The allowed values for "status" = "CANCELLED" are vendor specific. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName | It indicates the name of an inference output of an AI/ML entity. allowedValues: the name of the MDA output IEs (see 3GPP TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN-intelligence inference output IE name(s), and vendor's specific extensions. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric | It indicates the performance metric used to evaluate the performance of an AI/ML entity, e.g. "accuracy", "precision", "F1 score", etc. allowedValues: N/A. | Type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| performanceScore | It indicates the performance score (in unit of percentage) of an AI/ML entity when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of AI/ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the "F1 score". allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cancelRequest | It indicates whether the AI/ML training MnS consumer cancels the AI/ML training request. Setting this attribute to "TRUE" cancels the AI/ML training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " TRAINING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| suspendRequest | It indicates whether the AI/ML training MnS consumer suspends the AI/ML training request. Setting this attribute to "TRUE" suspends the AI/ML training request. Suspension is possible when the requestStatus is the not "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| cancelProcess | It indicates whether the AI/ML training MnS consumer cancels the AI/ML training process. Setting this attribute to "TRUE" cancels the AI/ML training request. Cancellation is possible when the progressStateInfo is the not "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| suspendProcess | It indicates whether the AI/ML training MnS consumer suspends the AI/ML training process. Setting this attribute to "TRUE" suspends the AI/ML training request. Suspension is possible when the progressStateInfo is the not "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| inferenceEntityRef | It describes the target entities that will use the AI/ML entity for inference. | Type: DN (see 3GPP TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| dataProviderRef | It describes the entities that have provided or should provide data needed by the AI/ML entity e.g., for training or inference | Type: DN (see 3GPP TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| areNewTrainingDataUsed | It indicates whether the other new training data have been used for the AI/ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| traingDataQualityScore | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore | It is the the numerical value that represents the dependability/quality of a given decision generated by the ML-based function. The lowest value indicates the lowest level of dependability of the decisions, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| NOTE: When the performanceScore is to indicate the performance score for AI/ML training, the data set is the training data set. |  |  |

Next Change

## B.2.1	OpenAPI document "AiMlNrm.yaml"

openapi: 3.0.1

info:

title: AI/ML NRM

version: 17.0.0

description: >-

OAS 3.0.1 specification of the AI/ML NRM

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: TS 28.105; AI/ML Management

url: http://www.3gpp.org/ftp/Specs/archive/28_series/28.105/

paths: {}

components:

schemas:

#-------- Definition of types-----------------------------------------------------

AIMLEntityList:

type: array

items:

$ref: '#/components/schemas/AIMLEntity'

AIMLEntity:

type: object

properties:

aIMLEntityId:

type: string

inferenceType:

type: string

aIMLEntityVersion:

type: string

expectedRunTimeContext:

$ref: '#/components/schemas/AIMLContext'

trainingContext:

$ref: '#/components/schemas/AIMLContext'

runTimeContext:

$ref: '#/components/schemas/AIMLContext'

AIMLContext:

type: object

properties:

inferenceEntityRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

dataProviderRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

RequestStatus:

type: string

enum:

- NOT_STARTED

- TRAINING_IN_PROGRESS

- SUSPENDED

- FINISHED

- CANCELLED

PerformanceRequirements:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

ModelPerformance:

type: object

properties:

inferenceOutputName:

type: string

performanceMetric:

type: string

performanceScore:

type: number

format: float

decisionConfidenceScore:

type: number

format: float

TrainingProcessMonitor:

description: >-

This data type is the "ProcessMonitor" data type defined in "genericNrm.yaml" with specialisations for usage in the "AIMLTrainingProcess".

type: object

properties:

aIMLTrainingProcessId:

type: string

status:

type: string

enum:

- RUNNING

- CANCELLING

- CANCELLED

- SUSPENDED

- FINSHED

progressPercentage:

type: integer

minimum: 0

maximum: 100

progressStateInfo:

type: string

enum:

- COLLECTING_DATA

- PREPARING_TRAINING_DATA

- TRAINING

resultStateInfo:

type: string

#-------- Definition of abstract IOCs --------------------------------------------

#-------- Definition of concrete IOCs --------------------------------------------

SubNetwork-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/SubNetwork-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/SubNetwork-ncO'

- type: object

properties:

SubNetwork:

$ref: '#/components/schemas/SubNetwork-Multiple'

ManagedElement:

$ref: '#/components/schemas/ManagedElement-Multiple'

AIMLTrainingFunction:

$ref: '#/components/schemas/AIMLTrainingFunction-Multiple'

ManagedElement-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

$ref: 'genericNrm.yaml#/components/schemas/ManagedElement-Attr'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedElement-ncO'

- type: object

properties:

AIMLTrainingFunction:

$ref: '#/components/schemas/AIMLTrainingFunction-Multiple'

AIMLTrainingFunction-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-Attr'

- type: object

properties:

aIMLEntityList:

$ref: '#/components/schemas/AIMLEntityList'

- $ref: 'genericNrm.yaml#/components/schemas/ManagedFunction-ncO'

- type: object

properties:

AIMLTrainingRequest:

$ref: '#/components/schemas/AIMLTrainingRequest-Multiple'

AIMLTrainingProcess:

$ref: '#/components/schemas/AIMLTrainingProcess-Multiple'

AIMLTrainingReport:

$ref: '#/components/schemas/AIMLTrainingReport-Multiple'

AIMLTrainingRequest-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

aIMLEntityId:

type: string

candidateTraingDataSource:

type: array

items:

type: string

traingDataQualityScore:

type: number

format: float

trainingRequestSource:

type: string

requestStatus:

$ref: '#/components/schemas/RequestStatus'

expectedRuntimeContext:

$ref: 'comDefs.yaml#/components/schemas/DateTime'

performanceRequirements:

$ref: '#/components/schemas/PerformanceRequirements'

cancelRequest:

type: boolean

suspendRequest:

type: boolean

AIMLTrainingProcess-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

aIMLTrainingProcessId:

type: string

priority:

type: integer

terminationConditions:

type: string

progressStatus:

$ref: '#/components/schemas/TrainingProcessMonitor'

cancelProcess:

type: boolean

suspendProcess:

type: boolean

trainingRequestRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/DnList'

trainingReportRef:

$ref: 'TS28623_ComDefs.yaml#/components/schemas/Dn'

AIMLTrainingReport-Single:

allOf:

- $ref: 'genericNrm.yaml#/components/schemas/Top'

- type: object

properties:

attributes:

allOf:

- type: object

properties:

aIMLEntityId:

type: string

areConsumerTrainingDataUsed:

type: boolean

usedConsumerTrainingData:

type: array

items:

type: string

confidenceIndication:

type: integer

modelPerformanceTraining:

type: array

items:

$ref: '#/components/schemas/ModelPerformance'

areNewTrainingDataUsed:

type: boolean

#-------- Definition of JSON arrays for name-contained IOCs ----------------------

SubNetwork-Multiple:

type: array

items:

$ref: '#/components/schemas/SubNetwork-Single'

ManagedElement-Multiple:

type: array

items:

$ref: '#/components/schemas/ManagedElement-Single'

AIMLTrainingFunction-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingFunction-Single'

AIMLTrainingRequest-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingRequest-Single'

AIMLTrainingProcess-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingProcess-Single'

AIMLTrainingReport-Multiple:

type: array

items:

$ref: '#/components/schemas/AIMLTrainingReport-Single'

#-------- Definitions in 3GPP TS 28.104 for 3GPP TS 28.532 ---------------------------------

resources-AiMlNrm:

oneOf:

- $ref: '#/components/schemas/SubNetwork-Single'

- $ref: '#/components/schemas/ManagedElement-Single'

- $ref: '#/components/schemas/AIMLTrainingFunction-Single'

- $ref: '#/components/schemas/AIMLTrainingRequest-Single'

- $ref: '#/components/schemas/AIMLTrainingProcess-Single'

- $ref: '#/components/schemas/AIMLTrainingReport-Single'

End of Change