3GPP TSG-SA5 Meeting #147 	S5-232384

Athens, Greece, 27th February - 3rd March 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.422 | CR | 409 | rev |  | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify behaviour when cell list is not specified |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2023-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The cell list parameter in trace job is optional but the expected behaviour for when the trace target is absent is undefined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the expected handling for undefined trace target. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Potential interoperability issue. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1.6, 4.1.1.9.1, (new) 5.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

#### 4.1.1.6	E-UTRAN activation mechanisms

In E-UTRAN the Management Based Trace Activation can be fulfilled with the E-UTRAN Cell Traffic trace functionality. In this case the Trace Session Activation is done to one or a list E-UTRAN cells within one eNodeB, where Trace Session is activated.

The following trace control and configuration parameters of the Trace Session are received by eNodeB in the Trace Session activation message from the EM:

-	Trace Reference

-	Trace Depth

-	Trace Target: list of E-UTRAN cells

-	List of Interfaces for eNB

-	Trace Collection Entity IP Address

When eNodeB receives the Trace Session Activation message from the EM for a given or a list of E-UTRAN cell(s) the eNodeB shall start a Trace Session for the given or list of E-UTRAN cell(s).  If no list of E-UTRAN cell(s) is present the eNodeB shall start a Trace Session for all cells.

2nd Change

#### 4.1.1.9	NG-RAN activation mechanisms

##### 4.1.1.9.1	General

In NG-RAN the Management Based Trace Activation can be fulfilled with the NG-RAN Cell Traffic trace functionality. In this case the Trace Session Activation is done to one or a list NG-RAN cells within one NG-RAN node, where Trace Session is activated.

The following trace control and configuration parameters of the Trace Session are received by NG-RAN node in the Trace Session activation message from the management system:

-	Trace Reference.

-	Trace Depth.

-	Trace Target: NG-RAN cells list.

-	List of Interfaces for NG-RAN node.

-	Trace Collection Entity IP Address for the file-based trace reporting or Trace Reporting Consumer URI for the streaming trace reporting.

-	Trace Reporting Format.

When NG-RAN node receives the Trace Session Activation message from the management system for a given or a list of NG-RAN cell(s) the NG-RAN node shall start a Trace Session for the given or list of NG-RAN cell(s).  If no list of NG-RAN cell(s) is present the NG-RAN node shall start a Trace Session for all cells.

3rd Change

## 5.x	Trace Target: list of cells (O)

This optional parameter specifies a list of cells.

When NG-RAN node receives the Trace Session Activation message from the management system for a given or a list of NG-RAN cell(s) the NG-RAN node shall start a Trace Session for the given or list of NG-RAN cell(s).  If no list of NG-RAN cell(s) is present the NG-RAN node shall start a Trace Session for all cells.

When eNodeB receives the Trace Session Activation message from the EM for a given or a list of E-UTRAN cell(s) the eNodeB shall start a Trace Session for the given or list of E-UTRAN cell(s).  If no list of E-UTRAN cell(s) is present the eNodeB shall start a Trace Session for all cells.