3GPP TSG-SA5 Meeting #147 	S5-232548

Athens, Greece, 27th February - 3rd March 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.535 | CR | 0068 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update coordination between closed control loops |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 4.2.3, providing governance and goals for the 5GC management domain and NG-RAN management domain are missing. It only contains providing governance and goals for 5GC NFs and gNBs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The missing part of providing governance and goals for the 5GC management domain and NG-RAN management domain are added. Providing goals for a closed control loop are according to goals decompostion of a higher level closed control loop, its goal fulfilment status may also be considered. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how goals for 5GC management domain and NG-RAN management domain are provided in the coordination between closed control loops. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2.3	Coordination between closed control loops

Different closed control loops reside in the management domains or in the network functions to support automation and the autonomous networks. Different domains may involve overlapping or nonoverlapping coverage areas. The purposes and results of a closed control loop may have impacts on other closed control loops. Coordination between closed control loops is needed for example in and between, the Cross Management Domain and 5GC management domain or the NG-RAN management domain, to improve the performance in order to achieve the goal(s) of the closed control loops. Furthermore, coordination may also be needed when conflicts happen between closed control loops related to their activities.

A closed control loop may coordinate with other closed control loops in the same domain or in a different domain. Closed control loops in domain management for 5GC and NG-RAN are responsible for local optimization. Closed control loops in cross management domain may need to coordinate with closed control loops in multiple other management domains for the end to end optimization.

The relationships between closed control loops can be hierarchical and/or peer-to-peer. Coordination in the management domains include the following categories:

- Coordination between Cross Management Domain and the 5GC Management Domain

- Coordination between Cross Management Domain and the NG-RAN Management Domain

- Coordination within Cross Management Domain, 5GC Management Domain or NG-RAN Management Domain

- Coordination within:

- Cross Management Domain,

- 5GC Management Domain and

- NG-RAN Management Domain

Coordination in management domains provides the SLS assurance from the overall management perspective. The closed control loop in the Cross Management Domain provides governance and goals for the closed control loops in the 5GC Management Domain and the NG-RAN Management Domain. The closed control loop in the 5GC management domain provides governance and goals for the 5GC NFs. The closed control loop in the NG-RAN management domain provides governance and goals for the gNBs. The configuration of goals for a closed control loop may be decided according to its goal fulfilment status and the goal decomposition from a higher level closed control loop.

Closed Loop Coordination (CLC) of Hierarchical and peer-to-peer interactions are described in ETSI GS ZSM 009-1 [11].