# 3GPP TSG-SA5 Meeting #147	S5-232696

# 27 Feb to 3 Mar 2023, E-meeting

# Source:	China Telecom

# Title:	Provide description of the requirements having been discussed in KI#5 TR28864

# Document for:	Approval

# Agenda Item:	6.7.6.1

# 1	Decision/action requested

The group is asked to discuss and agree on the proposal.

# 2	References

[1]	3GPP TR 28.864-050: "Study on Enhancement of the management aspects related to NWDAF".

# 3	Rationale

It's better to provide explicit requirements based on description of the KI#5 and the potential solutions discussed, so that the reader can follow the discussion easier.

# 4	Detailed proposal

Start of 1st Change

### 4.5.1      Description

Data collection play an important role for the NWDAF providing analytic services and model training services.

In the Rel-17, the enhanced data collection feature is introduced, with which the data collection may be performed via Nnwdaf DataManagement services provided by NWDAF.

The Nnwdaf_DataManagement services enable the consumer to subscribe/unsubscribe, be notified about data exposed by NWDAF, or fetch the subscribed data. The consumer of the Nnwdaf_DataManagement services can be the other NWDAF instances.

The operator may be interested in the how often the Nnwdaf_DataManagement services are requested or how many data are provided via Nnwdaf_DataManagement services.

On one hand, the service provider NWDAF will prepare the data before they are provided to consumer, and since the preparation includes both collection and processing of the data from the other data sources and historical data having been collected by NWDAF, this measurement will indicate how busy the NWDAF is on data collection related services which can be taken as a reflection of the overhead variation of NWDAF. This measurement will also lead to adjusting the network resource allocated to the service provider NWDAF instance to support data exchange with other consumers. And on some other cases, this measurement may indicate that deploying a dedicated DCCF is necessary.

On the other hand, as the enhanced data collection feature is introduced for Rel-17 NWDAF to increases data collection efficiency, this measurement will be a complimentary for the performance measurement of the NWDAFs which does not support DataManagement services. This measurement will also provide information on how many data collection is performed with the more effective manner, which can contribute the potential reduction of the CAPEX.

REQ-NWDAF-DM-SERV-COUNT-1：the 3GPP management system shall have a capability to measure the number subscriptions/requests received by the NWDAF for data management services.

REQ-NWDAF-DM-SERV-COUNT-2：the 3GPP management system shall have a capability to measure the number notifications/responses generated by the NWDAF for data management services.

In this key issue, the potential solutions are provided on the performance measurement about the Nnwdaf_DataManagement services provided by NWDAF.

End of 1st Changes