3GPP TSG-SA5 Meeting #148-e 	S5-233401

e-meeting, 17 April - 25 April 2023

# Source:	China Unicom

# Title:	pCR 28.841 add use case for monitoring of satellite components

# Document for:	Approval

# Agenda Item:	6.8.7.1

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1] 		3GPP draft TR 28.841: " Study on Management Aspects of IoT NTN Enhancements v0.4.0".

3	Rationale

This pCR is to add use cases for TR 28.841[1].

4	Detailed proposal

It proposes to make the following changes to the draft TR 28.841[1].

1st Change

5.1.2.2              Use case for monitoring the coverage holes or discontinuous satellite coverage

5.1.2.2.1 	Goal

The goal is to allow the monitoring of the coverage holes or discontinuous satellite coverage for idle mode mobility enhancements

5.1.2.2.2 	Pre-conditions

A NOP operates a non-terrestrial networks for BL UEs, UEs in enhanced coverage and NB-IoT UEs.

The  network as operated by the NOP satellite RAN supporting NB-IoT/eMTC.

5.1.2.2.3 	Description

As a satellite moves on a specified orbit, for example in case of a NGSO satellite, the satellite beam(s) coverage area may move and cover different portions of a geographical area due to the orbital movement of the satellite. As a consequence, a UE located in the concerned geographical area may experience a situation of discontinuous coverage, due to e.g., a sparse satellite constellation deployment.

The management system of the NB-IoT/eMTC Non-Terrestrial Network is designed to provide a measured of the  coverage holes or discontinuous satellite coverage

5.1.2.2.4 	Post-description

The management system monitors the  coverage holes or discontinuous satellite coverage.

End of change