3GPP TSG-SA5 Meeting #149 	S5-234492

# Berlin, Germany, 22nd May 2023 - 26th May 2023						     Revision of (S5-234435)

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Satellite backhaul Adding new use case

# Document for:	Approval

Agenda Item:	7.5.6

# 1	Decision/action requested

Approve new satellite backhaul use case.

# 2	References

[1]	3GPP TR 28.844 " Study on charging aspects of satellite in the 5G System (5GS)".

# 3	Rationale

Sattellite backhaul use case which makes use of QoS for SSC Charging.

# 4	Detailed proposal

The following changes are proposed for TR 28.844[1].

First change

#### 5.2.1.1	Use Case #2.1: MNO charging subscriber based on monitored QoS

This use case focuses on SSC, MNO and Satellite Provider business roles.

A SSC has a subscription with the MNO which allows usage of 5GS for satellite backhaul. The MNO can provide different QoS (e.g. delay) to the SSC because of the different satellite backhaul categories.

For this case, the charging party and charged party can be:

-	Charged party: SSC access a gNB which is using satellite backhaul link to CN which is consuming a service available from MNO.

-	Charging party: MNO who provides the satellite communication service to the SSC, and the Satellite Service Provider who provides the satellite to MNO.

The SSC can be charged, either by the MNO or Satellite Provider, on QoS whilst the service is being consumed via satellite.

The potential charging requirements for this Use Case are: REQ-CH_ SATBH_DC-01 and REQ-CH_ SATBH_DC-02.

End of changes