3GPP TSG-SA5 Meeting #150 	S5-235101

# Göteborg, Sweden, 21-25 August 2023

# Source:	Ericsson, Deutsche Telekom

# Title:	Add evaluation of alternatives in clause 6

# Document for:	Approval, Information, Discussion

Agenda Item:	6.7.3.4

# 1	Decision/action requested

The group is asked to discuss and agree this proposal

# 2	References

[1]	3GPP TS 28.312

[2]	3GPP TS 28.801

# 3	Rationale

The study proposes two alternatives in 6.2 Solution for expressing service requirements as intent expectations. This contribution makes an evaluation of the two alternatives.

.

# 4	Detailed proposal

First change

### 6.2.x	Evaluation

#### 6.2.X.1	Overview of alternatives

The alternatives described in 6.2.2 and 6.2.3 are evaluated based on what information elements must be specified or enhanced for a solution based on standards.

Table 6.2.x-1 Overview of identified new information needed for each alternative

| Alt. | Expectation on | To be defined in TS 28.312 | Examples of objects that may be allocated as result of an expectation (TS 28.541) |
| --- | --- | --- | --- |
| 1 | NetworkSlice RANSliceSubnet CNSliceSubnet | NetworkSliceExpectation RANSliceSubnetExpectation CNSliceSubnetExpectation ObjectType=NetworkSlice or RANSliceSubnet or CNSliceSubnet | NetworkSlice <<IOC>> with SliceProfile NetworkSliceSubnet <<IOC>> with RANSliceSubnetProfile NetworkSliceSubnet <<IOC>> with CNSliceSubnetProfile |
| 2 | CommunicationService RadioService CNService | CommunicationServiceExpectation RadioServiceExpectation CNServiceExpectation ObjectType=CommunicationService or RadioService or CNService | NetworkSlice <<IOC>> with SliceProfile NetworkSliceSubnet <<IOC>> with RANSliceSubnetProfile NetworkSliceSubnet <<IOC>> with CNSliceSubnetProfile |
| NOTE 1: Information imported from TS 28.541 is specified and only needs to be referenced to, from TS 28.312. NOTE 2: Expectation on Radio Service is described in [7] similarly, a consumer may have expectations on 5GC services which for this report is called CNService. |  |  |  |

#### 6.2.X.2	Evaluation

Alternative 1: Requires specification of two new dataTypes; NetworkSliceExpectation and, RadioServiceExpectation . The mapping from ServiceProfile to the  NetworkSliceExpectations can be done in normative work.

Alternative 2: Requires specification of two new dataTypes CommunicationServiceExpectation and, RadioServiceExpectation. The generic study for IDMS enhancements [7] also identified the need for RadioServiceExpectation. The mapping from ServiceProfile to the ServiceExpectations can be done in normative work.

Editor’s Note: The need for a 5GC specific expectation for example a CNServiceExpectation in the context of network slicing is FFS

End of change