3GPP TSG-SA5 Meeting #150 	S5-235121

Goteborg, Sweden, 21 - 25 August 2023

# Source:	MATRIXX Software

# Title:	pCR 28.204 Introduce IEC message flow

# Document for:	Approval

Agenda Item:	7.4.2

# 1	Decision/action requested

This pCR is to introduce IEC message flow

# 2	References

[1]		3GPP TR 32.847 "Study on Charging Aspects for Network Slicing Phase 2"

[2]	3GPP TS 28.204 "Network slice-specific authentication and authorization charging in the 5G System (5GS)".

# 3	Rationale

This pCR is to introduce IEC message flow.

# 4	Detailed proposal

The following changes are proposed to be incorporated into TS 28.204 [2]:

First change

##### 5.X.2.2.z	Network slice-specific authentication and authorization - IEC

The following figure 5.X.2.2.z-1 describes a Network slice-specific authentication and authorization charging in IEC scenario for NSSAAF and AMF in PEC scenario

Figure 5.X.2.2.z-1: Network slice-specific authentication and authorization - IEC

Steps 1 to 4 per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure, AMF sends the EAP Identity Response to the NSSAAF.

4ch-a: NSSAAF sends Charging Data Request [Event] to CHF with GPSI and S-NSSAI.

4ch-b: Account, Rating, control  by the CHF.

4ch-c: The CHF creates a CDR.

4ch-d: CHF provides response to NSSAAF.

Steps 5 to 18 per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

18ch-a: NSSAA procedure in AMF completed:  AMF sends Charging Data Request [Event] to CHF with EAP-Success/Failure, GPSI and S-NSSAI.

18ch-b: The CHF creates a CDR.

18ch-c: CHF provides response to AMF.

End of change