3GPP TSG-SA5 Meeting #150 	S5-235131

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.256 | CR | 0013 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on AMF identifier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MATRIXX Software |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The AMF Charging identifier is part of Charging Data Request message contents Table 6.1.1.2.1 but missing in CHF CDR data for each Registration Charging, N2 Connection Charging and Location Reporting Charging. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add AMF Identifier in Registration CHF CDR data, N2 Connection CHF CDR data and Location Reporting CHF CDR data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing AMF Identifier will lead to incorrect charging |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.2, 6.1.3.3, 6.1.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 32.291 CR#0482 |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 32.298 CR#0947 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

#### 6.1.3.2	Registration charging CHF CDR data

If enabled, CHF CDRs for Registration charging shall be produced for each registration and deregistration

The fields of Registration charging CHF CDR are specified in table 6.1.3.2.1.

Table 6.1.3.2.1: Registration charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| Subscriber Identifier | OM | This field holds the 5G Subscription Permanent Identifier (SUPI) of the served party. This fields should be present except for emergency session. |
| NF Consumer Information | M | This field holds the information of the AMF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. AMF) |
| NF Name | OC | This field holds the name of the AMF used. |
| NF Address | OC | This field holds the IP Address of the AMF used. |
| NF PLMN ID | Oc | This field holds the PLMN identifier (MCC MNC) of the AMF. |
| Record Opening Time | M | Described in TS 32.298 [51] |
| Duration | M | Described in TS 32.298 [51] |
| Record Sequence Number | C | Described in TS 32.298 [51] |
| Cause for Record Closing | M | Described in TS 32.298 [51] |
| Diagnostics | OM | Described in TS 32.298 [51] |
| Local Record Sequence Number | OM | Described in TS 32.298 [51] |
| Record Extensions | OC | Described in TS 32.298 [51] |
| AMF Identifier | OC | This field holds the AMF identifier. |
| Registration Charging Information | OM | This field holds the 5G registration specific information defined in clause 6.2.1.2. |

Next change

#### 6.1.3.3	N2 connection charging Information CHF CDR data

If enabled, CHF CDRs for N2 connection charging shall be produced for each N2 connection establishment and release.

The fields of N2 connection charging CHF CDR are specified in table 6.1.3.3.1.

Table 6.1.3.3.1: N2 connection charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| Subscriber Identifier | OM | This field holds the 5G Subscription Permanent Identifier (SUPI) of the served party. This fields should be present except for emergency session. |
| NF Consumer Information | M | This field holds the information of the AMF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. AMF) |
| NF Name | OC | This field holds the name of the AMF used. |
| NF Address | OC | This field holds the IP Address of the AMF used. |
| NF PLMN ID | Oc | This field holds the PLMN identifier (MCC MNC) of the AMF. |
| Record Opening Time | OC | Described in TS 32.298 [51] |
| Duration | M | Described in TS 32.298 [51] |
| Record Sequence Number | C | Described in TS 32.298 [51] |
| Cause for Record Closing | M | Described in TS 32.298 [51] |
| Local Record Sequence Number | OM | Described in TS 32.298 [51] |
| Record Extensions | OC | Described in TS 32.298 [51] |
| AMF Identifier | OC | This field holds the AMF identifier. |
| N2 connection charging Information | OM | This field holds the N2 connection specific information defined in clause 6.2.1.3 |

Next change

#### 6.1.3.4	Location reporting charging CHF CDR data

If enabled, CHF CDRs for Location reporting charging shall be produced for each Location reporting.

The fields of Location reporting charging CHF CDR are specified in table 6.1.3.4.1.

Table 6.1.3.4.1: Location reporting charging CHF record data

| Field | Category | Description |
| --- | --- | --- |
| Record Type | M | CHF record. |
| Recording Network Function ID | OM | This field holds the name of the recording entity, i.e. the CHF id. |
| Subscriber Identifier | OM | This field holds the 5G Subscription Permanent Identifier (SUPI) of the served party. This fields should be present except for emergency session. |
| NF Consumer Information | M | This field holds the information of the AMF that used the charging service. |
| NF Functionality | M | This field contains the function of the node (i.e. AMF) |
| NF Name | OC | This field holds the name of the AMF used. |
| NF Address | OC | This field holds the IP Address of the AMF used. |
| NF PLMN ID | Oc | This field holds the PLMN identifier (MCC MNC) of the AMF. |
| Record Opening Time | M | Described in TS 32.298 [51] |
| Duration | M | Described in TS 32.298 [51] |
| Record Sequence Number | C | Described in TS 32.298 [51] |
| Cause for Record Closing | M | Described in TS 32.298 [51] |
| Local Record Sequence Number | OM | Described in TS 32.298 [51] |
| Record Extensions | OC | Described in TS 32.298 [51] |
| AMF Identifier | OC | This field holds the AMF identifier. |
| Location reporting charging Information | OM | This field holds the Location reporting specific information defined in clause 6.2.1.4. |

End of changes