3GPP TSG-SA5 Meeting #150 	S5-235141

# Goteborg, Sweden, 21 Aug 2023 – 25 Aug 2023

# Source:	Nokia, Nokia Shanghai Bell, Amdocs

# Title:	Key Issue for CHF Selection #2

# Document for:	Approval

Agenda Item:	7.5.4

# 1	Decision/action requested

Include the proposed changes in TR 28.840

# 2	References

[1]		3GPP TR 28.840: "Study on CHF Segmentation"

# 3	Rationale

This pCR proposes the Topic 2 and key Issue in TR 28.840 "Study on CHF Segmentation".

# 4	Detailed proposal

Propose to include the following change into the TR 28.840 [1].

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 29.510: "Network Function Repository Services".

[3]	3GPP TS 32.290: " Services, operations and procedures of charging using Service Based Interface (SBI) ".

[4]	3GPP TS 32.255 “5G data connectivity domain charging; Stage 2”

[5]	3GPP TS 23.501 “System architecture for the 5G System (5GS);Stage 2”

Second change

# 5	Key Issues

## 5.1	Key Issue #1 CHF Discovery by NF Consumers

## 5.2	Topic 2: CHF Selection based on SUPI or Group ID

### 5.2.1		General description and assumptions

Currently, based on TS 23.501 [5], the CHF selection functionality in NF consumer or in SCP should consider one of the following factors:

- CHF Group ID of the UE's SUPI.

NOTE:	The NF Consumer can infer the CHF Group ID the UE's SUPI belongs to, based on the results of CHF discovery procedures with NRF.

- SUPI; the NF consumer selects a CHF instance based on the SUPI range the UE's SUPI belongs to or based on the results of a discovery procedure with NRF using the UE's SUPI as input for CHF discovery.

In the case of delegated discovery and selection in SCP, the NF consumer shall include all available factors in the request towards SCP. As a result, this procedure enforces the CHF to configure up front which CHF shall serve each SUPI or Group ID and to sync on any internal change of the SUPI while moving between CHF sites.

Therefore, it should be improved the time required for such operation, for use cases which make use of latency as one the main requirements.

### 5.2.2	Potential charging requirements

The following are potential high-level charging requirements, derived from the requirements in TS 32.255 [4].

REQ-CH_ SELCHF_LA -01: The 5GS should be able to optimize the CHF Selection during the PDU Session Establishment

### 5.2.3	Key issues#2.1: Charging events and charging information required

This key issue is for investigating how to support the selection of a CHF Instance based on the Location considering REQ-CH_ SELCHF_LA -01. This investigation covers the following:

-	CHF Instance Selection Time reduction;

-	identification and classification of the charging information required to reduce the time of CHF Instance Selection.

### 5.2.4	Possible Solutions

### 5.2.5	Evaluation

### 5.2.6	Conclusion

## 5.3	Key Issue #3 CHF Selection with Extended Subscription Data

End of changes