3GPP TSG-SA5 Meeting #150	S5-235155

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.310 | CR | 0030 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.310 Update NG-RAN data EE KPI definition with reference to TS 28.554 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | All EE KPIs are defined in TS 28.554: Management and orchestration; 5G end to end Key Performance Indicators (KPI), where the KPIs are defined along with the formula. The information in clause 6.1 in TS 28.310 has redundant NG-RAN data EE KPI definition, which has already been specified in clause 6.7.1 of TS 28.554. Hence this contribution removes these KPI definition in TS 28.310, and adds description of the KPI and reference to the clause 6.7.1 of TS 28.554 that defines the KPI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NG-RAN data EE KPI definition in clause 6.1 is replaced with a description of the KPI and reference to clause clause 6.7.1 of TS 28.554. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Duplicate information in multiple specifications may cause inconsistencies and confusion for the implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - Rel-16 tDoc reference S5-235153 - Rel-17 tDoc reference S5-235154 - Clause 2 of Rel-18 TS 28.310 already has a reference to TS 28.554, and hence clause 2 is not affected in this mirror CR. - Clause 6.1.1 has been introduced from Rel-17 with the contents of Rel-16 clause 6.1. Hence the same changes in this mirror CR is reflected 6.1.1 instead of 6.1 as in Rel-16 CR S5-235153. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1. S5-233889 is revised to S5-235155 |  |  |  |  |  |  |  |  |

1st Change

## 6.1	Solutions for assessment of mobile network data energy efficiency

### 6.1.1	Energy efficiency of NG-RAN

Assessment of NG-RAN data EE is based on the high-level mobile network data EE KPI defined in clause 3.1 and clause 5.3 of ETSI ES 203 228 [2]:

NG-RAN data EE KPI is is obtained by the data volume divided by Energy Consumption (EC) of the considered network elements. The KPI is defined for both non-split and split gNB scenarios. This KPI is defined in clause 6.7.1 of TS 28.554 [X].

The following PEE (Power, Energy and Environmental) measurement may be used as the ECMN:

-	PNF Energy consumption (cf. clause 5.1.1.19.3 of TS 28.552 [15]): This measurement provides the energy consumed (in kilowatt-hours) by the subject gNB.

### 6.1.2	Energy efficiency of network slices

#### 6.1.2.1	Introduction

Assessment of the energy efficiency of 5G network slices is based on KPIs defined in TS 28.554 [18] clause 6.7.2.

#### 6.1.2.2	Generic Network Slice Energy Efficiency (EE) KPI

The Generic network slice EE KPI is defined as:

, where ‘Performance of network slice’ (Pns) is defined per type of network slice.

#### 6.1.2.3	Energy efficiency of eMBB network slice

This KPI is obtained by the sum of UL and DL data volumes at N3 interface(s) of the network slice, divided by the energy consumption of the network slice. The unit of this KPI is bit/J.

The following measurements are used:

- GTP.InDataOctN3UPF.SNSSAI: Number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [15] clause 5.4.1.3;

- GTP.OutDataOctN3UPF.SNSSAI: Number of octets of outgoing GTP data packets on the N3 interface, from UPF to (R)AN, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [15] clause 5.4.1.4.

#### 6.1.2.4	Energy efficiency of URLLC network slice

##### 6.1.2.4.1	Introduction

This KPI is defined with two variants.

##### 6.1.2.4.2	Based on latency of the network slice

This KPI is obtained by the inverse of the average end-to-end User Plane (UP) latency of the network slice divided by the energy consumption of the network slice. The unit of this KPI is (0.1ms * J)-1.

In this KPI variant, latency is the only factor considered for evaluating the performance of network slice, i.e. the performance of network slice (Pns) for URLLC type of network slice is the inverse of the end-to-end User Plane (UP) latency of the network slice.

Where Network slice mean latency is defined as:

The following KPIs are used to calculate Network slice mean latency:

- DelayE2EUlNs: Average e2e uplink delay for a network slice, defined in TS 28.554 [18] clause 6.3.1.8.1 as the average e2e UL packet delay between the PSA UPF and the UE for a network slice;

- DelayE2EDlNs: Average e2e downlink delay for a network slice, defined in TS 28.554 [18] clause 6.3.1.8.2 as the average e2e DL packet delay between the PSA UPF and the UE for a network slice.

##### 6.1.2.4.3	Based on both latency and Data Volume (DV) of the network slice

This KPI is obtained by the product of the sum of UL and DL traffic data volumes at N3 interface(s) of the network slice multiplied by the inverse of the end-to-end User Plane (UP) latency of the network slice, divided by the energy consumption of the network slice. The unit of this KPI is bit/(0.1ms*J).

In this KPI variant, data volume and latency are two factors considered for evaluating the performance of network slice, i.e. the performance of network slice (Pns) for URLLC type of network slice is the sum of UL and DL traffic data volumes at N3 interface(s) multiplied by the inverse of the end-to-end User Plane (UP) latency of the network slice. This solution is applicable for the case of URLLC network slice is deployed and operators want to evaluate the slice EE KPI for different periods of time such as busy time slots (more DV) and idle time slots (less DV).

The following measurements, defined in TS 28.552 [15], are used to calculate the sum of UL and DL traffic data volumes at N3 interface(s) of the network slice:

- GTP.InDataOctN3UPF.SNSSAI: Number of octets of incoming GTP data packets on the N3 interface, from (R)AN to UPF, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [15] clause 5.4.1.3;

- GTP.OutDataOctN3UPF.SNSSAI: Number of octets of outgoing GTP data packets on the N3 interface, from UPF to (R)AN), where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [15] clause 5.4.1.4.

The following KPIs, defined in TS 28.554 [18], are used to calculate the latency of the network slice:

- DelayE2EUlNs: Average e2e uplink delay for a network slice, defined in TS 28.554 [18] clause 6.3.1.8.1 as the average e2e UL packet delay between the PSA UPF and the UE for a network slice;

- DelayE2EDlNs: Average e2e downlink delay for a network slice, defined in TS 28.554 [18] clause 6.3.1.8.2 as the average e2e DL packet delay between the PSA UPF and the UE for a network slice.

#### 6.1.2.5	Energy efficiency of MIoT network slice

##### 6.1.2.5.1	Introduction

This KPI is defined with two variants.

##### 6.1.2.5.2	Based on the number of registered subscribers of the network slice

This KPI is obtained by the mean number of active UEs of the network slice divided by the energy consumption of the network slice. The unit of this KPI is UE/J.

The following measurement is used:

- RM.RegisteredSubNbrMax.SNSSAI: maximum number of registered state subscribers per AMF, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [15] clause 5.2.1.2.

##### 6.1.2.5.3	Based on the number of active UEs in the network slice

This KPI is obtained by the mean number of active UEs of the network slice divided by the energy consumption of the network slice. The unit of this KPI is UE/J.

The following measurements are used:

- DRB.MeanActiveUeDl.SNSSAI, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [15] clause 5.1.1.23.1.

- DRB.MeanActiveUeUl.SNSSAI, where SNSSAI identifies the S-NSSAI, as defined in TS 28.552 [15] clause 5.1.1.23.3.

End of change