3GPP TSG-SA5 Meeting #150	S5-235316

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 Input to draft CR TS28.105 Training data effectiveness reporting and analytics use cases and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use cases of “Training data effectiveness reporting and analytics” have been discussed and agreed upon in the study (clause 5.1.6 in TR 28.908). This CR is to normatively propose the use cases and requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the use cases description and requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.M (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.105 DraftCR S5-235139 |  |  |
| Other comments: |  | Input to DraftCR TS 28.105 DraftCR S5-235139 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 6.2.M	ML training Data Management

### 6.2.M.1	Description

In analytics solutions, Performance Measurements (PMs) and Fault Reports (FRs) from various network function are collected and analytics applied on the PMs and FRs to derive events (statistical insights and predictions). For most algorithms, the prediction accuracy depends upon the amount of available relevant historical data, motivating the need to store ever more data, which correspondingly increases the storage and processing resource requirements. However, not all recorded data is useful as the derived events, e.g., captured through analytics processes, may have loss of information OR misinformation e.g., with respect to time of the event.

For ML model training, a large amount of data instances does not necessarily add value, e.g., if only a portion contribute to actual model training, the other portion will be discarded (e.g., by some well-designed algorithms). During ML model training the information can be provided on whether a specific sample is useful for the training or not and on how much such a sample is useful for the training.

### 6.2.M.2	Use cases

#### 6.2.M.2.x	Reporting Training data effectiveness

Training data effectiveness refers to the process of evaluating the contribution of a single data instance to model training process.

To train a ML model, high quality and large volume of training data instances are mandatory. The general practice is to collect as much data as possible and feed them to the ML model for pre-processing and training, in the hope to get high quality of trained model. This is usually done without considering the possibly different contributions of the different portions of input data samples to the accuracy of the trained model.  However, this open use of all available data is expensive both on data collection and on computation resources as even the unnecessary data samples are computed through the ML model. One solution is to leave the challenge to the specific ML model training function to optimize the training data usage during model training, or before every training or retraining, e.g., by simply resampling the training data to only use part of the collected training data. However, in mobile networks where the amount of data is large, resources are constrained yet models need to be very accurate, these methods may be inappropriate. Instead, it is better that the training function evaluates the usefulness of different data samples or features and indicated that usefulness to the consumer so that the data used for retraining can be optimized.

The 3GPP management system needs to support means to report the contributions to model training of the different training data instances/samples based on insight of how the different portion of data contribute differently to the model accuracy.

#### 6.2.M.2.y	Training data effectiveness analytics

A single/independent observation on whether a certain sample at a given timestamp contributed to model training (e.g., gradients calculation during model training) cannot provide understanding on whether using such a sample will contribute to model accuracy of the same model in further training/re-trainings, or if it will contribute to efficiency of training of further models related to the same use case.

To have such understanding, further analysis of the training data related to the importance of the data instances during training is needed. The patterns of the most effective training data generated from the analytics would be very helpful to improve data collection to optimize the quantity and quality of the data to be used for training.

### 6.2.M.3	Requirements for ML training Data Management

Table 6.2.M.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-TRAIN_EFF-01: | The MLT MnS producer should have the capability enabling an authorized consumer to configure an ML training function to report the effectiveness of data used for ML training. | Reporting Training data effectiveness (clause 6.2.M.2.x) |
| REQ-TRAIN_EFF-02: | The MLT MnS producer should have the capability enabling an authorized consumer to activate the ML training function /Entity to label the effectiveness of data used for training. | Training data effectiveness analytics (clause 6.2.M.2.y) |
| REQ-TRAIN_EFF-03: | The MLT MnS producer should have the capability enabling an authorized consumer to request analytics for data used for model training with respect to effectiveness of such data during ML training. | Training data effectiveness analytics (clause 6.2.M.2.y) |

End of Modification