3GPP TSG-SA5 Meeting #150 	S5-235331

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0083 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.312 Resolve intent conflict |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | How to resolve intent conflict is not descrbied in current TS28.312. This CR is to normatively propose the proceduer of intent conflict and new attributes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The proceduer of intent conflict with intent report and current procedure. Add the attributes to intent report |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The proceduer of intent conflict shall not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

5.3.2	Intent report

5.3.2.1	Introduction

The Fulfilment information (including the intentFulfilmentInfo, expectationFulfilmentInfo and targetFulfilmentInfo) are defined for the MnS consumer to monitor the intent fulfilment information. The intent report also can contain the current and optional predicted value for performance indicated by corresponding expectation targets (e.g. WeakRSRPRatio for the weakRSRPRatioTarget, Average UL RAN UE Throughput for aveULRANUEThptTarget), which can be used by MnS consumer to validate whether the intent is really fulfilled and to evaluate whether the intent (especially for expectation targets) needs to be updated if needed (improve the target value when corresponding target is fulfilled or reduce the target value when corresponding target is not fulfilled or not fulfilled with the reason of target confliction). Besides, intent conflict information and intent fulfilment feasibility check information which send by MnS producer to MnS consumer is another type of intent report information. So, following are the four types of information needs to be monitored by MnS consumer:

-	Intent Fulfilment information, which represents the properties of a specific fulfilment information for an aspect of the intent (i.e. either an expectation, a target or the whole intent), including fulfilment status and achieved values for targets.

-	Intent conflict information, which represents other intent with conflict occruing,  conflict type (i.e., intent conflict, expectation conflict and target conflict) and possible solution recommendation to address the conflicts with solution specific information(i.e. reject information, alternative target, recommened cotenxt).

-	Intent conflict resolution information, which represents result of conflict solution(i.e. successful, failed)  , report id associated with internt report include intent conflict information and solution specific information(i.e. intent object information for preemption).

-	Intent fulfilment feasibility check information, which indicates that the intent is feasible or infeasible. Intent fulfilment feasibility check information is provided after MnS producer automatically performs feasibility check when receive the intent creation and modification request from MnS consumer.

Different MnS consumer may have different requirements for intent report (e.g. Some MnS consumer may want to have corresponding performance value information while others do not want. Different MnS consumer may want to calculate or monitor the performance value in different period).

End of changes

Second change

6.3	Procedures for intent management

6.3.x	Resolve intent conflict

1.	Refert to 6.3.2

2.	Refert to 6.3.2

3.	Refert to 6.3.2

4.	Mns Producer detects intet conflict

5.	Mns Producer sends intent report with conflict inromatin which has other intent with conflict occruing

6.	Mns Producer sends intent report with conflict inromatin which has other intent with conflict occruing, conflict type, possible solution and solution specific information(i.e. reject information, alternative target, recommened cotenxt).

7.	Mns Producer resolve intent conflict with proposed solution.

NOTE:	The detail process of resolving intent confilict is FFS.

8.	Mns Producer sends ntent report with conflict resolution information which has result of conflict solution, report id associated with internt report include intent conflict information and solution specific information(i.e. intent object information for preemption).

9	Mns Producer sends ntent report with conflict resolution information which has result of conflict solution, report id associated with internt report include intent conflict information and solution specific information(i.e. intent object information for preemption).

10.	MnS Consumer may proceed 6.3.3	Modify an intent or 6.3.4 Delete an intent according to intent report with conflict resolution information

End of changes

Third change

A.1.x	Resolve intent conflict

@startuml

title "[resolve intent conflict]"

actor "MnS Consumer1" as MnS_Consumer1

actor "MnS Consumer2" as MnS_Consumer2

participant "MnS Producer" as MnS_Producer

Ref over  MnS_Producer , MnS_Consumer2

6.3.2	Create an intent

end

MnS_Consumer1 -> MnS_Producer: 1. Request to create an intent instance (list of attributes of intent IOC)

MnS_Producer -> MnS_Producer: 2. Create and configure intent MOI

MnS_Producer -> MnS_Consumer1: 3. Response for create an intent instance

MnS_Producer -> MnS_Producer: 4. Detect intent conflict

MnS_Producer -> MnS_Consumer1: 5. Intent Report with conflict inforamtion

MnS_Producer -> MnS_Consumer2: 6. Intent Report with conflict inforamtion

MnS_Producer -> MnS_Producer: 7. resolving intent conflict

MnS_Producer -> MnS_Consumer1: 8. Intent Report with conflict resolution information

MnS_Producer -> MnS_Consumer2: 9. Intent Report with conflict resolution information

alt need to modify

Ref over  MnS_Producer , MnS_Consumer1, MnS_Consumer2

10. 6.3.3	Modify an intent

end

else need to delete

Ref over  MnS_Producer , MnS_Consumer1, MnS_Consumer2

10. 6.3.4	Delete an intent

end

end

hide footbox

@enduml

End of changes