3GPP TSG-SA5 Meeting #150	S5-235418

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.158 | CR | 0116 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 32.158 Correct URI definition inconsistencies |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The use of ‘MnSName’ in the 32.158 URI definition is ambiguous with respect to the attributes actually defined in the NRM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace ‘MnSName’ with ‘MnSLabel’ to clarify which attribute the value maps to clarify behaviour when optional parts of URI are omitted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion could arise as to which attribute is supposed to be used to form the correct URI. Inconsistent behaviour could affect multi-vendor interop. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4, A.1, A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 4.4	URI structure

### 4.4.1	Introduction

MnS producers can be divided into two categories. The first category exposes MnS(s) to manipulate resources representing managed object instances. In this case the URI structure is governed by the mapping rules defined in clause 4.2.3. The second category exposes MnS(s) to manipulate resources not representing managed object instances. In this case the DN concept is not relevant. The URI structure for both categories is different.

### 4.4.2	URI structure for resources representing managed object instances

URIs identifying resources representing managed object instances shall follow, when being used as a target URI in HTTP requests, the structure given by

{scheme}://{URI-DN-prefix}/{root}/{MnSLabel}/{MnSVersion}/{URI-LDN}

with:

{scheme}		Scheme component "http" or "https"

{URI-DN-prefix}		Authority component (host identifier and optional TCP port), the host name is constructed from the DN prefix as defined in clause 4.2.3.

{root}		Part of the path component, allows specifying one or more optional path segments for structuring the resource hierarchy on a HTTP server. The DN or parts thereof shall not be mapped to this path component.

{MnSLabel}		Part of the path component, allows specifying an optional MnS name in a single path segment.

{MnSVersion}		Part of the path component, allows specifying an optional MnS version in a single path segment.

{URI-LDN}		Part of the path component, constructed from the LDN as defined in clause 4.2.3, containing zero, one or more path segments.

As seen above, to construct the URI from a DN, it is necessary to map the "DNPrefixPlusRDNSeparator" as defined in clause 7.3 of [3], the “LocalDN” as defined in clause 7.3 of [3], and to add the additional optional path segments  "/{root}/{MnSLabel}/{MnSVersion}".

To allow for a predictive mapping from an URI to the original DN it is necessary to specify "/{MnSLabel}/{MnSVersion}" in such a way that the beginning of the "LocalDN" can be unambigously identified.

Note it may be required when specifying a MnS to clearly identify the last RDN of "{URI-LDN}" and to use the following instead of "{URI-LDN}"

{URI-LDN-first-part}/{RDN}

or

{URI-LDN-first-part}/{className}={id}.

For the sake of brevity, "MnSRoot" is introduced that includes the "{scheme}" part, the colon (":"), the two slash characters ("//"), the "{authority}" part, a single slash character ("/") and the "{root}" part.

{MnSRoot} := {scheme}://{URI-DN-prefix}/{root}

When using "{MnSRoot}" the abbreviated URI structure is given by

{MnSRoot}/{MnSLabel}/{MnSVersion}/{URI-LDN}

or

{MnSRoot}/{MnSLabel}/{MnSVersion}/{URI-LDN-first-part}/{className}={id}

It is recommended to use this abbreviated form of the URI structure when defining Management Services.

The path segment "MnSVersion" allows access to resources with different MnS versions, for example:

http://operatorA.com/ProvMnS/v1500/SubNetwork=south/.../Cell=1

http://operatorA.com/ProvMnS/v1600/SubNetwork=south/.../Cell=1

Note that both URIs, though different as to the path segment indicating the version number of the ProvMnS, identify the same resource that is identified by the canonical URI:

http://operatorA.com/SubNetwork=south/.../Cell=1

and whose DN is:

DC=operatorA.com,SubNetwork=south,...,Cell=1

The optional path component "/{root}" may be used to separate the name space for 3GPP management from the name space for other domains:

http://operatorA.com/3gppManagement/ProvMnS/v1600/SubNetwork=south/.../Cell=1

or to provide dedicated URIs on the same host for different tasks:

http://operatorA.com/3gppManagement/cm/ProvMnS/v1600/SubNetwork=south/.../Cell=1

http://operatorA.com/3gppManagement/fm/ProvMnS/v1600/SubNetwork=south/.../Cell=1

Note that when different hosts are used for different management tasks, like in

http://cm.operatorA.com/3gppManagement/ProvMnS/v1600/SubNetwork=south/.../Cell=1

http://fm.operatorA.com/3gppManagement/ProvMnS/v1600/SubNetwork=south/.../Cell=1

then also the resources are different and identifierd by the canonical URIs

http://cm.operatorA.com/SubNetwork=south/.../Cell=1

http://fm.operatorA.com/SubNetwork=south/.../Cell=1

or the DNs

DC=cm.operatorA.com,SubNetwork=south,...,Cell=1

DC=fm.operatorA.com,SubNetwork=south,...,Cell=1

In the example above, it is assumed that both resources represent the same cell in the network. This information cannot be derived from the DN or canonical URI, though.

### 4.4.3	URI structure for resources not representing managed object instances

URIs identifying other resources shall follow, when being used as a target URI in HTTP requests, the structure given by

{scheme}://{authority}/{root}/{MnSLabel}/{MnSVersion}/{MnSResourcePath}

with:

{scheme}		Scheme component "http" or "https"

{authority}	Authority component (host identifier and optional TCP port)

{root}		Part of the path component, allows specifying optional path segments for structuring the resource hierarchy on a HTTP server.

{MnSLabel}		Part of the path component, specifies the mandatory MnS name in a single path segment.

{MnSVersion}		Part of the path component, specifies the optional MnS version in a single path segment.

{MnSResourcePath}		Part of the path component, one or more path segments, specifies a resource of the MnS

For the sake of brevity, {MnSRoot} is introduced that includes the "{scheme}" part, the two slash characters ("//"), the "{authority}" part, a single slash character ("/") and the "{root}" part. When using "{MnSRoot}" the abbreviated URI structure is given by

{MnSRoot}/{MnSLabel}/{MnSVersion}/{MnSResourcePath}

It is recommended to use this abbreviated form of the URI structure when defining Management Services.

When MnSVersion is absent the latest MnS version is assumed.

### 4.4.4	Resource "../{MnSLabel}/{MnSVersion}"

The resource identified by "../{MnSLabel}/{MnSVersion}" is called NRM root. It represents the conceptual parent of the top-level managed object instances. It is created by the MnS Producer. A MnS Consumer cannot create or delete this resource.

The resource is the target resource for many HTTP requests, such as requests to retrieve all top-level managed object instances in case there are multiple top-level managed object instances, or for requests to create objects in case there are no manged object instances yet and the creation request needs to be directed to the parent of the resource to be created.

Attempts to read the NRM root only shall return "204 No Content".

2nd Change

Annex A (informative):
Examples

# A.1	Example data model

The following JSON instance document is used for the examples in this clause.

{ "SubNetwork": [ { "id": "SN1", "objectClass": "SubNetwork", "objectInstance": "DC=example.org,SubNetwork=SN1", "attributes": { "userLabel": "Berlin NW", "userDefinedNetworkType": "5G", "plmnId": { "mcc": 456, "mnc": 789 } }, "ManagedElement": [ { "id": "ME1", "objectClass": "ManagedElement", "objectInstance": "DC=example.org,SubNetwork=SN1,\ ManagedElement=ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" }, "XyzFunction": [ { "id": "XYZF1", "objectClass": "XyzFunction", "objectInstance":"DC=example.org,SubNetwork=SN1,\ ManagedElement=ME1,XyzFunction=XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } }, { "id": "XYZF2", "objectClass": "XyzFunction", "objectInstance":"DC=example.org,SubNetwork=SN1,\ ManagedElement=ME1,XyzFunction=XYZF2", "attributes": { "attrA": "abc", "attrB": 552 } } ] }, { "id": "ME2", "objectClass": "ManagedElement", "objectInstance": "DC=example.org,SubNetwork=SN1,ManagedElement=ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } } ], "PerfMetricJob": [ { "id": "PMJ1", "objectClass": "PerfMetricJob", "objectInstance": "DC=example.org,SubNetwork=SN1,PerfMetricJob=PMJ1", "attributes": { "granularityPeriod": "5", "perfMetrics": [ "Metric1", "Metric2" ], "objectInstances": [ "Obj1", "Obj2" ] } } ], "ThresholdMonitor": [ { "id": "TM1", "objectClass": "ThresholdMonitor", "objectInstance": "DC=example.org,SubNetwork=SN1,ThresholdMonitor=TM1", "attributes": { "metric": "Metric1", "thresholdLevels": [ { "level": "1", "thresholdValue": 10 }, { "level": "2", "thresholdValue": 20 }, { "level": "3", "thresholdValue": 30 } ] } } ] } ] }

The corresponding JSON schema is

{ "SubNetwork": { "type": "array", "items": { "type": "object", "properties": { "id": { "type": "string" }, "objectClass": { "type": "string" }, "objectInstance": { "type": "string" }, "attributes": { "type": "object", "properties": { "userLabel": { "type": "string" }, "userDefinedNetworkType": { "type": "string" }, "plmnId": { "type": "object", "properties": { "mcc": { "type": "integer" }, "mnc": { "type": "integer" } } } } }, "ManagedElement": { "type": "array", "items": { "type": "object", "properties": { "id": { "type": "string" }, "objectClass": { "type": "string" }, "objectInstance": { "type": "string" }, "attributes": { "type": "object", "properties": { "userLabel": { "type": "string" }, "vendorName": { "type": "string" }, "location": { "type": "string" } } }, "XyzFunction": { "type": "array", "items": { "type": "object", "properties": { "id": { "type": "string" }, "objectClass": { "type": "string" }, "objectInstance": { "type": "string" }, "attributes": { "type": "object", "properties": { "attributeA": { "type": "string" }, "attributeB": { "type": "integer" } } }, "required": ["id"] } } }, "required": ["id"] } } }, "PerfMetricJob": { "type": "array", "items": { "type": "object", "properties": { "id": { "type": "string" }, "objectClass": { "type": "string" }, "objectInstance": { "type": "string" }, "attributes": { "type": "object", "properties": { "granularityPeriod": { "type": "integerstring" }, "perfMetrics": { "type": "array", "items": { "type": "string" } } }, "objectInstances": { "type": "array", "items": { "type": "string" } } }, "required": ["id"] } } }, "ThresholdMonitor": { "type": "array", "items": { "type": "object", "properties": { "id": { "type": "string" }, "objectClass": { "type": "string" }, "objectInstance": { "type": "string" }, "attributes": { "type": "object", "properties": { "thresholdLevels": { "type": "array", "items": { "type": "object", "properties": { "level": { "type": "string" }, "thresholdValue": { "type": "integer" } } } } } }, "required": ["id"] } } }, "required": ["id"] } } } }

The corresponding XML instance document is provided below as well. It can be helpful when evaluating XPath expressions.

<?xml version="1.0" encoding="UTF-8" ?> <nrmRoot> <SubNetwork> <id>SN1</id> <objectClass>SubNetwork</objectClass> <objectInstance>DC=example.org,SubNetwork=SN1</objectInstance> <attributes> <userLabel>Berlin NW</userLabel> <userDefinedNetworkType>5G</userDefinedNetworkType> <plmnId> <mcc>456</mcc> <mnc>789</mnc> </plmnId> </attributes> <ManagedElement> <id>ME1</id> <objectClass>ManagedElement</objectClass> <objectInstance>DC=example.org,SubNetwork=SN1,ManagedElement=ME1</objectInstance> <attributes> <userLabel>Berlin NW 1</userLabel> <vendorName>Company XY</vendorName> <location>TV Tower</location> </attributes> <XyzFunction> <id>XYZF1</id> <objectClass>XyzFunction</objectClass> <objectInstance> DC=example.org,SubNetwork=SN1,\ ManagedElement=ME1,XyzFunction=XYZF1</objectInstance> <attributes> <attrA>xyz</attrA> <attrB>551</attrB> </attributes> </XyzFunction> <XyzFunction> <id>XYZF2</id> <objectClass>XyzFunction</objectClass> <objectInstance> DC=example.org,SubNetwork=SN1,\ ManagedElement=ME1,XyzFunction=XYZF2</objectInstance> <attributes> <attrA>abc</attrA> <attrB>552</attrB> </attributes> </XyzFunction> </ManagedElement> <ManagedElement> <id>ME2</id> <objectClass>ManagedElement</objectClass> <objectInstance>SubNetwork=SN1,ManagedElement=ME2</objectInstance> <attributes> <userLabel>Berlin NW 2</userLabel> <vendorName>Company XY</vendorName> <location>Grunewald</location> </attributes> </ManagedElement> <PerfMetricJob> <id>PMJ1</id> <objectClass>PerfMetricJob</objectClass> <objectInstance>SubNetwork=SN1,PerfMetricJob=PMJ1</objectInstance> <attributes> <granularityPeriod>5</granularityPeriod> <perfMetrics>Metric1</perfMetrics> <perfMetrics>Metric2</perfMetrics> <objectInstances>Obj1</objectInstances> <objectInstances>Obj2</objectInstances> </attributes> </PerfMetricJob> <ThresholdMonitor> <id>TM1</id> <objectClass>ThresholdMonitor</objectClass> <objectInstance>SubNetwork=SN1,ThresholdMonitor=TM1</objectInstance> <attributes> <ThresholdLevels> <level>1</level> <thresholdValue>10</thresholdValue> </ThresholdLevels> <ThresholdLevels> <level>2</level> <thresholdValue>20</thresholdValue> </ThresholdLevels> <ThresholdLevels> <level>3</level> <thresholdValue>30</thresholdValue> </ThresholdLevels> </attributes> </ThresholdMonitor> </SubNetwork> </nrmRoot>

NOTE:	Void

The following examples do not always follow the URI structure specified in clause 4.4. For simplicity reasons, the path component "/{MnSLabel}/{MnSVersion}" is often omitted.

3rd Change

# A.2	Retrieval of resources

## A.2.1	Retrieval of a single complete resource with HTTP GET

To retrieve a complete "XyzFunction" resource the MnS Consumer might send the following request.

GET /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Accept: application/json

The response includes a JSON object with the resource representation.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } }

The MnS Consumer might request also to return a response constructed according to the flat response construction method. In this case the "Accept" header contains the " application/vnd.3gpp.object-tree-flat+json" media type.

GET /SubNetwork=SN1/ManagedElement=ME1/XyzFunction=XYZF1 HTTP/1.1 Host: example.org Accept: application/vnd.3gpp.object-tree-flat+json

The response is a JSON array with a single item, which is a JSON object with the resource representation. Note that the resource representation contains the "objectClass" and "objectInstance" in this case.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/ vnd.3gpp.object-tree-flat+json [ { "id": "XYZF1", "objectClass": "XyzFunction", "objectInstance": "DC=example.org,SubNetwork=SN1,ManagedElement=ME1,XyzFunction=XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } } ]

## A.2.2	Attribute and attribute field selection on a single resource

To retrieve only the "userLabel" attribute and the "mnc" attribute field of the "plmnId" attribute  of the "SubNetwork", the MnS Consumer might send:

GET /SubNetwork=SN1?attributes=userLabel&fields=/attributes/plmnId/mcc HTTP/1.1 Host: example.org Accept: application/json

Alternatively one might send as well

GET /SubNetwork=SN1?fields=/attributes/userLabel,/attributes/plmnId/mcc HTTP/1.1 Host: example.org Accept: application/json

The response contains only the selected attribute "userLabel" and the selected attribute field "mnc":

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "SN1", "attributes": { "userLabel": "Berlin NW", "plmnId": { "mnc": 789 } } }

In this example, the MnS Consumer retrieves the "userLabel" and "vendorName" of the "ManagedElement" whose "id" is equal to "ME1":

GET /SubNetwork=SN1/ManagedElement=ME1?attributes=userLabel,vendorName HTTP/1.1 Host: example.org Accept: application/json

The MnS Producer responds as follows:

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY" } }

The following request selects all attributes:

GET /SubNetwork=SN1/ManagedElement=ME1?fields=/attributes HTTP/1.1 Host: example.org Accept: application/json

It is thus identical to:

GET /SubNetwork=SN1/ManagedElement=ME1 HTTP/1.1 Host: example.org Accept: application/json

Both requests return the complete resource representation with all attributes:

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" } }

The following request returns the first item of the "perfMetrics" attribute, which is of type array:

GET /SubNetwork=SN1/ManagedElement=ME1/PerfMetricJob=PMJ1?fields=attributes/perfMetrics/0 HTTP/1.1 Host: example.org Accept: application/json

Note indices start with "0" in JSON Pointer. The response looks like:

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "PMJ1", "attributes": { "perfMetrics": [ "Metric1" ] } }

## A.2.3	Retrieval of multiple complete resources using scoping and filtering

The following example selects the "SubNetwork" as base object at scope level "0" and all objects at scope level "1":

GET /SubNetwork=SN1?scopeType=BASE_SUBTREE&scopeLevel=1 HTTP/1.1 Host: example.org Accept: application/json

The base object and all objects at scope level "1", irrespective of their object class, are included in the response. The acceptable response media type specified by the "Accept" header field is "application/json", which indicates to the MnS producer to use the hierarchical response construction method

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "SN1", "attributes": { "userLabel": "Berlin NW", "userDefinedNetworkType": "5G", "plmnId": { "mcc": 456, "mnc": 789 } }, "ManagedElement": [ { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" } }, { "id": "ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } } ], "PerfMetricJob": [ { "id": "PMJ1", "attributes": { "granularityPeriod": 5, "perfMetrics": [ "Metric1", "Metric2" ], "objectInstances": [ "Obj1", "Obj2" ] } } ], "ThresholdMonitor": [ { "id": "TM1", "attributes": { "metric": "Metric1", "thresholdLevels": [ { "level": "1", "thresholdValue": 10 }, { "level": "2", "thresholdValue": 20 }, { "level": "3", "thresholdValue": 30 } ] } } ] }

The MnS Consumer can request also to return a response constructed according to the flat response construction method. In this case the "Accept" header contains the "application/vnd.3gpp.object-tree-flat+json" media type.

GET /SubNetwork=SN1?scopeType=BASE_SUBTREE&scopeLevel=1 HTTP/1.1 Host: example.org Accept: application/vnd.3gpp.object-tree-flat+json

The response looks like:

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/vnd.3gpp.object-tree-flat+json [ { "id": "SN1", "objectClass": "SubNetwork", "objectInstance": "DC=example.org,SubNetwork=SN1", "attributes": { "userLabel": "Berlin NW", "userDefinedNetworkType": "5G", "plmnId": { "mcc": 456, "mnc": 789 } } }, { "id": "ME1", "objectClass": "ManagedElement", "objectInstance": "SubNetwork=SN1,ManagedElement=ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" } }, { "id": "ME2", "objectClass": "ManagedElement", "objectInstance": "DC=example.org,SubNetwork=SN1,ManagedElement=ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } }, { "id": "PMJ1", "objectClass": "PerfMetricJob", "objectInstance": "DC=example.org,SubNetwork=SN1,PerfMetricJob=PMJ1", "attributes": { "granularityPeriod": "5", "perfMetrics": [ "Metric1", "Metric2" ], "objectInstances": [ "Obj1", "Obj2" ] } }, { "id": "TM1", "objectClass": "ThresholdMonitor", "objectInstance": "DC=example.org,SubNetwork=SN1,ThresholdMonitor=TM1", "attributes": { "metric": "Metric1", "thresholdLevels": [ { "level": "1", "thresholdValue": 10 }, { "level": "2", "thresholdValue": 20 }, { "level": "3", "thresholdValue": 30 } ] } } ]

The "objectInstance" of each returned object is present in the response, as required in clause 6.1.4.

When only objects at scope level "1" are requested to be returned, the request looks like:

GET /SubNetwork=SN1?scopeType=BASE_NTH_LEVEL&scopeLevel=1 HTTP/1.1 Host: example.org Accept: application/json

The response does not include the attributes of "SubNetwork" any more, only its "id" is included:

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "SN1", "ManagedElement": [ { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" } }, { "id": "ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } } ], "PerfMetricJob": [ { "id": "PMJ1", "attributes": { "granularityPeriod": 5, "perfMetrics": [ "Metric1", "Metric2" ], "objectInstances": [ "Obj1", "Obj2" ] } } ], "ThresholdMonitor": [ { "id": "TM1", "attributes": { "metric": "Metric1", "thresholdLevels": [ { "level": "1", "thresholdValue": 10 }, { "level": "2", "thresholdValue": 20 }, { "level": "3", "thresholdValue": 30 } ] } } ] }

Similarly, for reading all objects on scope level "2", the MnS Consumer may send:

GET /SubNetwork=SN1?scopeType=BASE_NTH_LEVEL&scopeLevel=2 HTTP/1.1 Host: example.org Accept: application/json

When using the hierarchical response construction method, the response includes the complete representations of the two "XyzFunction" objects. The "SubNetwork" and "ManagedElement" are present with their "id" only; they provide the containment nodes for the "XyzFunction" objects.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "SN1", "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } }, { "id": "XYZF2", "attributes": { "attrA": "abc", "attrB": 552 } } ] } ] }

The "PerfMetricJob" and "ThresholdMonitor" are not included altogether, not even with the "id" only. This is because these nodes do not represent necessary path components to the scoped objects on the second level.

When using the flat response construction method, the response includes only the two "XyzFunction" objects without containment nodes.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json [ { "id": "XYZF1", "objectClass": "XyzFunction", "objectInstance": "DC=example.org,SubNetwork=SN1,ManagedElement=ME1,XyzFunction=XYZF1", "attributes": { "attrA": "xyz", "attrB": 551 } }, { "id": "XYZF2", "objectClass": "XyzFunction", "objectInstance": "DC=example.org,SubNetwork=SN1,ManagedElement=ME1,XyzFunction=XYZF2", "attributes": { "attrA": "abc", "attrB": 552 } } ]

The following example selects all objects of any class on scope level "1" that have a "location" attribute whose value is equal to "Grunewald":

GET /SubNetwork=SN1?\ scopeType=BASE_NTH_LEVEL&scopeLevel=1&\ filter=/*/*[attributes[location="Grunewald"]] HTTP/1.1 Host: example.org Accept: application/json

The response includes one "ManagedElement" object only:

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "SN1", "ManagedElement": [ { "id": "ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } } ] }

The input document to the XPath expression is a document whose root node is the object identified by the path component of the target URI and that includes the object representations of the scoped objects. In this example the root node is the "SubNetwork", but it is not scoped and hence included in the input document with its "id" only, i.e. without the "attributes" node. The input document includes furthermore all scoped objects on level "1" with their complete representations (without name-contained objects). These are the two "ManagedElement" objects, the "PerfMetricJob" object, and the "ThresholdMonitor" object.

{ "id": "SN1", "ManagedElement": [ { "id": "ME1", "attributes": { "userLabel": "Berlin NW 1", "vendorName": "Company XY", "location": "TV Tower" } }, { "id": "ME2", "attributes": { "userLabel": "Berlin NW 2", "vendorName": "Company XY", "location": "Grunewald" } } ], "PerfMetricJob": [ { "id": "PMJ1", "attributes": { "granularityPeriod": 5, "perfMetrics": [ "Metric1", "Metric2" ], "objectInstances": [ "Obj1", "Obj2" ] } } ], "ThresholdMonitor": [ { "id": "TM1", "attributes": { "metric": "Metric1", "thresholdLevels": [ { "level": "1", "thresholdValue": 10 }, { "level": "2", "thresholdValue": 20 }, { "level": "3", "thresholdValue": 30 } ] } } ] }

An implementation may be based on available XPath tools. In that case the JSON document may have to be converted to a XML document Note that a valid XML document has one and only one root element. For that reason the "SubNetwork" element needs to be added as root element..

<SubNetwork> <id>SN1</id> <ManagedElement> <id>ME1</id> <attributes> <userLabel>Berlin NW 1</userLabel> <vendorName>Company XY</vendorName> <location>TV Tower</location> </attributes> </ManagedElement> <ManagedElement> <id>ME2</id> <attributes> <userLabel>Berlin NW 2</userLabel> <vendorName>Company XY</vendorName> <location>Grunewald</location> </attributes> </ManagedElement> <PerfMetricJob> <id>PMJ1</id> <attributes> <granularityPeriod>5</granularityPeriod> <perfMetrics>Metric1</perfMetrics> <perfMetrics>Metric2</perfMetrics> <objectInstances>Obj1</objectInstances> <objectInstances>Obj2</objectInstances> </attributes> </PerfMetricJob> <ThresholdMonitor> <id>TM1</id> <attributes> <ThresholdLevels> <level>1</level> <thresholdValue>10</thresholdValue> </ThresholdLevels> <ThresholdLevels> <level>2</level> <thresholdValue>20</thresholdValue> </ThresholdLevels> <ThresholdLevels> <level>3</level> <thresholdValue>30</thresholdValue> </ThresholdLevels> </attributes> </ThresholdMonitor> </SubNetwork>

In this example the complete "ManagedElement" object is the result of applying the XPath expression:

<ManagedElement> <id>ME2</id> <attributes> <userLabel>Berlin NW 2</userLabel> <vendorName>Company XY</vendorName> <location>Grunewald</location> </attributes> </ManagedElement>

XPath predicates allow to specify also ranges. The following example selects objects on scope level "2" that have an attribute with name "attrB" whose value is equal to or greater than 552 and less than 562.

GET /SubNetwork=SN1?\ scopeType=BASE_NTH_LEVEL&scopeLevel=2&\ filter=/*/*/*[attributes[attrB>=552 and attrB<562]] HTTP/1.1 Host: example.org Accept: application/json

The response includes one "XyzFunction" object only:

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "SN1", "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF2", "attributes": { "attrA": "abc", "attrB": 552 } } ] } ] }

An identical response is returned when using the following requests:

GET /SubNetwork=SN1?\ scopeType=BASE_ALL&\ filter=//*[attributes[attrB>=552 and attrB<562]] HTTP/1.1 Host: example.org Accept: application/json

or

GET /SubNetwork=SN1?\ scopeType=BASE_SUBTREE&scopeLevel=2&\ filter=//*[attributes[attrB>=552 and attrB<562]] HTTP/1.1 Host: example.org Accept: application/json

or

GET /SubNetwork=SN1?\ scopeType=BASE_ALL&\ filter=//XyzFunction[attributes[attrB>=552 and attrB<562]] HTTP/1.1 Host: example.org Accept: application/json

It is possible to combine scoping and filtering with attribute and attribute field selection. The following example returns the containment tree, starting with the "SubNetwork" identified by the target URI.

GET /SubNetwork=SN1?scopeType=BASE_ALL&attributes= HTTP/1.1 Host: example.org Accept: application/json

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "SN1", "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF1" }, { "id": "XYZF2" } ] }, { "id": "ME2" } ], "PerfMetricJob": [ { "id": "PMJ1" } ], "ThresholdMonitor": [ { "id": "TM1" } ] }

The next example scopes the same subtree as in the previous example and requests to return only "vendorName" attributes instead of no attributes at all.

GET /ProvMnS/v1700?\ scopeType=BASE_ALL&\ attributes=vendorName HTTP/1.1 Host: example.org Accept: application/json

This results, according to clause 6.2.3, in removing from the response all scoped resources that do not have a "vendorName" attribute.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "id": "SN1", "ManagedElement": [ { "id": "ME1", "attributes": { "vendorName": "Company XY" } }, { "id": "ME2", "attributes": { "vendorName": "Company XY" } } ] }

If the retrieval request identifies resources that do not exist, such as in

GET /ProvMnS/v1700?scopeType=BASE_NTH_LEVEL&scopeLevel=3 HTTP/1.1 Host: example.org Accept: application/json

then the MnS producer returns "404 Not Found" in the response status line. More error information may be provided in the response body.

HTTP/1.1 204 Not Found Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "error": { "errorInfo": "No resources at scope level specified in the request." } }

When the MnS Consumer does not know the root objects of the containment tree and wants to retrieve the complete trees starting with the roots, the target URI needs to identify the NRM root, i.e. the resource above the root objects. According to clause 4.4.2, this resource is identified by the path segment "/{MnSLabel}/{MnSVersion}", for example "/ProvMnS/v1700". In the following example, the "attributes" query parameter is empty and only the name-containment hierarchy (without attributes) is returned.

GET /ProvMnS/v1700?scopeType=BASE_ALL&attributes= HTTP/1.1 Host: example.org Accept: application/json

The response is illustrated below.

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "SubNetwork": [ { "id": "SN1", "ManagedElement": [ { "id": "ME1", "XyzFunction": [ { "id": "XYZF1" }, { "id": "XYZF2" } ] }, { "id": "ME2" } ], "PerfMetricJob": [ { "id": "PMJ1" } ], "ThresholdMonitor": [ { "id": "TM1" } ] } ] }

Multiple root resources can be returned as well. For example, assume a NRM with three "SubNetwork" root resources, then the response may look like:

HTTP/1.1 200 OK Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json { "SubNetwork": [ { "id": "SN1", ... }, { "id": "SN2", ... }, { "id": "SN3", ... } ] }

Note that when the target URI identifies the NRM root, then the name of the document (root) element, to which an XPath expression is applied, is "nrmRoot". The first step of the location path of an XPath expression is hence "/nrmRoot". For example, the following HTTP GET request returns the "SubNetwork" with the identifier "SN1".

GET /ProvMnS/v1700?\ scopeType=BASE_ALL&\ filter=/nrmRoot/SubNetwork[id="SN1"]/attributes HTTP/1.1 Host: example.org Accept: application/json

Note the presence of the location step "/attributes". This step is necessary to select only the "attributes" container and hence only the SubNetwork" with the identifier "SN1" without any name-contained objercts.

{ "SubNetwork": [ { "id": "SN1", "objectClass": "SubNetwork", "objectInstance": "DC=example.org,SubNetwork=SN1", "attributes": { "userLabel": "Berlin NW", "userDefinedNetworkType": "5G", "plmnId": { "mcc": 456, "mnc": 789 } } } ] }

Without the location step "/attributes" the complete subtree would be returned.

End of Changes