3GPP TSG-SA5 Meeting #150	S5-235512

# Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.533 | CR | draftCR | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | InputToDraftCR Rel-18 28.533 on conditions for MOIs related to Access control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Telefonica |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MSAC |  |  |  |  |  | Date: |  |  | 2023-05-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Stage 2 contributions related to - Definition of IOCs related to conditions needed according to the agreed usecases for identifying the instances of Managed objects in the scope of access control |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of Stage 2 related to conditions for MOIs related to identifying the scope in access control |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The design and subsequent implementation for access control in management systems for role-based access control will not progress |  |  |  |  |  |  |  |  |
| Clauses affected: |  | X.x, Y.x, Annex Z.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This is input to latest approved DraftCR S5-234845 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[xx]	IETF RFC 6749: " The OAuth 2.0 Authorization Framework".

[yy]	IETF RFC 8341: " Network Configuration Access Control Model".

## 4.9.x	Identity and Access Control

Network Management systems are becoming a challenge to manage in terms of the users of the system as well as the access control that needs to be applied continuously on a need-to-know basis.

Flexibility and agility to adapt to these growing needs is the key for a sustainable identity and access control implementation.

The service based architecture needs to factor seamless integration to any system for authentication and authorization of Management Service (MnS) consumers.

The need to make continuous access control related changes should be supported with ease and simplicity.

Today we have various human users, machine users and various resources that are continuously growing with complexities below:

- Human users seem to require various levels of access control.

- Machine type of communication in the direction towards automated systems require another type of access and may not necessarily be user name password based.

- Resources and modules themselves require various level of clearance depending on the sensitivity of the data being accessed.

The identity and access control system should be designed for role based access control. Also, the authorization rules may support the fine grained permissions i.e. based on MnS component A, B and C.

### 4.9.x.1    Usecases

#### 4.9.x.1.1 	Role based access control

This is based upon the concept of assigning the appropriate permissions and privileges to authorized users. The combinations of the permissions and privileges make up a role. The users who belong to a role should be assigned to resources based on a least privilege principle and access rules associated to the resource. The principle of least privileges states that the users should be granted access only to the data and the operations that are required to perform the job. This minimizes the possibility of a security breach.

The management system should be setup for access control by a system administrator who will have the know-how of creating the required users and roles. Roles will use the various access rules. Additionally, the system administrator will also need to setup the access rules for a MnS producer.

Roles could be in various categories like full access and restricted access. The access rules will be defined on a MnS producer which could relate to all or combinations of component A, component B and component C.

Figure 4.9.x.1.1-y - role based access

Figure 4.9.x.1.1-y - role based access

The Figure 4.9.x.1.1-y - role based access shows how a user is assigned to a role. The roles in turn are able to act upon a resource with the required operation on a MnS Producer. The actions that can be performed are defined by the access rules.

#### 4.9.x.1.2	 Authentication and authorization

- To carry out authentication and authorization based on role-based access the following tasks needs to be in place.

- Pre-deployment task – This relates to the identification of the resources represented by the MnS component B and C and it associated operations represented by MnS component A. This is typically done by a Network Equipment Provider (NEP) during the design phase.

- Post deployment task – this relates to the set of administrative tasks which are requires to enable role-based access control typically caried out by a network operator (NOP). This is done once the system is up and running and access control needs to be administered.

NOTE:     The NOP may have a system administration department in charge of defining and configuring role-based access control.

Post the above tasks we have the possibility to have role-based access in operation with every service call being authenticated and authorized.

When an MnS is invoked, the MnS consumer authenticates towards an authentication service producer. This is then followed by checking the access rights with respect to the role-based access. This takes place in the authorization service producer which is invoked from the MnS producer to check if its resources can be accessed with respect to the related operation.

#### 4.9.x.1.3	 Identity to roles association

The identity or user of the system needs to be assigned the permissions to carry out the management operations. A set of permissions is defined as a role. An efficient and flexible means is to define various roles by a network operator. In a second step, association of the roles to an identity (entity or human user) is carried out.

#### 4.9.x.1.4	 Roles to associated to resources (consisting of IOCs(static),MOIs(dynamic) and corresponding operations

Role-based access control defines roles. Roles are associated to access rules which are combinations of resources and operations (e.g., CRUD operations). The resources can be represented by IOCs(static) and/or MOIs (dynamic). Hence the resources in context are Component B and Component C and the operations are Component A.

### 4.9.x.2    Requirements

- UC-MSAC-0x - 3GPPP management system shall support role-based access control for the resources represented by the MnS component A, B and C.

- UC-MSAC-0x.01- 3GPP management system shall support authentication and authorisation for management services.

- UC-MSAC-0x.02 - 3GPP management system shall support identity to role assignment.

- UC-MSAC-0x.03 - 3GPP management system shall support roles which are associated to resources (IOCs(static) and/or MOIs(dynamic)) and the corresponding operations.

## X.2	Class diagrams

### X.2.1	Relationships

This clause depicts the set of classes (e.g. IOCs) that encapsulates the information relevant for the concept of the role based access control. This clause provides an overview of the relationships between relevant classes in UML. Subsequent clauses provide more detailed specification of various aspects of these classes.

In order to realise the use cases for authentication and authorization, the need is to model the required IOCs and the relevant operations which utilise the IOCs. The operations supported on the IOCs are those supported by the generic management services.

As mentioned in the usecases the IOCs are provisioned during the integration time by a network operator. The data is sent to an authentication and authorization service producer.

Post this during integration time when a MnS consumer invokes an operation on the MnS producer, the authentication and authorization service producer validates the action on the resource to enable the decision for the MnS producer. The decision could be that the MnS producer allows or disallows the action on the resource.

Below are the IOCs that need to interact in order to store the data required for the authentication and authorization operations with respect to role based access control.

@startuml for S5-23xxxx

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class SubNetwork <<InformationObjectClass>>

class Identity <<InformationObjectClass>>

class Role <<InformationObjectClass>>

class PermissionForMnSs <<InformationObjectClass>>

class ConditionForDynamicMOIs <<InformationObjectClass>>

SubNetwork "1" *-down- "*" Identity: <<names>>

SubNetwork "1" *-down- "*" Role: <<names>>

SubNetwork "1" *-down- "*" PermissionForMnSs: <<names>>

SubNetwork "1" *-down- "*" ConditionForDynamicMOIs: <<names>>

note top of SubNetwork

Represents the following IOCs:

Subnetwork

end note

'or ManagedFunction or ManagedElement

@enduml

Figure X.2.1-1 NRM for role based access control name containment

The IOCs related to Identity, Role, PermissionForMnSs, ConditionForDynamicMOIs are name contained within the Subnetwork or ManagedFunction or ManagedElement.

@startuml for S5-23xxxx

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Identity <<InformationObjectClass>>

class Role <<InformationObjectClass>>

class PermissionForMnSs <<InformationObjectClass>>

class Permission <<DataType>>

class ConditionForDynamicMOIs <<InformationObjectClass>>

class Condition <<DataType>>

Identity "1" o-down-> "*" Role

Role "1" o-down-> "*"  PermissionForMnSs

PermissionForMnSs "1" o-down-> "*"  Permission

Role "1" o-down-> "*" ConditionForDynamicMOIs

ConditionForDynamicMOIs "1" *-down-> "1" Condition

@enduml

Figure X.2.1-1 NRM for role based access control relationship

Editor’s note: access right, for particular consumer, pertaining to attributes of Component B will differ. How to model this functionality is FFS

### X.2.2	Inheritance

This subclause depicts the inheritance relationships. The IOCs are derived from the TOP class

@startuml for S5-23xxxx

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class Identity <<InformationObjectClass>>

class Role <<InformationObjectClass>>

class PermissionForMnSs <<InformationObjectClass>>

class Permission <<DataType>>

class ConditionForDynamicMOIs <<InformationObjectClass>>

class Condition <<DataType>>

Top <|-- Identity

Top <|-- Role

Top <|-- PermissionForMnSs

Top <|-- Permission

Top <|-- ConditionForDynamicMOIs

Top <|-- Condition

@enduml

Figure X.2.2-1 Inheritance diagram for role based access control

## X.3	Class definitions

### X.3.1	Identity

#### X.3.1.1	Definition

This IOC represents an identity of a MnS consumer.

It is used for authentication and authorization. It can be name-contained by SubNetwork or Managed Function or a Managed Element.

The MnS consumer can be a human or a machine user. This IOC enables the creation and storage of an identity of a MnS consumer. The information in this IOC is the starting point for a MnS consumer to identity who it is. This is validated against an authentication service producer.

For the authentication operation to take place the identity related information has to be provisioned into the system by a network operator who could be an administrator. The administrator adds the identityType attribute and identityName attribute which characterizes a machine user or human user respectively. For example we could have an identity like a tenant mapped to the relevant list of roles.

Attribute credential is used to provide information for the credential used together with identity when requesting authentication. The examples of credential are password, certificate, biometric, etc.

The roleRefLis attribute defines the role names associated to a particular user.

The IOC stores the details of the expected tasks to be performed by an identity. The tasks are what is to be done on the network management system. To ease the administration on the system, the tasks are organised as roles. The user can be associated to one or more roles.

#### X.3.1.2	Attribute

The Identity IOC includes the attributes inherited from Top IOC (as defined in Generic resource Model) and the following attributes:

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| identityType | M | T | F | F | T |
| identityName | M | T | F | F | F |
| credential | O | T | T | F | T |
| Attributes related to role |  |  |  |  |  |
| roleRefList | M | T | F | F | T |

#### X.3.1.3	Attribute constraints

None

#### X.3.1.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.2	Role

#### X.3.2.1	Definition

The Role IOC represents a task or collection of tasks in a network management system.

The Role can be name contained under the Subnetwork or Managed Function or a Managed Element.

The Role IOC enables the storage of information as to what resources and actions an identity can work upon. This IOC maintains the static resources that are known to the management system during integration time. This contains all the granular level resources and the corresponding actions.

The permissionsForMnSs attribute contains a list of PermissionForMnSs to contain the list of granular permission sets

The conditionsForMOIsList attribute contains a list of ConditionForDynamicMOIs to contain the list of conditions rules to be resolved later resulting in instances of managed objects.

#### X.3.2.2	Attribute

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| roleName | M | T | F | F | T |
| Attribute related to role |  |  |  |  |  |
| permissionsForMnSsList | M | T | F | F | T |
| conditionsForDynamicMOIsList | M | T | F | F | T |

#### X.3.2.3	Attribute constraints

None

#### X.3.2.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.3	PermissionForMnSs

#### X.3.3.1	Definition

The PermissionForMnSs IOC represents the granular resource and actions in a network management system.

The PermissionForMnSs can be name contained under the Subnetwork or Managed Function or a Managed Element

This IOC enables the storage of the resource types in the system and the possible actions that are allowed on it. The permutations and combinations of these permissions are assigned to a role.

The mnsType attribute is the type of the management service as defined TS 28.622.

The permissionList attribute contains a list of permissions.

#### X.3.3.2	Attribute

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mnsType | M | T | T | F | F |
| permissionList | M | T | T | F | F |

#### X.3.3.3	Attribute constraints

None

#### X.3.3.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.4	Permission <<dataType>>

#### X.3.4.1	Definition

The Permission data type represents the most granular resource and corresponding actions in a network management system.

#### X.3.4.2	Attribute

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| componentA | M | T | F | F | F |
| componentB | M | T | F | F | T |
| componentC | O | T | F | F | T |

#### X.3.4.3	Attribute constraints

None

#### X.3.4.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions

### X.3.4 ConditionForDynamicMOIs

##### X.3.4.1 Definition

This IOC represents a condition for dynamic MOI selection.

This IOC defines the container for the required conditions for a specific scope to the authorized user. The union of all the conditions put together is resolved to result in the managed object instances that a particular user should access.

##### X.3.4.2 Attributes

The ConditionForDynamicMOIs shall be associated with one or more Role.

- The conditionName is a human readable name for a condition.

- conditionList indicates a list of conditions.

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| conditionName | M | T | F | F | T |
| Attribute related to Role |  |  |  |  |  |
| conditionListRef | M | T | F | F | T |

#### X.3.4.3	Attribute constraints

None

#### X.3.4.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

### X.3.5 Condition

##### X.3.5.1 Definition

This IOC results in a list of managed object instances

##### X.3.5.2 Attributes

The attributes are explained in the table below

| Attribute Name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| objectType | CM | T | T | F | F |
| objectInstance | CM | T | T | F | F |
| ConditionAttribute | M | T | F | F | T |
| conditionOperator | M | T | F | F | T |
| valueRange | M | T | F | F | T |

X.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| objectType S CM | Condition: The condition is not for a specific object instance or MnS consumer have no knowledge of the DN of this specific object instance. |
| objectInstance S CM | Condition: The condition is for a specific object instance and MnS consumer have the knowledge of the DN of this specific object instance. |

#### X.3.5.4	Notifications

The common notifications defined in clause X.5 are valid for this IOC, without exceptions or additions.

## X.4	Attribute definitions

### X.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

First change

First change

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| identityType | This defines a readable string to uniquely represent an identity AllowedValues: username, email address, phone number, IP address, machineuser | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| identityName | This indicates a type of identifier, AllowedValues: NA | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| credential | The credential of an MnS consumer or producer used for authentication with authentication service producer. It could be password, certificate, key, pass phrase, etc., based on authentication protocol and factor. AllowedValues: NA | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: No value isNullable: False |
| roleRefList | This defines the list of roles associated with an identity AllowedValues: NA | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| roleName | This string defines the name of a role AllowedValues: NA | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| mnsType | This attribute defines the type of management service AllowedValues: ProvMnS, FaultSupervisionMnS, StreamingDataReportMnS, FileDataReportingMnS | type: ENUM multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| permissionsForMnSsList | This defines the list of permissions for MnSs which constitue the resources and corresponding actions AllowedValues: NA | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| permissionList | This defines the list permissions for resources and corresponding actions for static objects AllowedValues: NA | type: Permission multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentA | This attribute is as indicated in 28.532 AllowedValues: values as defined in 28.532 | type: FFS multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| componentB | This attribute defines a MnS component type B represented by information models of managed entities. A MnS component type B is also called Network Resource Model (NRM). MnS component type B examples are: 1) Network resource models as defined in TS 28.622 2) Network resource models as defined in TS 28.541 AllowedValues: NA | type: FFS multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| componentC | This attribute defines a MnS component type C which is the performance information of the managed entity and fault information of the managed entity. The following are examples of Management service component type C: 1. Alarm information as defined in TS 28.532 and TS 28.545 2. Performance data as defined in TS 28.552 , TS 28.554 and TS 32.425. AllowedValues: NA | type: FFS multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| conditionName | This is a string which defines the name of a condition | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| conditionListRef | This results in a list of DN for conditions which together results in the managed object instances | type: DN multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| conditionAttribute | This defines the properties of an Managed Object Class. It can be a 3GPP management system specified attribute name in an information object class, or it may be a measurement metric defined in TS 28.552 or KPI in TS 28.554, a class name etc. | type: String multiplicity: 1 isOrdered: NA isUnique: NA defaultValue: None isNullable: False |
| conditionOperator | It expresses the limits within which the conditionAttribute is allowed/supposed to be Operators include but not limited to ‘<’, ‘>’, ‘=’’!=’. The oprators may be: "IS_EQUAL_TO", "IS_LESS_THAN", "IS_GREATER_THAN", "IS_WITHIN_RANGE", "IS_OUTSIDE_RANGE”, "IS_EQUAL_TO_OR_LESS_THAN", "IS_EQUAL_TO_OR_GREATER_THAN", “START_WITH”, “FULL_MATCH_STRING” | type: Enum multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| valueRange | This defines these numerical values When the conditionAttribute is a number type of attribute (e.g., integer, Real), the valueRange will be a Real. When the conditionAttribute is a string to matchwith, the valueRange will be a String (e.g., a IP address to match with) | type: Choice multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| objectType | It describes the type of condition object of the condition that are required to be applied on. It can be class name of the managed object. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| objectInstance | It describes a specific condition instance (e.g. instance of managed object) to which the condition should apply. allowedValues: Not Applicable | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

## X.5	Common Notifications

### X.5.1	Condition related notifications

The below CM notifications are valid for the access control related IOCs

### 4.5.2	Configuration notifications

This clause presents a list of notifications, defined inTS 28.532, that a MnS consumer can receive. The notification header attribute objectClass/objectInstance, defined in TS 32.302, captures the DN of an instance of an IOC defined in the present document.

| Name | S | Notes |
| --- | --- | --- |
| notifyMOIObjectCreation | O |  |
| notifyMOIObjectDeletion | O |  |
| notifyMOIAttributeValueChanges | O |  |
| notifyMOIChanges | O |  |
| notifyEvent | O |  |

End of first change

Start of next modification

## Y.x	Access Control management service

### Y.x.1	Operations

#### Y.x.1.1	authentication operation (CM)

##### Y.x.1.1.1	Description

This operation is invoked by MnS consumer to request the authentication service producer to authenticate MnS consumer. MnS consumer and authentication service producer shall use one of the following methods for authentication:

-	Mutual authentication between MnS consumer and authentication service producer provided by the transport layer protection solution.

-	Client credentials assertion based authentication.

When client credentials assertion based authentication is used, the operation is invoked by a MnS consumer to request authentication by authentication service producer. The authentication service producer gets the identifier and credential of the MnS consumer for validation. Based on identifier in the request, authentication service producer gets identity information, e.g. credential of the identity, associated role(s) of the identity, etc., from local data store (e.g., a directory). The authentication service producer authenticates the MnS consumer by validating the identity information. associated to the role(s) the MnS consumer belongs to.

If authentication is successful, the authentication service producer sends successful response with assertion to the MnS consumer. MnS producer validates the assertion to authenticate a MnS consumer.

If fail for authentication, the authentication service producer sends failure response to the MnS consumer.

##### Y.x.1.1.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| identityName | M | String | This parameter specifies the identity of the MnS consumer that is to be authenticate. |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter may have a null value. When this parameter is supplied, it contains a list of name/value pairs specifying attribute identifiers and their values to be assigned to the identity to be authenticated. |

##### Y.x.1.1.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the authenticated MnS consumer and the actual value assigned to each. |
| Status | M | ENUM (OperationSucceeded, OperationFailed) |  |

#### Y.x.1.2	authorization operation (M)

##### Y.x.1.2.1	Definition

The operation is used to authorize a MnS consumer by the authorization service producer.

MnS consumer, MnS producer and authorization service producer shall use one of the following methods for authorization:

-

-    Token based authorization framework including various grant modes (e.g., as specified in RFC 6749 [xx])

-	Static authorization as specified (e.g., in RFC 8341 for NACM [yy])

When token based authorization framework is used, authorization service producer receives authorization request, gets the assertion of the MnS consumer related to the authentication and probably other context information (e.g. resource, action) from the request. If access token is supported by the MnS producer and consumer:

-  The MnS consumer gets access token from authorization service producer by providing the assertion from authentication session.

-  The MnS consumer starts normal operation with MnS producer with the access token. The MnS producer validates the token. If the token is valid, the MnS producer performs the action(operation) on the resource authorized and returns result to the MnS consumer.

If access token is not supported by the MnS producer and consumer:

-  When normal operation starts, the MnS producer validate the authentication assertion and check permission of the MnS consumer with authorization service producer.

-  The MnS producer performs the operation on the targeted resource which is part of the authorized scope and returns result to the consumer if the MnS request is allowed according to permissions.

If fail for authorization, the authorization service producer sends failure response to the consumer.

##### Y.x.1.2.2	Input parameters

| Parameter Name | S | Information Type / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListIn | M | LIST OF SEQUENCE< attribute name, attribute value> | This parameter may have a null value. When this parameter is supplied, it contains a list of name/value pairs specifying attribute identifiers and their values to be assigned to the identity to be authorized. |

##### Y.x.1.2.3	Output parameters

| Parameter name | S | Matching Information / Legal Values | Comment |
| --- | --- | --- | --- |
| attributeListOut | M | LIST OF SEQUENCE< attribute name, attribute value> | This list of name/value pairs contains the attributes of the authorization for the MnS consumer and the actual value assigned to each. |
| Status | M | ENUM (OperationSucceeded, OperationFailed) |  |

End of changes

# Annex Z (Informative):

# Z.1	Informative sequence diagram: Commissioning flow to be carried out by an administrator:

The sequence diagram below explains the IOCs to be used as a pre-requisite for enabling the role based access control. The notes above in the diagram also depict an example of each of the tasks.

The sequence diagram below explains the operations and the interactions with the authentication and authorization service which typically should suit any implementation be in oAUTH or a NACM implementation.

The notes along the flows show the examples related to the IOCs provisioning operations.

@startuml for S5-23xxxx   Setting up pre-requisite for generic Authentication and Authorization with both oAuth and NACM

autonumber

participant "MnS consumer (Administrator)" as MnScA

participant "MnS producer (Authentication and Authorization Service producer)" as MnSpA

== Commissioning for Role based access control ==

MnScA -> MnSpA++: createMOI(PermissionForMnSs,resource,action) request

note right: "e.g.:networkElement/Alarm/Severity, Read/Write"

create PermissionForMnSs

MnSpA -> PermissionForMnSs: MnS producer creates a "PermissionForMnSs " instance

MnScA <- MnSpA--: createMOI(PermissionForMnSs,resource,action) response

MnScA -> MnSpA++: createMOI(ConditionForDynamicMOIs,conditionName,conditionAttribute, conditionOperator, valueRange) request

note right: "e.g.:Alarm Severity > 4, Read/Write"

create ConditionForDynamicMOIs

MnSpA -> ConditionForDynamicMOIs: MnS producer creates a "ConditionForDynamicMOIs " instance

MnScA <- MnSpA--: createMOI(ConditionForDynamicMOIs,conditionName,conditionAttribute, conditionOperator, valueRange) response

MnScA -> MnSpA++: createMOI(Role, roleName, resources, List<actions>) request

note right: "Alarm Survelience, IOCs(network element/Alarm/Severity, Read/Write)"

create  Role

MnSpA -> Role: MnS producer creates a "Role" instance

MnScA <- MnSpA--: createMOI(Role, roleName, roleID, resources, List <actions>) response

MnScA -> MnSpA++: createMOI(Identity, identityName, password, List<Role>) request

note right: "User1, *****, Alarm Survelience"

create  Identity

MnSpA -> Identity: MnS producer creates a "Identity" instance

MnScA -> MnSpA--: createMOI(Identity, identityName, password, List<Role>) response

@enduml

Figure Z.1 - Sequence of commissioning for role based access control

## Commissioning usecases to be carried out by an administrator

- To carry out authentication and authorization based on role based access a set of tasks needs to be in place at the commissioning phase.

The network equipment vendor will typically take care of the below:

    - The granularity level at which the management objects need to be authorized

    - The actions that need to be permitted on these granular level objects

This is typically carried out by an administrator who has to setup the system for operational purposes which included authentication and authorization for every operation. Hence the administrator will need to have a good view of the permissions of the devices being integrated and managed. The administrator will need to know the possible scope of action for various users in the system on the instances that are created. The administrator will need to know the roles that need to be created in the system as well as the mapping of the same to the users. Below is the detail of every task.

The below elaborates the sequence flow to depict the IOCs involved for provisioning.

### a. Permissions with resources and actions

- Permissions are combinations of static resources and actions. The network equipment vendor can introduce the resources and actions at various levels of granularity as well as in the language that can be suitable for the vendor. When the commissioning phase happens it is important to translate these permissions to the authentication and authorization system so that it is aware of the static resources and corresponding actions on them for access control.

Hence the IOC for PermissionForMnSs is required to create the necessary data.

### b. Conditions for Managed Object Instances

The resources in the system that need to be access controlled are also dynamic in nature. They cannot be identified and assigned at integration time. As and when the system expands there will be new instance of devices created along with its corresponding attributes which are specific to that instance. All this needs to be scoped to a particular used with proper access control rights. One of the ways of scoping these dynamic instances is with the aid of a condition or filter. Once the authentication and authorization service producer has access to the result of these conditions for a particular user then we have access control in place including the dynamic resources.

Hence the IOC for ConditionForDynamicMOIs is required to create the necessary conditions.

### c Role creation with association to permissions and conditions

The meaningful task is the role which defines the responsibility realm for the user. Hence the role contains the overall distribution of resources and actions it can work on by resolving the condition and permissions. The static resources identified need to be combined with the dynamically created instances of the resource in order to be assigned to a meaningful task in the system.

The roles are typically created according to the convenience and hierarchy of the organisation.  Coarse grained or fine grained permissions can be created with the help of roles.

The management system should be setup for access control by a system administrator who will have the know how of creating the required users and roles. Roles will use the various possibilities to map resources and actions.

Hence the IOC for Role is required to create the necessary data related to various roles in the system.

### d.Identity to role assignment

The identity or user of the system needs to be assigned the responsibilities to carry out the OAM operations. Hence the set of responsibilities are defined in various roles. Assignment of the roles to an identity is required to make the administrative task more efficient and flexible. This also shields the administrator from getting into the details of the know how of granular resources and actions relevant to it.

Hence the IOC for Identity is required to create the necessary data.

# Z.2 Informative sequence diagram: Authentication and Authorization with role based access control:

Post the above tasks we have the possibility to have role-based access in operation with every service call being authenticated and authorized.

The below sequence diagram shows how the operations are invoked from a consumer towards an authentication and authorization service producer.

When an MnS is invoked, the MnS consumer authenticates towards an authentication service producer. This is then followed by checking the access rights with respect to the role-based access. This takes place in the authorization service producer which is invoked from the MnS producer to check if its resources can be accessed with respect to the related operation.

Hence then need of a authentication and authorization service producer which will validate a user who want to access the resources as well as validates what the user can access.

@startuml happy flow for S5-234716 Setting up operations for generic Authentication and Authorization with both oAuth and NACM

== Operational ==

participant "MnS consumer" as MnSc

participant "MnS pfor roducer (Authentication and Authorization Service producer)" as MnSpA

participant "MnS producer" as MnSp

alt oAuth

'create AandAServiceProvider

MnSc -> MnSpA++: Authenticate(identityName)

note right: "User1"

MnSpA -> MnSc--: return identity token

MnSc -> MnSpA++: Authorise with identity token

MnSpA -> MnSpA: getRolesForIdentity(identity=)

MnSpA -> MnSpA: permissionsOnResourcesAndAction(Role=)

MnSpA -> MnSc--: return access token

MnSc -> MnSp++: Executes an operation( Modify Alarm operation /n Component A on Component B or C)

MnSp -> MnSp: Validate the Token()

MnSp -> MnSpA: getDynamicMOIsForCondition(Role=)

MnSpA -> MnSp: return getDynamicMOIsForCondition(Role=)

MnSp -> MnSp--:    Execute operation

'MnSp -> AandAServiceProvider: MnS producer creates a AuthenticationAndAuthorizationServiceProvider".

else NACM

MnSc <- MnSc: Authentication is completed on the transport layer

MnSc -> MnSp++: Executes an operation( Modify Alarm /n Component A on Component B or C)

create NACMServer

MnSp -> NACMServer: AuthorizeResourcesAndActions() request

NACMServer -> NACMServer: getRolesForIdentity(identity=)

NACMServer -> NACMServer: permissionsOnResourcesAndActions(Role=)

NACMServer -> MnSp--: AuthorizeResourcesAndActions() response

MnSp <- MnSp:    Execute operation

end

MnSc <- MnSp: Execution completed

@enduml

Figure Z.2 - Sequence diagram for operations of authentication and authorization

End of changes