3GPP TSG-SA5 Meeting #150	S5-235585

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0109 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.312 Intent Conflict Resolution Requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung Electronics France SA, NTT DOCOMO INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The Intent conflict resolution was studied and need to be specified normatively. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Intent conflict resolution use case and requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Section

### 5.3.x	Intent Conflict Resolution

#### 5.3.x.1	Introduction

The MnS consumer may create an intent containing two or more intent expectations, and each intent expectation may contain multiple expectation targets. On receiving and after analysing the intent, the MnS producer may realize that the intent expectations or expectation targets in one intent are contradicted, i.e. the MnS producer may detect conflicts in the intent. Also, an intent is considered to have conflict with other intents if the requirements stated in one intent is conflicting with the requirements (includes intent expectation and corresponding expectation targets) stated in another intent. Following are the intent related conflict scenarios:

-	Target conflict, which represents the conflict between two or more expectation targets within the same intent expectation.

-	Expectation conflict, which represents the conflict between two or more intent expectations with the same intent

-	Intent conflict, which represents the conflict between two or more different intents.

When such conflicts are detected, the MnS producer needs to notify the MnS consumer about the conflict, indicating the intent, intent expectations or expectation targets which give rise to the conflict. The MnS producer can decide to take measures to mitigate the intent conflicts, such as termination of the whole intent, recommended intent (e.g. recommended expectations or targets, or termination of part of intent), or updating the execution time of the intent. Before the producer implements the mitigations, consumer will be notified. At this time consumer can choose to reject the particular mitigation procedure.

The MnS consumer may may give some intent conflict handling guidelines (e.g. assign priority for such intent or intent expectation or expectation targets) to MnS producer to solve such intent conflict or eliminate the terminated state which is caused by the reason of conflict detected.

#### 5.3.x.2	Requirements

REQ-Intent_Driven_MnS- ConflictResolution -a: The intent driven MnS should have the capability to inform the authorized MnS consumer about intent related conflicts (both explicit and implicit conflicts) as soon as they are identified (either during creation or operation), including intent conflict, expectation conflict and target conflict.

REQ-Intent_Driven_MnS- ConflictResolution -b: The intent driven MnS should have the capability to allow the authorized MnS consumer to give intent conflict handling guidelines to MnS producer to solve such intent conflict and potentially affect the terminated state which is caused by the reason of conflict detected.

REQ-Intent_Driven_MnS- ConflictResolution -c: The intent driven MnS should have the capability to inform the authorized MnS consumer about possible solutions for conflicts, including suggesting to terminate intent instances, recommended intent targets, or recommended execution time of the intent instances.

REQ-Intent_Driven_MnS- ConflictResolution -d: The intent driven MnS should have the capability to allow the authorized consumer to reject the particular solution identified by the producer to mitigate the conflict.

End Of Modified Section