3GPP TSG-SA5 Meeting #150 	S5-235643

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.554 | CR | 0139 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.554 add KPI on Energy Efficiency KPI of URLLC Network Slice based on reliability |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Unicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | EE5GPLUS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Performance of a URLLC network slice can be its latency and reliability. The lower the latency or higher the reliability of a URLLC network slice is, the higher its performance is. However, existing standardised solution (TS 28.554, clause 6.7.2.3), determines Energy Efficiency (EE) for a URLLC slice only based on Latency performance, reliability performance is not considered.Reliability performance is an integral part of URLLC slice by nature. Hence, it is important that a variant of Energy Efficiency KPI should also consider reliability performance of a URLLC slice for comprehensive assessment of EE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new KPI on Energy Efficiency KPI of URLLC Network Slice based on reliability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

6.4.2.3.x	Based on reliability of the network slice

a) A KPI that shows the energy efficiency of network slices of type URLLC based on reliability. The Pns for a network slice of type URLLC is the multiplication of downlink packet transmission reliability and  uplink packet transmission reliability of the network slice [2]. In this KPI variant, reliability is the factor considered for evaluating the performance of network slice.

b)

PURLLC, Reliability=Network slice DL reliability*Network slice UL reliability

,where

‘Network slice DL reliability’ is defined as the downlink packet transmission reliability of the network slice, and where the downlink packet transmission reliability of the network slice for one S-NSSAI is defined by:

Network slice DL reliability=DLRelPSR_Uu.SNSSAI100*DLRelPSR_N3.SNSSAI100

,Where

DLRelPSR_Uu.SNSSAI and DLRelPSR_N3.SNSSAI are defined according to clause 6.8.1 in TS 28.554.

‘Network slice UL reliability’ is defined as the uplink packet transmission reliability of the network slice, and where the uplink packet transmission reliability of the network slice for one S-NSSAI is defined by:

Network slice UL reliability=ULRelPSR_Uu.SNSSAI100*ULRelPSR_N3.SNSSAI100

, Where

ULRelPSR_Uu.SNSSAI and ULRelPSR_N3.SNSSAI are defined according to clause 6.8.1 in TS 28.554[2].

Besides, URLLC Performance management on reliability in RAN has been discussed in the Study on management aspects of URLLC, and the discussion results would be considered if needed.

This KPI is obtained by the muliplication of downlink reliability and uplink reliability of the network slice divided by the energy consumption of the network slice. KPI The unit of this KPI is Reliability per Joule (Rel/J).

c)

EEURLLC,Reliability=Network slice DL reliability*Network slice UL reliabilityECns

d) The KPI object is network slice.

End of Change