3GPP TSG-SA5 Meeting #150                    	S5-235777

# Gothenburg (Sweden), 21 - 25 August 2023

Title:	Reply to LS on AI/ML for NG-RAN Energy Saving Energy Cost index

Response to:	S5-235036 (R3-233515: LS on AI/ML for NG-RAN Energy Saving Energy Cost index)

Release:	3GPP Rel-18

Work Item:	EE5GPLUS_Ph2

Source:	3GPP SA5

To:	3GPP RAN3

Cc:

Contact Person:

#### Name:	Jean-Michel Cornily

E-mail Address:	jean <dot> michel <dot> cornily <at> huawei <dot> com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

3GPP SA5 thanks RAN3 for your LS on AI/ML for NG-RAN Energy Saving Energy Cost index.

Here below, 3GPP SA5 provides its understanding of the problem to be solved, preliminary thoughts about potential solutions and questions for clarification.

SA5 understanding of the problem is:

- NG-RAN nodes (i.e. gNBs and ng-eNBs) should exchange information about their respective energy consumption. However, instead of exchanging their actual energy consumption (e.g. in Kw/h), RAN3 expects the actual energy consumption be converted into an ‘Energy Cost’, i.e. an index which reflects the energy consumption of the NG-RAN Nodes.

- The rule to convert the actual energy consumption of the NG-RAN nodes into the Energy Cost is provisioned via OAM into the NG-RAN nodes and is unified across the Operator’s network.

2. Questions to RAN3 for clarification:

- Who should be responsible for defining the unified mapping rule: 3GPP / NG-RAN node vendor / Operator / Other?

- Should the calculation of the Energy Cost be based on instantaneous power consumption measurement (in W) (implying that the Energy Cost can be constantly updated) or on min/max/mean energy consumption measurements (in KwH) made over a past period (whose duration can be configured by the Operator)?

- Should the Energy Cost be calculated in near-real time or non-real time (see above)?

- What is your view on how to apply this Energy Cost concept to split NG-RAN nodes, where NG-RAN node components can be geographically distributed and/or virtualized?

3. Actions:

To RAN3: please provide your feedback on SA5 current understanding and reply to SA5 questions for clarification.

5. Date of Next TSG-SA WG5 Meetings:

SA5#151	9 – 13 October 2023	Xiamen, China

SA5#152	13 – 17 November 2023	Chicago, US