3GPP TSG-SA5 Meeting #150	S5-235826

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | DraftCR | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.105 Use case and requirements for ML Training activation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of ML Training activation has been discussed and agreed (in TR 28.908). This CR is to normatively propose the use case and requirements for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use case description and requirements have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use case “ML Training activation” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.B (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | X |  | O&M Specifications |  |  |  | TS 28.105 DraftCR S5-235139 |  |  |
| Other comments: |  | Input to DraftCR TS 28.105 DraftCR S5-235139 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-235317 |  |  |  |  |  |  |  |  |

Start of First change

### 6.2.B		ML training Configuration management

#### 6.2.B.1	Description

The ML training function and the associated ML entity may need to be configured to conduct training in the 5G system to align with the consumer´s expectation, e.g., to enable the ML training function to perform training on the right ML entity using the right data. The ML training MnS should provide a capability to configure the ML training function or its associated ML entities (e.g., with policies, target, conditions where applicable).

The following aspects of ML training may be included in capabilities for ML training configuration:

- MnS consumer-initiated activation and deactivation of training on ML entities.

- MnS producer-initiated and policy based activation from of ML training on ML entities

#### 6.2.B.2		Use cases

##### 6.2.B.2.1	MnS consumer-initiated activation and deactivation of training on ML entities

The MnS consumer monitors the network performance and determines whether to, and when to activate or deactivate the training on or for a specific ML entity. For Energy Saving (ES) purpose for instance, the ML entity for deciding the loads at which energy saving is triggered may already be active. But after collecting information of the coverage and capacity booster cells in an area, the MnS Consumer may decide that it is necessary to estimate the future load in the cells and so may decide to also activate the ML entity for predicting the future cell loads. The ML training MnS should include a capability to activate or deactivate training on one or more ML entities associated to a given training function.

#### 6.2.B.3	Requirements for ML training Configuration management

Table 6.2.B.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-AIML_ INF_ACT-1 | The ML training MnS producer should have a capability enabling an authorized MnS consumer to activate or deactivate one or more ML entities associated with an ML training function. | MnS consumer-initiated activation and deactivation of ML training (clause 6.2.B.2.1) |
| REQ-AIML_ INF_ACT-2 | The ML training MnS producer should have a capability to inform an authorized MnS consumer about the activation and deactivation of an ML entity triggered by the ML training MnS producer. | MnS producer-initiated activation and deactivation of MLtraining on ML entities (clause 6.2.B.2.1) |

End of Changes