3GPP TSG-SA5 Meeting #150 	S5-235034

Goteborg, Sweden, 21 - 25 August 2023

# Source:	3GPP RAN3

# Title:	Reply LS on RAN UE Id optionality

# Document for:	Information, Discussion

Agenda Item:     6.1

3GPP TSG-RAN WG3 Meeting #120	R3-233476

# Incheon, South Korea, May 22nd – 26th 2023

Title:	Reply LS on RAN UE Id optionality

Response to:	S5-232761 / R3-231121

Release:	Release 18

Work Item:	Study on Management of Trace/MDT phase 2 - 950034

Source:	RAN3

To:	SA5

Cc:

Contact Person:

#### Name:	Julien Muller

Tel. Number:

E-mail Address:	julien dot muller at ericsson dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN3 thanks SA5 for the LS on RAN UE Id optionality.

SA5 asked RAN3 to consider and provide feedback on the following:

- Availability of UE identifier(s) suitable for trace data correlation for Dual Connectivity and split deployments

- Any relevant details and/or limitations in their usage, including persistency and conditionality

- Details of any ongoing work which could affect any details provided

RAN3 has considered the above questions and would like to provide the following feedback:

- RAN3 has discussed UE identifiers available within the RAN, and concluded that the RAN UE ID can be suitable for trace data correlation within the gNB for split deployments. A combination of UE E1AP and UE F1AP IDs in gNB-CU-CP could also be used for split deployments.

- Relevant details and/or limitations in the usage of the RAN UE ID, including persistency and conditionality:

  - The RAN UE ID was introduced in Rel-15 to correlate logs captured from the different logical entities composing a gNB (i.e. gNB-CU-CP, gNB-CU-UP and gNB-DU), and belonging to the same UE.

  - The RAN UE ID is only used over E1 and F1 interfaces, allowing correlation within the same gNB. However, this IE has an optional presence.

  - The RAN UE ID is not present in X2AP or XnAP procedures used for dual-connectivity, and therefore does not allow trace data correlation between MN and SN.

  - The RAN UE ID is not used for inter-node HO case.

  - RAN3 do not see any other limitations.

- RAN3 has not identified any ongoing work that could affect the provided feedback.

# 2	Actions

Action to SA5 group.

RAN3 respectfully asks SA5 to take the above feedback into account for the study on Management of Trace/MDT phase 2.

# 3	Dates of next TSG-RAN WG3 meetings

TSG-RAN WG3 Meeting#121			21 - 25 August 2023		 Toulouse, France

TSG-RAN WG3 Meeting#121-bis		9 - 13 October 2023		 Xiamen, China