3GPP TSG-SA5 Meeting #150	S5-235208

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | draftCR | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.104 Enhancing resource analytics use case with correlation across NFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The resource analytics use case has been described to provide recommendations to orchestrate 3GPP NFs by performing resource related analytics in 3GPP NFs. In order to perform this, the correlation of resource analytics across 3GPP NFs need to be performed. Stand alone resource analytics of 3GPP NFs cannot provide recommendations to orchestrate different 3GPP NFs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the correlation of resource analytics across 3GPP NFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description of resource analytics use case description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.x.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

### 7.2.x	Resource related analytics

#### 7.2.x.1	NF resource utilization analysis

##### 7.2.x.1.1	Description

This MDA capability is for analysis of resource utilization of 3GPP NFs.

##### 7.2.x.1.2	Use case

The 3GPP system is a resource limited system, no matter whether the NF is working on virtualized resources or physical resources.

Resource shortage would affect the QoS and potentially impact users’ quality of experience (QoE), e.g., by lowering the users’ data throughput, prolonging the users’ data delay, raising the rejections for the establishment of new connections (e.g., RRC connection), sessions (e.g., PDU session) and resources (e.g., QoS flows, DRBs, etc.) and increasing the drops of the existing connections, sessions, and resources. This may also consequently lead to risking or failing SLAs.

On the other hand, resource excess would cause wastage that leads to additional CapEx and OpEx.

Therefore, it is imperative to ensure optimum and efficient resource utilization for the NFs.

The resource utilization of an NF is heavily dependent on load or traffic patterns, which could vary in different coverage areas (e.g., business area, entertainment area, and residential area) and in different time periods (weekdays and time of the day). It is desirable that the spare resource of the low-usage areas can be allocated to the busy areas.

It is expected that MDA can perform an analysis of the resource utilization for physical resources or virtualized resources for the 3GPP NFs (in a specific domain or cross domains) to indicate the resource usage patterns in the past and predict the resource usage trend for some time periods in the future. The physical resources to be analyzed may include hardware resources (e.g., CPU), DL and UL PRBs (for gNB), etc., while the virtualized resources to be analyzed may include virtual CPU, virtual memory, virtual disk, etc.

It is also very useful that MDA correlates the resource analytics across 3GPP NFs and provides recommendations that can be utilized to efficiently orchestrate the resources among NFs between the low usage and high usage areas for some time periods. The recommended actions could be for example to schedule the "scale in" and "scale out" of VNFs via ETSI MANO system to optimize the allocation of the virtualized resources.

##### 7.2.x.1.3	Requirements

Table 7.2.x.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-RES_UTI_ANA-01 | MDA capability for resource utilization analysis shall include identifying the 3GPP NFs with low usage of physical resources (see Note 1). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-02 | MDA capability for resource utilization analysis shall include identifying the 3GPP NFs with high usage of physical resources (see Note 1). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-03 | MDA capability for resource utilization analysis shall include providing the prediction of physical resource usage for a 3GPP NF (see Note 1). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-04 | MDA capability for resource utilization analysis shall include identifying the 3GPP NFs with low usage of virtualized resources (see Note 2). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-05 | MDA capability for resource utilization analysis shall include identifying the 3GPP NFs with high usage of virtualized resources (see Note 2). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-06 | MDA capability for resource utilization analysis shall include providing the prediction of virtualized resource usage for a 3GPP NF (see Note 2). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-07 | MDA capability for resource utilization analysis shall include correlating resource analytics across 3GPP NFs and providing recommended actions. e.g. to orchestrate the resource allocation or load balancing for 3GPP NFs. | NF resource utilization analysis |
| NOTE 1: The requirement is valid only if the subject 3GPP NF uses physical resources. NOTE 2: The requirement is valid only if the subject 3GPP NF uses virtualized resources. |  |  |

End of Changes