3GPP TSG-SA5 Meeting #150	S5-235239

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0447 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add number of MA PDU sessions for ATSSS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecomunication Corp., Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | ATSSS can reduce mobile network congestion and deliver better customer experience. The key enabler of ATSSS is the Multi Access-PDU (MA PDU) Session. In the process of PDU session esteblishment, for the MA PDU session, the SMF includes an "MA PDU session Accepted" indication in the Namf_Communication_N1N2MessageTransfer message, then the AMF marks this PDU session as MA PDU session based on the received "MA PDU session Accepted" indication. Therefore, it is feasible and necessary to monitor the performance of MDA PDU session establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add ATSSS related performance measurement including: - Number of MA PDU sessions (Mean) - Number of MA PDU sessions (Maximum) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot effectively monitor the performance of ATSSS and solve the corresponding network problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1.13.X(new), 5.3.1.13.Y(new), A.107 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

5.3.1.13	MA PDU session management

5.3.1.13.X	Number of MA PDU sessions (Mean)

a)	This measurement provides the mean number of MA PDU sessions.

b)	SI.

c)	The measurement is obtained by sampling at a pre-defined interval, the number of MA PDU sessions established by SMF, and then taking the arithmetic mean. The measurement is optionally split into subcounters per S-NSSAI.

d)	A single integer value.

e)	SM.MaPduSessionNbrMean, and optionally
SM.MaPduSessionNbrMean.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	SMFFunction.

g)	Valid for packet switched traffic.

h)	5GS.

2nd change

5.3.1.13.Y	Number of MA PDU sessions (Maximum)

a)	This measurement provides the max number of MA PDU sessions.

b)	SI

c)	The measurement is obtained by sampling at a pre-defined interval, the number of MA PDU sessions established by SMF, and then selecting the maximum value. The measurement is optionally split into subcounters per S-NSSAI.

d)	A single integer value

e)	SM.MaPduSessionNbrMax, and optionally
SM.MaPduSessionNbrMax.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

3rd change

A.107	Use case of monitoring of MA PDU session management for ATSSS

The ATSSS feature enables a multi-access PDU Connectivity Service, which can exchange PDUs between the UE and a data network by simultaneously using one 3GPP access network and one non-3GPP access network. The multi-access PDU Connectivity Service is realized by a Multi-Access PDU (MA PDU) Session.

When establishing an MA PDU session, the AMF informs the SMF that the request is for a MA PDU Session. The number and success rate of MA PDU session creations, and the number of MA PDU sessions running on the SMF are some of the basic performance measurements for monitoring the performance of MA PDU session establishment and reflecting the effect of ATSSS. And the failed MA PDU session creations measured per cause are helpful to pinpoint and solve the network issues in case the performance is below the expectation.

End of changes