3GPP TSG-SA5 Meeting #150	S5-235240

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0448 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add MA PDU session conversion measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecomunication Corp., Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | ATSSS is a feature that can be supported by the UE and the 5GC network to route data traffic across 3GPP access and non-3GPP access networks. To study and develop the policies of ATSSS to improve network efficiency, network capacity and user experience, operators would like to know how many PDU sessions can be (and successfully) converted to MA PDU sessions to enable ATSSS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add numbers of PDU sessions which can be converted to MA PDU sessions and the successful convered number. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot effectively monitor the performance of ATSSS and solve the corresponding network problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.3.1.13.X(new), 5.3.1.13.Y(new), A.107 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 23.501 [4].

CHO	Conditional Handover

CLI	Cross Link Interference

DAPS	Dual Active Protocol Stack

GP	Guard Period

HO	Handover

kbit	kilobit (1000 bits)

LHO	Legacy Handover

MA PDU	Multi-Access PDU

MN	Master Node.

MPQUIC	Multi-Path QUIC

MPTCP	Multi-Path TCP Protocol

NG-RAN	Next Generation Radio Access Network

PI	Performance Indicator

SA PDU	Single-Access PDU

SN	Secondary Node.

SRS	Sounding Reference Signal

TEID	Tunnel Endpoint IDentifier

2nd change

5.3.1.13	MA PDU session management

5.3.1.13.X	Number of allowed MA PDU session conversions

a)	This measurement provides the number of SA PDU sessions that may be converted to a MA PDU Session, if the 5GC network wants to.

b)	CC.

c)	On receipt by the SMF from AMF of Nsmf_PDUSession_CreateSMContext Request including "MA PDU Network-Upgrade Allowed" indication (see clause 4.22.3 of TS 23.502 [7]). Each requested single-access PDU Session that may be converted to a MA PDU Session is added to the relevant subcounter per S-NSSAI.

d)	Each subcounter is an integer value.

e)	SM.MaPduSessionConvAllowed.SNSSAI,
Where SNSSAI identifies the S-NSSAI.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS.

5.3.1.13.Y	Number of successful MA PDU session conversions

a)	This measurement provides the number of the requested SA PDU Sessions that have been successfully converted by the network to a MA PDU session.

b)	CC

c)	On transmission by the SMF of Namf_Communication_N1N2MessageTransfer message with an "MA PDU session Accepted" indication to the AMF (see clause 4.22.3 of TS 23.502 [7]). Each requested SA PDU Session that is successfully converted by the network to a MA PDU Session is added to the relevant subcounter per S-NSSAI.

d)	Each subcounter is an integer value.

e)	SM.MaPduSessionConvSucc.SNSSAI,
Where SNSSAI identifies the S-NSSAI.

f)	SMFFunction

g)	Valid for packet switched traffic

h)	5GS

3rd change

A.107	Use case of monitoring of MA PDU session management for ATSSS

The ATSSS feature enables a multi-access PDU Connectivity Service, which can exchange PDUs between the UE and a data network by simultaneously using one 3GPP access network and one non-3GPP access network. The multi-access PDU Connectivity Service is realized by a Multi-Access PDU (MA PDU) Session.

When establishing an MA PDU session, the AMF informs the SMF that the request is for a MA PDU Session. The number and success rate of MA PDU session creations, and the number of PDU sessions running on the SMF are some of the basic performance measurements for monitoring the performance of MA PDU session establishment. And the failed MA PDU session creations measured per cause are helpful to pinpoint and solve the network issues in case the performance is below the expectation.

When an ATSSS-capable UE requests to establish a single-access PDU Session, but no policy in the UE and no local restrictions mandate a single access, the 5GC network may decide to modify it to a Multi-Access PDU (MA PDU) Session. This decision may be taken when e.g. the SMF wants to offload some traffic of the requested PDU Session to non-3GPP access or when the SMF wants to apply MPTCP and/or MPQUIC to provide bandwidth aggregation for the requested PDU Session(see TS 23.502 [7]). Whether a requested PDU Session can be successful converted to a MA PDU Session by the network may help the analysis and update of ATSSS policy. Therefore the performance of UE requested PDU session establishment with network modification to MA PDU session procedures need to be monitored.