3GPP TSG-SA5 Meeting #150	S5-235416

# Goteborg, Sweden, 21st Aug 2023 - 25th Aug 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.537 | CR | 0012 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR 28.537 Clarify management service discovery use cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GDMS |  |  |  |  |  | Date: |  |  | 2023-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current use case descriptions for service management discovery do not reflect the implementation. The management of the MnS Registry data is open to interpretation and its relation to other management capabilities (currently under study) is unclear. There is ongoing work in TR 28.831 to define the detailed capabilities retrieval which will require further updates to the new use cases added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the use case descriptions for service management discovery to reflect what has been implemented and how the MnS Registry is intended to be used. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency in interpretation affects interoperability. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1, 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-234370 |  |  |  |  |  |  |  |  |

First change

# 5	Discovery of Management Services

## 5.1	Overview

To enable communication between MnS Consumers and MnS Producers, MnS Consumers need mechanisms to discover management service information available in the 3GPP management system, and their management capabilities.

To this end, MnS Producers and their management capabilities need to be exposed in the 3GPP management system.

From management service perspective the following information can be exposed:

- Basic data describing a management service , e.g. name, version, type

- Detailed capabilities of a management service, e.g. supported operations, supported notifications

From MnS Consumer perspective such information can be used for different purposes, including:

- MnS Producer discovery:  allows MnS Consumer to discover basic information about MnS Producer instances (MnsInfo instances in MnSRegistry).  In short, allows MnS Consumer to know which MnS Producers instances are exposed.

- MnS Producer Capabilities retrieval:  allows MnS Consumer to retrieve detailed information about a specific MnS instance.  The specific methods may be solution set and/or MnS Type specific (e.g. yang-library).  In short, allows MnS Consumer to retrieve the detailed capabilities of a specific MnS Producer instance.

In case registered MnS Producer information changes, the 3GPP management system needs to be updated and MnS consumers of the modified MnS producer should be informed.

MnS Consumers wishing to discover MnS Producers might have different questions. For example, a consumer may wish to know which MnS producers manage a certain geographical area or civic location. Or, after receiving an alarm notification specifying that a specific NF is alarmed, they may wish to know the MnS producers where they can request management data from that NF or to retrieve the configuration of that NF.

Second change

## 5.2	Specification level requirements

### 5.2.1	Use cases

#### 5.2.1.1	Adding a management service to MnS registry

| Use case stage |  |  | Evolution/Specification | <<Uses>> Related use |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Goal |  |  | Add a management service to the MnS Registry. |  |  |  |  |
| Actors and Roles |  |  | MnS Producer, MnS Registry |  |  |  |  |
| Telecom resources |  |  | - |  |  |  |  |
| Assumptions |  |  | Management service is ready to be added to MnS Registry. |  |  |  |  |
| Pre-conditions |  |  | The MnS Producer is available. |  |  |  |  |
| Begins when |  |  | There is a need for a management service to be exposed via the MnS Registry. |  |  |  |  |
| Step 1 (M) |  |  | The management service is added to the MnS Registry. |  |  |  |  |
| Post-conditions |  |  | The management service is discoverable. |  |  |  |  |
| Traceability |  |  | REQ-DMS-1 |  |  |  |  |

#### 5.2.1.2	Removing a management service producer from MnS registry

| Use case stage |  | Evolution/Specification |  | <<Uses>> Related use |  |
| --- | --- | --- | --- | --- | --- |
| Goal |  | Remove a management service from the MnS Registry. |  |  |  |
| Actors and Roles |  | MnS Producer, MnS Registry |  |  |  |
| Telecom resources |  | - |  |  |  |
| Assumptions |  | The management service no longer needs to be exposed via MnS registry. |  |  |  |
| Pre-conditions |  | The MnS Producer is available. |  |  |  |
| Begins when |  | The management service is ready to be removed from MnS Registry. |  |  |  |
| Step 1 (M) |  | The management service is removed from the MnS Registry. |  |  |  |
| Post-conditions |  | The management service is no longer discoverable. |  |  |  |
| Traceability |  | REQ-DMS-1 |  |  |  |

5.2.1.3	Void

#### 5.2.1.x	MnS Consumer retrieves basic management service information from MnS registry

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | MnS Consumer retrieves basic information about management service(s) from registry. |  |
| Actors and Roles | MnS Consumer, MnS Registry |  |
| Telecom resources | - |  |
| Assumptions | Management service is in MnS Registry. |  |
| Pre-conditions | MnS Consumer is authorized to read MnS Registry. |  |
| Begins when | MnS Consumer needs to access a specific MnS Producer(s). |  |
| Step 1 (M) | MnS Consumer queries MnS Registry with filter criteria based on the management service(s) of interest. |  |
| Step 2 (M) | MnS Consumer receives response with basic MnS Info for the management service(s) which match the criteria. |  |
| Post-conditions | MnS Consumer has basic information about the management service(s). |  |
| Traceability | REQ-DMS-2, REQ-DMS-3, REQ-DMS-4 |  |

#### 5.2.1.x	Providing detailed capabilities about management service

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | Management service detailed capabilities are exposed. |  |
| Actors and Roles |  |  |
| Telecom resources | - |  |
| Assumptions | Management service detailed capabilities are available. |  |
| Pre-conditions | Management service detailed capabilities are ready to be exposed. |  |
| Begins when | MnS Producer wants to expose its detailed capabilities. |  |
| Step 1 (M) | Management service detailed capabilities are exposed by MnS Producer. |  |
| Post-conditions | Management services detailed capabilities have been exposed. |  |
| Traceability |  |  |

#### 5.2.1.x	MnS Consumer retrieves detailed capabilities about management service

| Use case stage | Evolution/Specification | <<Uses>> Related use |
| --- | --- | --- |
| Goal | MnS Consumer retrieves detailed capabilities for specific MnS Producer(s). |  |
| Actors and Roles | MnS Consumer |  |
| Telecom resources | - |  |
| Assumptions | Management service has ability to expose its detailed capabilities. |  |
| Pre-conditions | Authorized MnS Consumer knows location and method to retrieve detailed capabilities. |  |
| Begins when | MnS Consumer requires to retrieve detailed capabilities of specific MnS Producer(s). |  |
| Step 1 (M) | MnS Consumer reads detailed capabilities from MnS Producer of interest. |  |
| Post-conditions | MnS Consumer has retrieved detailed capabilities of specific MnS Producer(s). |  |
| Traceability |  |  |

### 5.2.2	Requirements

REQ-DMS-1: The 3GPP management system shall provide capabilities allowing MnS producers to be registered (including endpoint address) in MnS registry for use by MnS consumers wishing to interact with these MnS producers.

REQ-DMS-2: The 3GPP management system shall provide capabilities allowing MnS consumers to retrieve the MnS Producers registered in MnS registry.

REQ-DMS-3: The 3GPP management system shall provide capabilities allowing to discover MnS producers that are managing a specified managed entity.

REQ-DMS-4: The 3GPP management system shall provide capabilities allowing to discover the managed entities a MnS producer is responsible for.