3GPP TSG-SA5 Meeting #150 	S5-235481

# Goteborg, Sweden, 21 - 25 August 2023	     										Revision of S5-22XXXX

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.291 | CR | 0490 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the data type of Trigger |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the description of date type of Trigger attribute, the data type TriggerType is optional. In the Open API, the date type TriggerType is mandatory . In order to keep date type description consistent with Open API, the data type TriggerType specifed in clause 6.2.5.2.1.5 should be corrected. The category of smf home provided charging id should be aligned with the home provide id. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the "P" column for the date type of Triggers from "Oc" to "M". Change the "P" column for the date type of smfHomeProvidedChargingId from "Om" to "Oc". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Nchf_Convergedcharging is not aligned in the data type and openAPI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.6.2.1.7, 6.1.6.2.2.6,6.2.5.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS 32.255 CR 0427 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.1.6.2.1.7	Type Trigger

Table 6.1.6.2.1.7-1: Definition of type Trigger

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| triggerType |  | TriggerType |  | M |  | 1 |  | the events whose occurrence lead to charging event is issued towards the CHF |  |  |  |
| triggerCategory |  | TriggerCategory |  | M |  | 1 |  | This field indicates whether the charging data generated by the NF consumer for the trigger lead to a Charging Event towards the CHF immediately or not. |  |  |  |
| timeLimit |  | DurationSec |  | OC |  | 0..1 |  | Time limit if trigger type is "Expiry of data time limit" |  |  |  |
| volumeLimit |  | Uint32 |  | OC |  | 0..1 |  | Volume limit if trigger type is "Expiry of data volume limit". This attribute is not valid from Nchf_ ConvergedCharging API version v2.0.0 |  |  |  |
| volumeLimit64 |  | Uint64 |  | OC |  | 0..1 |  | Volume limit if trigger type is "Expiry of data volume limit". This attribute replaces the volumeLimit attribute from Nchf_ ConvergedCharging API v2.0.0 |  |  |  |
| eventLimit |  | Uint32 |  | OC |  | 0..1 |  | Event limit if trigger type is "Expiry of data event limit". |  |  |  |
| maxNumberOfccc |  | Uint32 |  | OC |  | 0..1 |  | Maximum number if trigger type is "Max nb of number of charging condition changes" |  |  |  |
| tariffTimeChange |  | DateTime |  | OC |  | 0..1 |  | This field contains UTC time indicating the switch time when the tariff will be changed. |  |  |  |

Next change

6.1.6.2.2.6	Type PDUSessionChargingInformation

Table 6.1.6.2.2.6-1: Definition of type PDUSessionChargingInformation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| chargingId | ChargingId | OM | 0..1 | Charging identifier for correlation between different records of a single PDU session |  |
| sMFChargingId | string | OM | 0..1 | Used instead of ChargingId when feature is active. | SMF_Charging_Id |
| homeProvided ChargingId | ChargingId | Oc | 0..1 | Charging identifier for correlation between H-SMF and V-SMF. |  |
| sMFHomeProvidedChargingId | string | Oc | 0..1 | Used instead of homeProvidedChargingId when feature is active. | SMF_Charging_Id |
| userInformation | UserInformation | OM | 0..1 | including information of user and user equipment, |  |
| userLocationinfo | UserLocation | OC | 0..1 | provides information on the location |  |
| mAPDUNon3GPPUserLocationInfo | UserLocation | OC | 0..1 | provides information on the location under the non-3GPP access for the MA PDU session | ATSSS |
| non3GPPUserLocationTime | DateTime | OC | 0..1 | represents the UTC time provided by the non-3GPP access, and is related to the userLocationTime. This filed is only present if the non-3GPP access provides a time. |  |
| mAPDUNon3GPPUserLocationTime | DateTime | OC | 0..1 | represents the UTC time provided by the non-3GPP access, and is related mAPDUNon3GPPUserLocationInfo. This filed is only present if the non-3GPP access for the MA PDU session provides a time. | ATSSS |
| presenceReportingAreaInformation | map(PresenceInfo) | OC | 0..N | When the data type is present in response message, it includes the PRA information provisioned by the CHF, in which case the "presenceState" attribute within the PresenceInfo data type shall not be supplied. When the data type is present in request message, it’s used to report user presence reporting area status. The "praId" attribute within the PresenceInfo data type shall be the key of the map. The location related attributes (i.e. "trackingAreaList", "ecgiList", "ncgiList") within the PresenceInfo data type are not required in the request message, and may be ignored by the CHF. |  |
| uetimeZone | TimeZone | OC | 0..1 | the UE Timezone the UE is currently located |  |
| pduSessionInformation | PDUSessionInformation | OC | 0..1 | PDU session level information, including PDU session ID, PDU type, SSC Mode, QoS, network slicing etc. It needs to be present in the request, but it is optional in the response. |  |
| unitCountInactivityTimer | DurationSec | OC | 0..1 | threshold for the time period resource idle Upon the initial interaction with the CHF, the SMF use this attribute to provide pre-configured threshold to CHF. when present in response message, it contains the threshold supplied by CHF in response of initial request to override existing threshold in SMF. It’s only present when unit count inactivity timer trigger is active. |  |
| rANSecondaryRATUsageReport | RANSecondaryRATUsageReport | OC | 0..1 | Secondary RAT usage reported from RAN. |  |

Next change

6.2.5.2.1.5	Type Trigger

Table 6.2.5.2.1.5-1: Definition of type Trigger

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| triggerType |  | TriggerType |  | M |  | 1 |  | the events whose occurrence lead to charging event is issued towards the CHF |  |  |  |
| triggerCategory |  | TriggerCategory |  | M |  | 1 |  | This field indicates whether the charging data generated by the NF consumer for the trigger lead to a Charging Event towards the CHF immediately or not. |  |  |  |
| timeLimit |  | DurationSec |  | OC |  | 0..1 |  | Time limit if trigger type is "Expiry of data time limit" |  |  |  |
| volumeLimit64 |  | Uint64 |  | OC |  | 0..1 |  | Volume limit if trigger type is "Expiry of data volume limit". |  |  |  |
| eventLimit |  | Uint32 |  | OC |  | 0..1 |  | Event limit if trigger type is "Expiry of data event limit" |  |  |  |
| maxNumberOfccc |  | Uint32 |  | OC |  | 0..1 |  | Maximum number if trigger type is "Max nb of number of charging condition changes" |  |  |  |

End of change