3GPP TSG-SA5 Meeting #150 	S5- 235499

Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.255 | CR | 0431 | rev | - | Current version: | 18.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add charging principle for 5MBS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS_CH |  |  |  |  |  | Date: |  |  | 2023-08-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In order to support 5G MBS charging, charging principle for broadcast and multicast communication of different charging senarios need to be clarified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add charging information to be collected by SMF and MB-SMF in different charging senarios. Add cases of reporting these charging information for 5G Multicast-Broadcast Services. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Charging principle for 5G Multicast-Broadcast Services is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.3, Annex X.2.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.1.3	Charging information

Charging information in the 5GC domain network is collected for each UE by the SMFs. PDU session charging allows the SMF to collect and categorize per UE per UPF per PDU session, charging information related to data volumes.

The SMF shall collect the following charging information for converged online and offline charging:

-	usage of the access and core network resources: the charging information shall describe the amount of data delivered to and forwarded from the UE;

-	usage duration: duration of PDU session is counted as the time interval from PDU session establishment to PDU session release;

-	user: the charging information shall provide the actual UE addresses used by the user for the PDU session;

-	data network: the charging information shall describe the data network addresses with a level of accuracy as determined by the DNN;

-	usage of the external data networks: the charging information shall describe the amount of data sent and received to and from the external data network. External networks can be identified by the DNN;

-	start time: identifying the time when the PDU session was started;

-	user location: HPLMN, VPLMN, inside/outside presence reporting area, plus optional higher-accuracy location information.

The service data flows categorization is achieved by rating group or combination of the rating group and service id: i.e. based on the level of reporting defined per PCC rule, counting per rating group or combination of the rating group and service id. According to TS 23.503 [202], flow based charging shall support different charging models per PCC rule. These charging models may be based on volume, time and/or on number of events matching a specific service data flow template in PCC rule.

For service data flows defined for FBC, the SMF shall collect the following charging information:

-	the information described above for PDU session;

-	the amount of data transmitted in uplink and downlink directions categorized by rating group or combination of the rating group and service id when volume based charging applies;

-	the duration of service data flows is counted and categorized by rating group or combination of the rating group and service id when time based charging applies;

-	the number of events and corresponding time stamps categorized by rating group or combination of the rating group and service id when event based charging applies.

Within the PDU session the SMF shall collect the charging information for service data flows per UPF, categorized by rating group or combination of the rating group and service id.

Within the PDU session for local traffic offload scenarios with I-SMF insertion, the SMF shall collect the charging information for service data flows per I-SMF and categorized by rating group or combination of the rating group and service id.

Editor's note:	To have I-UPF as well as PSA2 UPF for the I-SMF controlled UPF is FFS.

The user can be identified by a Generic Public Subscription Identifier (GPSI) and/or a 5G Subscription Permanent Identifier (SUPI). For wireline access, SUPI may be used to identify subscriber via wireline network as specified in clause 5.9.2 of TS 23.501 [200].

For the multicast communication, the SMF may collect following charging information.

-	The duration of time from UE joining to UE leaving multicast MBS session;

-	The duration of time using 5GC shared MBS traffic delivery method, and/or the duration of time using 5GC individual MBS traffic delivery method;

-	The amount of multicast data transferred to UE via per-UE PDU sessions using 5GC individual MBS traffic delivery method. The SMF shall include the information of usage of per-UE PDU session for a 5GC individual MBS traffic delivery in charging information towards CHF.

SMF may report above charging information to CHF for the following cases.

-	UE joining multicast MBS session.

-	UE leaving multicast MBS session.

Charging information collected by MB-SMF for MBS session is specified in X.2.4 of Annex X.

2rd Change

Annex X (normative):
MB-SMF charging support based on 5G data connectivity

X.2	MB-SMF charging based on 5G data connectivity princples

X.2.4	Charging information

The enhancement of the 5GS to support 5G multicast-broadcast services is specified in TS 23.247 [204]. Following charging senarios of 5G multicast-broadcast services supported by MB-SMF and related charging information should be considered.

X.2.4.1	MB-SMF supported charging for broadcast communication

For the broadcast communication, the MB-SMF may collect following charging information.

- The amount of broadcast data transmitted;

- The session duration from the creation of the MBS session to the deletion of the MBS session;

- Service area or a list of gNBs being sent the broadcast service from MB-UPFs;

MB-SMF may report above charging information to CHF for the following cases.

- MBS session creation, establishment and deletion.

X.2.4.2	MB-SMF supported charging for multicast communication

For the multicast communication, the MB-SMF may collect following charging information.

- The amount of multicast data transmitted to the UEs;

- The session duration from the creation of the MBS session to the deletion of the MBS session;

- Service area or a list of gNBs serving this MBS service from MB-UPFs for 5GC shared MBS traffic delivery method, and UPFs serving this MBS service for 5GC individual MBS traffic delivery method;

MB-SMF may report above charging information to CHF for the following cases.

- MBS session creation and deletion;

- Establishment and release of shared delivery towards gNBs;

- Establishment and release of individual delivery towards UPFs.

End of Change