3GPP TSG-SA5 Meeting #150 	S5-235526

# Goteborg, Sweden, 21 - 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0457 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update EAS Discovery procedure related measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are many errors in the EAS discovery failure description in 5.15.1.3. The use case description in A.100 is not aligned with the performance data description either the EAS instantiation triggered by measurement data procedure description in TS 28.538, 1, it is the number of failures collected not the failure rate. 2, it is the consumer (e.g. ASP) determines whether an EAS VNF needs to be instantiated |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the errors in the EAS discovery failure description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The performance meausre related to EAS discovery failure may lead to mistake. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.1.3, A.100 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge link: |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.15.1.3	EAS discovery failure

a)	This measurement provides the number of EAS discovery failures when EES fails to discover and select a suitable EAS under its service area.

b)	CC

c)	On transmission of EAS discovery response (see clause 8.5.2 in TS 23.558 [52]) by the EES to the EEC that indicates the EAS discovery failure, the measurement is obtained by incrementing the appropriate subcounters. Some the examples are shown below:

- UeLocation (see table 8.5.3.2-1 in TS 23.558 [52]), as contained in the corresponding EAS discovery request (see clause 8.5.2 in TS 23.558 [52]). This will give the total number of EAS discovery Request got failed containing the given UE location.

- EasType (see table 8.5.3.2-2 in TS 23.558 [52]), as contained in the corresponding EAS discovery request (see clause 8.5.2 in TS 23.558 [52]). This will give the total number of EAS discovery Request got failed containing the given EasType as EAS discovery filter.

- EasSchedule (see table 8.5.3.2-2 in TS 23.558 [52]), as contained in the EAS Discovery request (see clause 8.5.2 in TS 23.558 [52]). This will give the total number of EAS discovery Request got failed containing the given EasSchedule as EAS discovery filter.

This measurement is performed per EAS Discovery Filter (see table 8.5.3.2-2 in TS 23.558 [52]).

d)	Each subcounter is an integer value

e)	DIS.EasDisFail.UeLocation

where UeLocation: identifies where the UE is connected to the network or the position of the UE (see clause 7.3.2 in TS 23.558 [52]), and is represented by the cell ID.

DIS.EasDisFail.provider

where provider is the EAS provider identifier of EAS Discovery Filter (see table 8.5.3.2-2 in TS 23.558 [52]).

DIS.EasDisFail.id

where id is the EAS identifier of EAS Discovery Filter (see table 8.5.3.2-2 in TS 23.558 [52]).

DIS.EasDisFail.EasType

where EasType is the category or type of required EAS (e.g., V2X)  of EAS Discovery Filter (see table 8.5.3.2-2 in TS 23.558 [52]).

DIS.EasDisFail.schedule

where schedule is the EAS schedule of EAS Discovery Filter (see table 8.5.3.2-2 in TS 23.558 [52]).

DIS.EasDisFail.geoloc

where geoloc is the EAS Geographical Service Area of EAS Discovery Filter (see table 8.5.3.2-2 in TS 23.558 [52]).

DIS.EasDisFail.toploc

where toploc is the  EAS Topological Service Area of EAS Discovery Filter (see table 8.5.3.2-2 in TS 23.558 [52]).

DIS.EasDisFail.srvCont

where srvCont is the Service continuity support of EAS Discovery Filter (see table 8.5.3.2-2 in TS 23.558 [52]).

DIS.EasDisFail.srvPerm

where srvPerm is the required level of service permissions e.g. trial, gold-class of EAS Discovery Filter (see table 8.5.3.2-2 in TS 23.558 [52]).

DIS.EasDisFail.srvFeature

where srvFeature is the required service features e.g. single vs. multi-player gaming service of EAS Discovery Filter (see table 8.5.3.2-2 in TS 23.558 [52]).

f)	EESFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for EES performance assurance.

2nd Change

A.100	Use case of measurements for EES

EES related measurements are used to measure the performance of an EES on each of the supported functionality.

Discovery procedures enable entities in an edge deployment to obtain information about EAS and their available services, based on specified criteria of interest. It is useful to analyse the EAS discovery success rate in order to assess EES performance. The discovery procedure may fail due to the unavailability of the relavant EAS. If the number of failures within a time period increases beyond a defined threshold, corrective actions (e.g instantiating the required EAS instance(s) in an appropriate EDN) can be taken by MnS consumer (e.g. ASP). Hence, it is necessary to collect measurement related with EAS discovery procedures. EAS discovery failure measurement can be used to mitigate the EAS discovery failure issue, based on the UE locations, application characteristics, and number of EAS discovery failures.

An EEC performs registration, registration update, and de-registration with an EES in order to provide information that can be used by the EES in Edge Computing services. It is useful to analyse the success rate of registration, registration update, and de-registration in order to assess EES performance. If the number of failures within a time period increases beyond a defined threshold, corrective actions can be taken by the OAM systems. Hence, it is necessary to collect measurement related with EEC registration, registration update, and de-registration procedures.

The EAS Registration procedure allows an EAS to provide its information to an EES in order to enable its discovery. It is useful to analyse the EAS registration success rate in order to assess EES performance. If the number of failures within a time period increases beyond a defined threshold, corrective actions can be taken by the OAM systems. Hence, it is necessary to collect measurement related with EAS Registration procedures.

End of Change