3GPP TSG-SA5 Meeting #150 	S5-235565

Berlin, Germany, 21 – 25 August 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to Draft CR TS 28.105 Use case and requirements for AIML training trustworthiness |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case on AI/ML training trustworthiness has been discussed and agreed (clause 5.3.1 in TR 28.908). This CR is to normatively propose the use case and requirements on AI/ML training trustworthiness. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use case and requirements have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use case “AI/ML training trustworthiness” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 (new clauses added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

### 6.2	ML training phase

### 6.2.X	Trustworthiness management for ML training

### 6.2.X.2	Use cases

#### 6.2.X.2.3	AI/ML training trustworthiness

AI/ML will play an important role as a means to automate and optimize complex network operations in RAN, core and management domains. Therefore, it is of utmost importance to consider trustworthiness of AI/ML training and the management of such training trustworthiness across end-to-end networks. To manage AI/ML training trustworthiness, the ML training may need to be performed according to the desired trustworthiness measure of the ML model. For example,

-	The ML model can be trained to generate explanations for the predictions.

-	The ML model can be trained to detect and mitigate biased outcomes.

-	The ML model can be trained to perform well on unseen or missing data.

-	The ML model can be trained together with adversarial input samples so that the trained model can detect adversaries.

Depending on the use case, one or more training trustworthiness techniques can be applied during training the ML model. Therefore, the ML training producer can be queried to provide information on the supported training trustworthiness capabilities enabling the ML training MnS consumer to request for a subset of supported training trustworthiness characteristics to be configured, measured, and reported.

### 6.2.X.3	Requirements for ML training trustworthiness management

Table 6.2.X.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN_TRUST-1 | The ML training MnS producer should have a capability to define new use case specific AI/ML training trustworthiness metrics in addition to the usage of generic AI/ML training trustworthiness metrics. | AI/ML training trustworthiness (clause 6.2.X.2.3) |
| REQ-ML_TRAIN_TRUST-2 | The ML training MnS producer should have a capability allowing the authorized MnS consumer to discover supported AI/ML training trustworthiness capabilities and metrics and select all or a subset of them based on the authorized MnS consumer training requirements. | AI/ML training trustworthiness (clause 6.2.X.2.3) |
| REQ-ML_TRAIN_TRUST-3 | The ML training MnS producer should have a capability to train an ML entity to satisfy the desired training trustworthiness measure. | AI/ML training trustworthiness (clause 6.2.X.2.3) |
| REQ-ML_TRAIN_TRUST-4 | The ML training MnS producer should support a capability to report to the authorized MnS consumer the evaluated training trustworthiness metrics. | AI/ML training trustworthiness (clause 6.2.X.2.3) |

End of Changes