3GPP TSG-SA5 Meeting #150 	S5-235617

Goteborg, Sweden, 21 - 25 August 2023

# Source:	Ericsson

# Title:	Update of trigger handling for NSACF

# Document for:	Approval

Agenda Item:	7.4.1

# 1	Decision/action requested

Include the proposed changes in TS 28.203.

# 2	References

[1]		3GPP TS 28.203: "Network Slice Admission Control charging in the 5G System (5GS); Stage 2"

# 3	Rationale

The trigger handling for quota management is incomplete.

# 4	Detailed proposal

First change

#### 5.2.1.2	Applicable triggers in the NSACF

##### 5.2.1.2.1	General

When a charging event is issued towards the CHF, it includes details of charging information such as S-NSSAI.

Each trigger condition (i.e., chargeable event) defined for the Network Slice Admission Control converged charging functionality with the associated behaviour when they are met, is specified this document, and the basic trigger mechanism is specified in the 3GPP TS 32.290 [5].

For session based charging the following applies:

-	A Charging Data Request [Initial] is invoked by the NSACF towards the CHF when a trigger for "initial" is met for an S-NSSAI, and no charging session exists for this S-NSSAI.

-	A Charging Data Request [Termination] is invoked by the NSACF towards the CHF when a trigger for "termination" is met for an S-NSSAI.

-	A Charging Data Request [Update] is invoked by the NSACF towards the CHF, when any trigger which is not a trigger for "initial" nor a trigger for "termination" is met, and the charging session exists for an S-NSSAI.

Table 5.2.1.2.1.1 summarizes the set of default trigger conditions which shall be supported by the NSACF when charging is active for the corresponding NSACF functionality, with the associated Charging Data Request message sent from NSACF towards the CHF.

Table 5.2.1.2.1.1: Default trigger conditions in NSACF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Initial |  |  |  |  |  |
| Number of UEs threshold reached for initial | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Initial] (NOTE 1) |
| Number of PDU sessions threshold reached for initial | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Initial] (NOTE 1) |
| Change of charging conditions |  |  |  |  |  |
| Number of UEs threshold crossed upwards | - | Immediate | Yes | Yes | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |
| Number of UEs threshold crossed downwards | - | Immediate | Yes | Yes | PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |
| Number of PDU sessions threshold crossed upwards | - | Immediate | Yes | Yes | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |
| Number of PDU sessions threshold crossed downwards | - | Immediate | Yes | Yes | PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |
| Quota management |  |  |  |  |  |
| Number of UEs quota threshold reached | - | Immediate | No | Yes | Charging Data Request [Update] |
| Number of PDU sessions quota threshold reached | - | Immediate | No | Yes | Charging Data Request [Update] |
| Quota exhausted | - | Immediate | No | Yes | Charging Data Request [Update] |
| Expiry of quota validity time | - | Immediate | No | Yes | Charging Data Request [Update] |
| Expiry of quota holding time | - | Immediate | No | Yes | Charging Data Request [Update] |
| Re-authorization request by CHF | - | Immediate | No | No | Charging Data Request [Update] |
| Termination |  |  |  |  |  |
| Number of UEs threshold reached for termination | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] (NOTE 2) |
| Number of PDU sessions threshold reached for termination | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] (NOTE 2) |
| Network slice termination | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] |
| NOTE 1: If both Nb of UEs threshold reached for initial and Nb of PDU sessions threshold reached for initial are active at the same time, then whichever is reached first will trigger the SCUR: Charging Data Request [Initial] NOTE 2: If both Nb of UEs threshold reached for termination and Nb of PDU sessions threshold reached for termination are active at the same time, then both needs to be reached to trigger the SCUR: Charging Data Request [Termination] |  |  |  |  |  |

End of changes