3GPP TSG-SA5 Meeting #150 	S5-235619

Goteborg, Sweden, 21 - 25 August 2023

# Source:	Ericsson

# Title:	New solution for IoT Charging information Optimization

# Document for:	Approval

Agenda Item:	7.5.1

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: "Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

Shortening the table for solution #1.15 and adding a new solution covering the same key issue.

# 4	Detailed proposal

First change

#### 5.1.5.15	Solution #1.15: IoT Charging information Optimization

A possible solution for key issues #1f and #1g covering requirements REQ-CH_INFO-03, optimization of charging information.

Currently, charging for data transfer is based on volume based measurement of the connections in down and uplink directions which has been enhanced to cover big data transfers.

This solution would bring the following advantages for IoT Charging:

- Charging for loT subscriptions (which can be devices, group of devices, loT applications, external Identifiers) is simplified.

- Overload of charging system due to many CDRs, one CDR per payload, may be avoided, by reducing the number of requests to Charging, limit down the Charging Data to IoT related one.

- A new option to control loT devices and/or their environment, on this case its considered that each IoT device corresponds to a PDU Session (one UE and one subscriber).

The solution proposes the availability of a new count for the measurement and calculation of NIDD (Non-IP Data Delivery) of the Charging Data Request, and the availability of new default triggers specific to IoT use cases.

The Message Payload is a PFCP session related message (N4 Interface between UPF and SMF) as specified in TS 29.244 [19]. The new attributes Payload-Input-Count and Payload-Output-Count, would be used to indicate the number of times/events each one of an input payload for the subscription and an output payload from the subscription are delivered., it would subsequently generate the respective charging information, once it reaches a threshold leading to a reduction of Charging Requests towards CHF.

A payload can be defined as trigger. The basic trigger mechanism is specified in TS 32.290 [12]

According to TS 32.255 [2], when a PDU session starts, and the converged charging is activated, the SMF invokes a Charging Data Request [Initial] towards the CHF to get authorization to start based on the default triggers. The SMF is optionally provided in a Charging Data Response [Initial] to override the default triggers, with a set of chargeable event triggers to be enabled, and the associated category (on this case deferred report).

The triggers remain active until they are updated or disabled by subsequent Charging Data Response [Update] from the CHF or the PDU session is terminated.

Example of Payload Triggers in TS 32.255[2] table 5.2.1.4.1

Editor’s note: The trigger of the payload needs to be further clarified.

Table 5.1.5.15-1: Extension to default trigger conditions in SMF

| Trigger Conditions | Trigger level | Converged Charging default category | Offline only charging default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- | --- |
| Limit per PDU session |  |  |  |  |  | Charging Data Request [Udate] |
| IoT payload threshold reached | PDU session | Immediate | Immediate | No | Yes |  |

Editor’s note:	If the Payload-Input-Count and Payload-Output-Count should be seen as triggers is ffs.

Example of Payload-Input-Count and Payload-Output-Count in the Charging Data Request Message in the TS 32.254 [17] table 6.2a.1.2.1.1:

Table 5.1.5.15-2: Charging Data Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57], and holds the identifier of the AF |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Retransmission Indicator | - | This field is not applicable. |
| Payload-Input-Count | OC | This field indicates a number of times the input payload for the subscription is delivered |
| Payload-Output-Count | OC | This field indicates the number of times the output payload from the subscription is delivered |
| One-time Event | OC | This field indicates, if included, that this is a one-time event and that there will be no update or termination. |
| One-time Event Type | OC | Described in TS 32.290 [12]. |
| Notify URI | - | This field is only applicable for the notification of abort charging. |
| Triggers | OC | This field is described in TS 32.290 [12] and holds the NEF specific triggers described in clause 5.4.1.x |
| Multiple Unit Usage | OC | This field contains the parameters for the quota management request and/or usage reporting. |
| Rating Group | M | Described in TS 32.290 [12] |
| Requested Unit | OC | Described in TS 32.290 [12] |
| Used Unit Container | OC | Described in TS 32.290 [12] |
| NEF API Charging Information | OM | This field holds the NEF API specific information described in clause 6.3.1.4 |

Second change

#### 5.1.5.x	Solution #1.x: IoT NEF charging information optimization

A possible solution for key issues #1f and #1g covering requirements REQ-CH_INFO-03, optimization of charging information.

Currently, charging for data transfer is based on volume based measurement of the connections in down and uplink directions which has been enhanced to cover big data transfers.

This solution would bring the following advantages for IoT Charging:

- Charging for loT subscriptions (which can be devices, group of devices, loT applications, external Identifiers) is simplified.

- Overload of charging system due to many CDRs, one CDR per payload, may be avoided, by reducing the number of requests to Charging, limit down the Charging Data to IoT related one.

- A new option to control loT devices and/or their environment, on this case its considered that each IoT device corresponds to a PDU Session (one UE and one subscriber).

The solution proposes the use of the current NIDD (Non-IP Data Delivery) handling as specified in TS 32.254 [x] and adding new triggers for number of API invocation/notifications.

The NIDD delivery uses the following, according to TS 29.599 []:

-	MO NIDD, the interaction between the SMF and the NEF uses Nnef_SMContext service, as specified in 3GPP TS 29.541 [x];

-	MT NIDD, the interaction between the SMF and the NEF uses Nsmf_NIDD service as specified in 3GPP TS 29.542 [x].

When delivering the message to/from the AF the following can be used:

-	NiddDownlinkDataTransfer service, and it can be updated by the NiddDownlinkDataTransferPatch service

-	NiddUplinkDataNotification service

To have a trigger each time an uplink message is delivered to the AF or received from the AF may cause a lot of interaction and adding a new limit triggering after a number of events to default trigger conditions in NEF TS 32.254 [x] table 5.4.1.2.1.1

Table 5.1.5.x-1: Extension to default trigger conditions in NEF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| API Invocation limit | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| API Notification limit | - | Immediate | Not Applicable | Not Applicable | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |

Example of Payload-Input-Count and Payload-Output-Count in the Charging Data Request Message in the TS 32.254 [17] table 6.2a.1.2.1.1:

Table 5.1.5.x-2: Charging Data Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57], and holds the identifier of the AF |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | This field indicates, if included, that this is a one-time event and that there will be no update or termination. |
| One-time Event Type | OC | Described in TS 32.290 [12]. |
| Notify URI | - | This field is only applicable for the notification of abort charging. |
| Triggers | OC | This field is described in TS 32.290 [12] and holds the NEF specific triggers described in clause 5.4.1.x |
| Multiple Unit Usage | OC | This field contains the parameters for the quota management request and/or usage reporting. |
| Rating Group | M | Described in TS 32.290 [12] |
| Requested Unit | OC | Described in TS 32.290 [12] |
| Used Unit Container | OC | Described in TS 32.290 [12] |
| NEP API Container Information | OC | Used instead of NEF API Charging Information if more than one NEF API Charging Information needs to be reported described in table 5.1.5.x-3. |
| NEF API Charging Information | OM | This field holds the NEF API specific information described in TS 32.254 [x] clause 6.3.1.4 |

Table 5.1.5.x-3: Structure of the NEF API Container Information

| Information Element | Description |
| --- | --- |
| NEP API Container Information | This parameter holds the same information as the NEF API Charging Information. |

Extension of the Triggers information element in the Charging Data Request message contents in TS 32.290 table 6.1.1.2.1:

Table 5.1.5.x-4: Extension of Triggers information element

| Information Element |  | Converged Charging Category |  | Offline Only Charging Category |  | Description |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Triggers |  | OC |  | OC |  | This field holds the triggers supplied from the CHF for the charging session that are independent of rating group for quota management and without quota management. |  |
| Trigger Type |  | OC |  | OC |  | This field holds the events whose occurrence led to charging event is issued towards the CHF |  |
| Trigger Category |  | M |  | M |  | This field indicates whether the charging data generated by the NF consumer for the trigger lead to a Charging Event towards the CHF immediately or not. |  |
| Time Limit |  | OC |  | OC |  | This field holds the time limit if trigger type is "Expiry of data time limit" |  |
| Volume Limit |  | OC |  | OC |  | This field holds the total (uplink and downlink) volume limit if trigger type is "Expiry of data volume limit". |  |
| Max Numbers Of Events |  | OC |  | OC |  | This field holds the maximum number of events if trigger type is "Expiry of data event limit". |  |
| Max Numbers Of Charging Condition Changes |  | OC |  | OC |  | This field holds the maximum number if trigger type is "Max number of charging condition changes". |  |
| Max Numbers Of Invocations |  | OC |  | OC |  | This field indicates the maximum number of invocations accepted |  |
| Max Numbers Of Notifications |  | OC |  | OC |  | This field indicates the maximum number of notifications delivered |  |
| Tariff Time Change |  | OC |  | OC |  | This field contains UTC time indicating the switch time when the tariff will be changed. |  |

End of changes